3GPP TSG-RAN WG2 Meeting #105 Bis	R2-1903868

Xi’an, China, April 08th – April 12th 2019

Agenda item:	12.2.3

Source:	Gemalto N.V

Title:	Considerations to WUS Grouping

WID/SID:	Additional Enhancements for NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

Rel-16 Work Item for LTE-M and NB-IOT has an objective to discuss about Wake up signal enhancements. WUS (Wake up Signal) was introduced in Rel-15. Following agreements were made during RAN2 #104 bis:

Further discuss the benefit and feasibility of using service based parameters for grouping in addition to UE-ID

Can discuss group distribution further, including Rel-15/16 mechanism interaction, once we know more about number of groups and more about the grouping solution (e.g. service based parameters)

RAN2 will decide on the UE to WUS group mapping

There was an email discussion proposed for RAN 2 #105 and Following are the agreements made in RAN2#105 related to UE Group Wake up Signal functionality.

- Feasibility of the solution based on the following attributes for deriving the service-type for GWUS can be studied further.

  - Paging Probability

  - Mobility

We would like to emphasize on the fact that why UE_ID based grouping is better.

# 2	 Discussion

WUS grouping based on UE_ID was agreed in RAN 2 # 103 bis and it was mentioned that we can explore more options for grouping and hence an email discussion was proposed in last meeting. Even though there is an agreement related to studying feasibility of more options but Gemalto thinks it will add more complexity and at least for Rel-16, UE_ID based solution should be considered as only solution and other solutions can be postponed to next release.

The reason for going with only UE_ID based solution in Rel-16 is that Gemalto think UE_ID based solution allows equal distribution. If we move to service based solution which includes solutions based on paging probability and Mobility it will lead to different size groups and will require further sub-division too. Also there can be further issues which can be raised if we start mixing different types of devices.

Observation 1: UE_ID based solution provides equal distribution

Observation 2: Service based solutions will lead to different group sizes and hence more complexity.

We think there is a need to define one WUS grouping scheme to make things less complicated. If we define more than one sub-grouping schemes, then we might not be able to use the feature efficiently as it will make things more complicated. Also even if it is decided to have more than one solutions then RAN1 and other groups also needs to get involved which will make process a bit more complex

One important factor for this feature is false paging and each device has its own specific use of services for example the eDRX and PSM configurations. For example in Japan there is a specific market for gas meters where there is an industry standard that LP gas meter should be reachable within 3 minutes to turn off the gas. To make sure we use this feature in efficient way we need to make sure that we reduce false paging that will provide gains in terms of Power for Rel-16 UE’s as compared to Rel-15 UE’s.

Observation 3: gWUS should be used in an efficient way to reduce False paging hence the power consumption of UE.

UE_ID is one of the parameters to calculate time diversity for sending of paging messages which also limits the scheduling conflicts and allowing UE’s to go into power saving mode (DRX, eDRX etc) to reduce their power consumption.  It splits the UE’s into groups with identical paging occasions and hence there is a uniform distribution. There can be discussions where we can try to calculate new formula as discussed by other companies in later discussions but to have baseline based on UE_ID only for Rel-16 is the key.

Hence we propose following:

Proposal 1: For Rel-16, only UE_ID based solution should be used for Group Wake up signal (gWUS) feature.

Proposal 2: It is FFS whether there need to be a new calculation for UE_ID based formula to make sure UE_ID based grouping can be made more efficient.

# 3           Conclusion

Based on the discussion above we would like to propose following

Observation 1: UE_ID based solution provides equal distribution

Observation 2: Service based solutions will lead to different group sizes and hence more complexity.

Observation 3: gWUS should be used in an efficient way to reduce False paging hence the power consumption of UE.

Proposal 1: For Rel-16, only UE_ID based solution should be used for Group Wake up signal (gWUS) feature.

Proposal 2: It is FFS whether there need to be a new calculation for UE_ID based formula to make sure UE_ID based grouping can be made more efficient.

# 4        Reference

- RAN2 #105 - Chairman notes

- RAN2 #104 - Chairman notes

- R2-1900317- Report on email discussion [104#42][NB-IoT R16]  Service based

Group Wake Up signal analysis