3GPP TSG-RAN WG2 Meeting #105bis	R2-1904237

# Xian, China, 08 – 12 April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 998 | rev | - | Current version: | draft15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaton on access barring check for RRC Connected |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-03-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS24.501 specifies unified access control applicability to UL NAS Transport message (section 4.5.1) The assumption on Access Barring check for the UL Information Transfer follows CT1 specification and is reflected in the TS38.331 by an Editor’s Note: Editor's Note: It is assumed that NAS triggers the Unified Access Control specified in 5.3.x before initiating this procedure. UE performs this procedure if the access attempt is allowed according to 5.3.14. However, the Editor’s Note assume a separate NAS and AS interaction for UAC. As it is done for the other procedures the AS invokes the UAC as a starting step of the UL Information Transfer in case NAS provides Access Category and Access Identitie(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove Editor’s Note in 5.7.2 - Add procedural steps for initiating ULInformationTransfer with access barring check first. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: Unified Access Control Inter-operability: - If the network is implemented according to the CR and the UE is not: Access Barring Check will not apply correctly. The UE won’t apply UAC procedure for UL NAS dedicated message transfer, according to UAC design, and access attempt may pass, even though from system level it should not. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. The NW broadcast UAC configuration with assumption that all access attemps should be subject to UAC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned UE behaviour in NAS and RRC specifications. Not all access attempts assumed by NAS layer will be processed by UAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | 5.7.2 |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.7.2	UL information transfer

#### 5.7.2.1	General

Figure 5.7.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS dedicated information from the UE to the network.

#### 5.7.2.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer NAS dedicated information.

Upon initiation of the procedure, the UE shall:

- if the upper layers provide an Access Category and one or more Access Identities:

2> perform the unified acccess control procedure as specified in 5.3.14 using the Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	initiate transmission of the of the ULInformationTransfer message in accordance with 5.7.2.3.

#### 5.7.2.3	Actions related to transmission of ULInformationTransfer message

The UE shall set the contents of the ULInformationTransfer message as follows:

1>	if the upper layer provides NAS PDU:

2>	set the dedicatedNAS-Message to include the information received from upper layers

1>	submit the ULInformationTransfer message to lower layers for transmission, upon which the procedure ends.

#### 5.7.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1>	if AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1>	if mobility (i.e. handover, RRC connection re-establishment) occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers:

2>	inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages.

End of Modified Subclause