3GPP TSG-RAN WG2 Meeting #106                                                        R2-1905743

Reno, USA, 13th May – 17th May 2019

Source:                CATT

Title:	Further Discussion on Multiple Active SPS/CGs

Agenda Item:	11.7.2.2

Document for:	Discussion and Decision

Introduction

In RAN2#105bis meeting, RAN2 discussed several issues related to supporting multiple active SPS and CGs for TSN traffic, and the following agreements were reached:

- R2 assumes that the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS). - R2 assumes that activation/deactivation is done by DCI. - RAN1 should address activation/deactivation DCIs related with configured grant Type 2 and SPS in the case of multiple configurations - When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID

In this contribution, we will further discuss the remaining open issues for multiple active SPSs/CGs:

- Issue 1: How to configure the multiple SPSs and CGs?

- Issue 2: How to calculate the HARQ ID?

- Issue 3: How to design the CG confirmation MAC CE?

- Issue 4: What is the impact of introducing multiple CGs on LCP?

Based on the analysis, our preferences are given.

Discussion

Multiple SPSs/CGs configurations

According to [1], RAN1 defined two use cases for multiple CGs, listed below:

- Use case 1: Multiple CG configurations are for different service/traffic types;

- Use case 2: Multiple CG configurations are used for enhancing reliability and latency.

For use case 1, it is similar to the LTE multiple UL SPS, i.e. different SPS/CG configurations aim at addressing different logical channels. Hence for use case 1, the multiple SPS/CG configurations can follow the LTE multiple UL SPS configurations. In other words, one SPS/CG index should be introduced for each SPS/CG configuration, and the SPS/CG related configuration, e.g., periodicity, number of HARQ processes, HARQ process ID offset and so on, can be linked to the SPS/CG index. This is illustrated in following Figure 1:

Figure 1: Example of multiple CGs for different TSC traffics

Proposal 1: CG index should be introduced to identify the multiple CGs for different TSC traffic patterns.

For use case 2, the multiple SPS/CG configurations aim at addressing only one periodic and low latency traffic, which data burst arrival times are known but may experience some jitter. In TSN networks, this is typically the case e.g. of a flow scheduled within a specific time slot of an IEEE802.1Qbv recurring cycle. As a result, such traffic cannot be safely supported by only one SPS/CG allocation but requires multiple consecutive identical configured grant allocations (see Figure 2). Since such staggered SPS/CG configurations come in support of only one logical channel, they should share the same configuration parameters, such as the periodicity, the HARQ process ID offset, etc. Hence there is no need to treat them as separate SPS/CG configurations. As suggested in [4], the best way is to gather them into one SPS/CG group. We can further discuss how to represent this group when configuring the staggered SPSs/CGs. From the RRC signaling overhead perspective, the simplest way is to link them with the same CG index and a common time offset for the group (tied to the 1st SPS/CG occurrence) and a 2nd time offset defining the relative spacing of the consecutive SPS/CG allocations. This is illustrated in Figure 2 where we group three sub-sets of CGs (represented by the green, the yellow and the blue) into one CG group using the same CG index (CG config#0).

Figure 2: Example of multiple CGs for enhancing reliability and/or reducing latency

Proposal 2:  The multiple SPS/CGs used for addressing the arrival time jitter of a low-latency traffic flow can be gathered into one group sharing the same SPS/CG configuration index.

Currently RAN2 assumes that the maximum number of active CG configurations for a given BWP of a serving cell is 8 or 16, and further guidance was asked to RAN1 in [6] on their preferred value. The above sets the maximum number of SPS/CG configurations per BWP, but in case of CA a maximum number should also be agreed, at MAC entity level. We think the number of 16 is also quite reasonable for the CA case. That is to say, for one MAC entity, it is suggested that at most 16 SPS/CGs can be configured.

Proposal 3:  The maximum number of SPS/CGs per MAC entity is 16.

HARQ process ID calculation

In RAN2# 105bis meeting, it was agreed that one offset is needed for the calculation of the HARQ process ID [5]. In this section, we discuss different alternatives addressing the Offset issue.

When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID

Alternative 1: Offset is signaled by RRC

In this alternative, the RRC explicitly configures the offsets of different SPS/CG configurations. For example, for CG configuration i, the HARQ process ID offset is offseti. When HARQ process ID calculation is performed, offseti  will be directly used in the equation.

Alternative 2: Offset is calculated based on the HARQ process number of each SPS/CG configuration.

In alt 2, the offset is implicitly calculated based on the SPS/CG configuration index and the HARQ process number of each SPS/CG configuration. The offseti is calculated according to the equation below,

offset0=0,                              i=0offseti=j=0i-1HarqNumj, i>0	(1)

where, HarqNum is the HARQ process number of each SPS/CG configuration, and i is the configuration index which is defined in section 2.1. Based on Equation (1), the HARQ process ID will be grouped in term of SPS/CG configuration without HARQ ID collision. The entire procedure is illustrated in Figure 3.

Figure 3: One example for HARQ process ID calculated based on HARQ Process Number

Compared with alternative 1, one improvement is that there is no need for RRC configuration on the parameter offset. Furthermore, if the index of the SPS/CG configuration changes, it is not necessary to reconfigure the offset value.

Proposal 4: RAN2 to down select one alternative for the offset determination in HARQ process ID calculation.

CG confirmation MAC CE

In Rel-15 NR Uu interface, Type 1 and Type 2 CGs are configured by RRC per Serving Cell and per BWP. Multiple CGs can be active simultaneously only on different Serving Cells. It means there is at most one type 2 CG on one BWP. The CG confirmation MAC CE can be sent on the corresponding BWP, hence zero-bit payload CG confirmation MAC CE is used in Rel-15, which is shown in the following Figure-3:

CG confirmation MAC CE subheader

Figure 3: Rel-15 zero-bit payload CG confirmation MAC CE

For Rel-16 IIOT, multiple active CGs are supported in one BWP, and their corresponding CG resources may be overlapped in time domain. It is proposed in [3] to address this by keeping the legacy zero-bit payload CG confirmation MAC CE, but restricting its transmission to only occur in a PUSCH resource associated to the CG the confirmation relates to. However, this approach has some drawbacks, listed below:

- On one BWP, in case of the CG used for transmitting the CG confirmation MAC CE collides with a dynamic grant and the dynamic grant is prioritized, UE cannot send the CG confirmation MAC CE. It will delay the CG confirmation.

- On one BWP, in case of the CG collides with another CG, and they are both used for transmitting their own CG confirmation MAC CE, UE can only send one of them and the other will be delayed.

- On one BWP, it is possible that only the CG confirmation MAC CE needs to be sent on the CG resource, no data matching this CG being available. But it may overlap with another grant (either dynamic or configured) which has matching data to be sent. Considering the CG confirmation MAC CE has higher priority than the data, the prioritization rule will favor its CG, and the data in the other CG will be delayed. Without the resource mapping restriction for the CG confirmation MAC CE, it could have been sent in the other CG together with the data, without requiring any prioritization.

In order to avoid the above drawbacks, we suggest introducing the CG index into the CG confirmation MAC CE. Assuming at most 16 CGs are supported for one UE, the new CG confirmation MAC CE format is shown in Figure 4:

- R: is the reserved bit;

- CGi: indicates whether the CG with index i is confirmed by this MAC CE.

CG confirmation MAC CE subheader

CG confirmation MAC CE

Figure 4: Example of the new CG confirmation MAC CE

Another benefit of this approach is that compared with sending each CG confirmation MAC CEs in separate PUSCH resources, the resource consumption by MAC CE(s) is lower for the single multi-bit MAC CE, i.e. more user data can be conveyed in the two CGs.

Proposal 5: Introduce CG index in the CG confirmation MAC CE.

Proposal 6: Use a new LCID to identify the legacy CG confirmation MAC CE and the new CG confirmation MAC CE.

Impact on LCP

In Rel-15 NR Uu, regarding to the CG, there is only one LCP restriction parameter, configuredGrantType1Allowed, which restricts whether one logical channel can use the type 1 CG. For the type 2 CG, there is no such restriction.

In Rel-16 IIOT, if the same LCP restriction parameters are used, the LCP procedure cannot ensure a logical channel is served by the CG configured based on its traffic pattern. This will impact the IIOT service latency. In order to solve this problem, some association is needed between the logical channel and its related CG(s).  The most direct method is to introduce the CG index as one of the LCP restriction parameters for each logical channel.

Proposal 7:  When a CG configuration targets one or multiple logical channels, the CG index is used as one of the LCP restriction parameters for the related logical channel(s).

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: CG index should be introduced to identify the multiple CGs for different TSC traffic patterns.

Proposal 2:  The multiple SPS/CGs used for addressing the arrival time jitter of a low-latency traffic flow can be gathered into one group sharing the same SPS/CG configuration index.

Proposal 3:  The maximum number of SPS/CGs per MAC entity is 16.

Proposal 4: RAN2 to down select one alternative for the offset determination in HARQ process ID calculation.

Proposal 5: Introduce CG index in the CG confirmation MAC CE.

Proposal 6: Use a new LCID to identify the legacy CG confirmation MAC CE and the new CG confirmation MAC CE.

Proposal 7:  When a CG configuration targets one or multiple logical channels, the CG index is used as one of the LCP restriction parameters for the related logical channel(s).

Reference

- R1-1905609   Summary of 7.2.6.6 Enhanced UL configured grant transmission

- R2-1903149   MAC-specific aspects of multiple active SPS/CG configurations   CATT

- R2-1904046	   On Confirmation for multiple configured grant configurations   Ericsson

- R2-1903584	  SPS enhancements for TSC traffic	NTT DOCOMO INC;

- RAN2#105bis meeting chairman notes;

- R2-1905457	  LS on shorter SPS periodicities NTT DOCOMO INC