3GPP TSG-RAN2 Meeting #106	R2-1905812

Reno, USA, 13th –17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCG link fast recovery in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson(Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | To be merged to Running CR, if agreed |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capture the agreements from RAN2_105_bis: Agreements for MCG fast recovery: 0 MCG fast recovery targets all MRDC architecture options 1: When MCG failure occurs, UE follows SCG failure-like procedure: i. UE does not trigger RRC connection re-establishment. ii. UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG. 2: MCG fast recovery targets the following use cases MCG leg RLF FFS: Other uses cases. Can consider in future whether the mechanism can be also be applied in the case of other MCG failures. 3 MCG fast recovery can only be triggered after AS security has been activated and the SRB2 and at least one DRB have been setup 4 MCG failure indication should include: i. Available measurement results of MCG ii. MCG link failure cause iii. Available measurement results of SCG iv. Available measurement results of non-serving cells 5: For MCG failure indication, new RRC message in introduced, e.g. MCGFailureInformation. 6: SCG leg of the split SRB1 can be used for MCG fast recovery. FFS: If configured, SRB3 can be used for MCG fast recovery. Priority is to complete the solution based on split SRB1 7: New SRB is not introduced for MCG fast recovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Captured agreements from RAN2_105bis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------------------Change Start----------------------------------------------------

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

If radio link failure is detected for MCG, and the UE does not support fast MCG link recovery, the UE initiates the RRC connection re-establishment procedure.

The following MCG failure case triggers fast MCG link recovery:

-	MCG RLF;

Editor's note: If other MCG failure case(s) can be considered is FFS.

During fast MCG link recovery, the UE suspends MCG transmissions for all radio bearers and reports the MCG Failure Information to the MN via the SCG, using the SCG leg of split SRB1 (if configured), instead of triggering re-establishment.

Editor's note: If MCG failure information can be sent via SRB3 is FFS.

The UE includes in the MCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.

The following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	For EN-DC, NGEN-DC and NR-DC, SCG configuration failure (only for messages on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, SCG RRC integrity check failure (on SRB3).

Upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE can not maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

---------------------------------------------------------Change End-------------------------------------------------