3GPP TSG-RAN2 Meeting #106	R2-1905891

# Reno, USA, 13th May – 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0639 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PUCCH spatial relation Activation/Deactivation MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is some misalignment between RRC and MAC specs for “PUCCH spatial relation Activation/Deactivation MAC CE”. In 38.331, the value range of PUCCH-SpatialRelationInfoId is (1…8). However, in 38.321, it is stated “Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i”. And the value range of subscript of Si is (0…7). So for example, if RRC configures a sequence of PUCCH-SpatialRelationInfoId = 3 and 8, MAC would activate PUCCH-SpatialRelationInfoId 3 via below MAC CE according to the sentence “Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i”: But it is not possible to activate PUCCH-SpatialRelationInfoId 8 with current MAC CE mapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised as “Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i – 1.” Impact analysis: Impacted 5G architecture options: Both EN-DC and standalone. Impacted functionality: PUCCH spatial relation Activation/Deactivation MAC CE Inter-operability: - If the network is implemented according to the CR and the UE is not, there is different understandings on the activation status of PUCCH Spatial Relation Info between gNB and UE. - If the UE is implemented according to the CR and the network is not, there is different understandings on the activation status of PUCCH Spatial Relation Info between gNB and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUCCH-SpatialRelationInfoId 8 cannot be activated by PUCCH spatial relation Activation/Deactivation MAC CE even if it is configured by RRC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 6.1.3.18	PUCCH spatial relation Activation/Deactivation MAC CE

The PUCCH spatial relation Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	PUCCH Resource ID: This field contains an identifier of the PUCCH resource ID identified by PUCCH-ResourceId as specified in TS 38.331 [5]. The length of the field is 7 bits;

-	Si: If there is a PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i as specified in TS 38.331 [5], configured for the uplink bandwidth part indicated by BWP ID field, Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i – 1, otherwise MAC entity shall ignore this field. The Si field is set to 1 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i – 1 shall be activated. The Si field is set to 0 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i – 1 shall be deactivated. Only a single PUCCH Spatial Relation Info can be active for a PUCCH Resource at a time;

-	R: Reserved bit, set to 0.

Figure 6.1.3.18-1: PUCCH spatial relation Activation/Deactivation MAC CE

End of change