3GPP TSG RAN WG2 #106	R2-1906014

Reno, Nevada, USA, 13th – 17th May 2019	Resubmission of R2-1904552

Agenda Item:	11.10.3 Early measurement reporting

Source:	Ericsson

Title:	Handling of idle/inactive measurement configuration during 2-step Resume/Release

Document for:	Discussion, Decision

# Introduction

The WID “DC and CA enhancements” was approved in RAN#80 in RP-181469, and updated in RAN#81 in RP-182076 and the again in RAN#83 in RP-190452.One of the objectives is the early and fast reporting of measurements information availability from neighbor and serving cells to reduce the delay of setting up MR-DC and/or CA [1].

In RAN2#105bis the following was agreed:

Agreement

1	For NR IDLE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is setup will be supported.

2	For both LTE and NR, sending full idle mode measurements before security activation shall not be allowed.

FFS if some measurement information (detail TBD) related to idle mode measurements can be sent before security activation.

3	SMC and SMC complete messages will not be modified to enable the signalling of early measurements.

4	For both LTE and NR, RAN2 confirm that current specification allow that UEInformationRequest (or equivalent message to be specified in NR) can be sent by the network immediate after Security Mode Command without network having to wait for  Security Mode Complete (i.e. similar to sending of Reconfiguration after SMC)

5	For NR INACTIVE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is resumed will be supported.

6	Sending early measurement report is network controlled

7	For NR INACTIVE, the network can request early measurement report in RRCResume

8	For NR INACTIVE, early measurement reporting can be sent in RRCResumeComplete

FFS Whether agreements 7 and 8 should be applied to LTE RRCConnectionResume and RRCConnectionResumeComplete message.

This paper focus on the INACTIVE state solution. This paper discusses a new case in NR where the UE may receive an RRC Release message in response to an RRC Resume Request (2-step resume procedure).

# Discussion

In LTE Rel-15, the solution relies on a measurement configuration provided in the RRCConnectionRelease message, as shown below:

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond UP-EDT

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

MeasIdleConfigDedicated-r15 ::= SEQUENCE {

measIdleCarrierListEUTRA-r15	EUTRA-CarrierList-r15				OPTIONAL,	-- Need OR

measIdleDuration-r15		ENUMERATED {sec10, sec30, sec60, sec120,

sec180, sec240, sec300, spare},

...

}

EUTRA-CarrierList-r15 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierEUTRA-r15

MeasIdleCarrierEUTRA-r15::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

allowedMeasBandwidth-r15			AllowedMeasBandwidth,

validityArea-r15					CellList-r15					OPTIONAL,	-- Need OR

measCellList-r15					CellList-r15					OPTIONAL,	-- Need OR

reportQuantities					ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r15				SEQUENCE {

idleRSRP-Threshold-r15				RSRP-Range					OPTIONAL,	-- Need OR

idleRSRQ-Threshold-r15				RSRQ-Range-r13				OPTIONAL	-- Need OR

}																	OPTIONAL, 	-- Need OP

...

}

CellList-r15 ::=		SEQUENCE (SIZE (1.. maxCellMeasIdle-r15)) OF PhysCellIdRange

Upon entering IDLE the UE stores that configuration in a UE variable and perform idle measurements accordingly.

**********************************************************************************************

. . .

1>	if the RRCConnectionRelease message includes the measIdleConfig:

2>	clear VarMeasIdleConfig and VarMeasIdleReport;

2>	store the received measIdleDuration in VarMeasIdleConfig;

2>	start T331 with the value of measIdleDuration;

2>	if the measIdleConfig contains measIdleCarrierListEUTRA:

3>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

2>	else:

3>	store the measIdleCarrierListEUTRA received in SIB5 in VarMeasIdleConfig;

2>	start performing idle mode measurements as specified in 5.6.20;

. . .

**********************************************************************************************

While the timer T331 is running, the UE shall perform idle measurements according to that configuration.  According to LTE RRC (TS 36.331), the UE is not required to perform these measurements if the timer is not running, which occurs in the following cases:

UE reselects to a cell not within the configured validity area, then timer T331 stops; or

Timer T331 expires;

UE receives an RRCConnectionResume or an RRCConnectionSetup message i.e. UE enters RRC_CONNECTED;

This is summarized in the RRC specifications of LTE in the following table:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T331 | Upon receiving RRCConnectionRelease message including measIdleConfig. | Upon receiving RRCConnectionSetup, RRCConnectionResume or, if validityArea is configured, upon reselecting to cell that does not belong to validityArea. | Release the stored VarMeasIdleConfig. |

When the timer expires or is stopped (i.e. in the cases above), the UE releases the measurement configuration used for the idle measurements i.e. it releases the UE variable where the configuration is stored (VarMeasIdleConfig), as shown below:

**********************************************************************************************

5.6.20.3	T331 expiry or stop

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig;

. . .

**********************************************************************************************

In LTE Rel-15 solution for early measurements, UE releases its measurement configuration upon reception of RRCConnectionResume or RRCConnectionSetup.

Handling of inactive/idle measConfig in 2-steps Resume

The existing solution specifies that the UE stops performing idle measurements (i.e. to support early measurements reporting) upon cell reselection to a cell not in the UE’s configured validity area, upon the reception of RRCConnectionSetup or the reception of RRCConnectionResume.

However, in NR, a first new scenario is when an RRC_INACTIVE UE transmits an RRCResumeRequest or RRCResumeRequest1 message and receives an RRCRelease message with suspend configuration (indicating that the UE shall remain in RRC_INACTIVE, which is equivalent to do a very quick transient transition to RRC_CONNECTED and then to RRC_INACTIVE again) in response, as shown below:

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

A second new scenario is when an RRC_INACTIVE UE transmits an RRCResumeRequest and receives an RRCRelease without a suspend configuration (indicating that the UE shall transition to RRC_IDLE, which is equivalent to do a very quick transient transition to RRC_CONNECTED and then to RRC_IDLE) in response, as shown below:

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

There are different cases where the network may respond a Resume Request message (RRCResumeRequest or RRCResumeRequest1 as shown in the flows above) with an RRCRelease message, such as when the resume procedure is triggered due to a mobility-based RAN area Update, or periodic RAN area Update or when the network wants to redirect the UE to another frequency (in the same RAT or different RAT).

In NR an INACTIVE UE trying to resume may receive an RRCRelease in response to an RRCResumeRequest message.

As it has been agreed to support early measurements in NR (e.g. according to a configuration provided in RRCRelease or obtained in a SIB), when a UE tries to resume (i.e. it sends an RRC Resume Request message) and receives in response a new RRCRelease message that may or may not contain a configuration for early measurements, it remains unclear what the UE shall do with the stored configuration for early measurements, the timer controlling the measurements, the measurement results, and whether the UE shall continue to perform measurements or not.

If the UE receives an RRCRelease message in response to an RRCResumeRequest message which includes idle/inactive measurements, the UE shall apply these configurations and perform measurements accordingly

RAN2 to discuss whether the reception of an RRCRelease without an idle/inactive measurement configuration is an implicit indication for the UE to release any previous idle/inactive measurement configuration or an explicit release indication is required.

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE Rel-15 solution for early measurements, UE releases its measurement configuration upon reception of RRCConnectionResume or RRCConnectionSetup.

Observation 2	In NR an INACTIVE UE trying to resume may receive an RRCRelease in response to an RRCResumeRequest message.

Based on the discussion in section 2 we propose the following:

Proposal 1	If the UE receives an RRCRelease message in response to an RRCResumeRequest message which includes idle/inactive measurements, the UE shall apply these configurations and perform measurements accordingly

Proposal 2	RAN2 to discuss whether the reception of an RRCRelease without an idle/inactive measurement configuration is an implicit indication for the UE to release any previous idle/inactive measurement configuration or an explicit release indication is required.

# References

RP-190452, “Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, RAN #83