3GPP TSG-RAN WG2 Meeting #106	R2-1906098

# Reno, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1028 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handover from NR to LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the UE performs the handover from NR to LTE, the target eNB generates handoverCommandMessage which contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message as follows. TS 36.331 – HandoverCommand This message is used to transfer the handover command generated by the target eNB. Direction: target eNB to source eNB/ source RAN HandoverCommand message -- ASN1START HandoverCommand ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ handoverCommand-r8 HandoverCommand-r8-IEs, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } HandoverCommand-r8-IEs ::= SEQUENCE { handoverCommandMessage OCTET STRING (CONTAINING DL-DCCH-Message), nonCriticalExtension SEQUENCE {} OPTIONAL } -- ASN1STOP HandoverCommand field descriptions. handoverCommandMessage Contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message used to perform handover within E-UTRAN or handover to E-UTRAN, generated (entirely) by the target eNB.. However in the description of MobilityFromNRCommand message, the description of targetRAT-MessageContainer states it is just RRCConnectionReconfiguration message, not entire DL-DCCH-Message. As the source gNB will not change the content of targetRAT-MessageContainer, the inconsistence between LTE and NR will result in the incorrect message decoding in UE side, and make the handover from NR to LTE failed. TS 38.331 MobilityFromNRCommand targetRAT-Type; Standard to apply; targetRAT-MessageContainer. eutra; 3GPP TS 36.331 [10] (clause 5.4.2); RRCConnectionReconfiguration. |
| Summary of change: |  | - Clarify that the targetRAT-MessageContainer of MobilityFromNRCommand message includes the entire DL-DCCH-Message including the RRCConnectionReconfiguration message for handover to LTE. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: Handover from NR to LTE Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE may incorrectly decode the MobilityFromNRCommand message if the UE assumes that MobilityFromNRCommand only contains RRCConnectionReconfiguration message. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue, as there is an consistent understanding of targetRAT-MessageContainer content, i.e. it contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistence between the understandings of targetRAT-MessageContainer content in LTE and NR will result in the incorrect message decoding in UE side, and make the handover from NR to LTE failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

–	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA (connected to EPC or 5GC).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

-- TAG-MOBILITYFROMNRCOMMAND-START

MobilityFromNRCommand ::=       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

mobilityFromNRCommand               MobilityFromNRCommand-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=   SEQUENCE {

targetRAT-Type                          ENUMERATED { eutra, spare3, spare2, spare1, ...},

targetRAT-MessageContainer              OCTET STRING,

nas-SecurityParamFromNR                 OCTET STRING                                OPTIONAL,   -- Cond HO-ToEPC

lateNonCriticalExtension                OCTET STRING                                OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                 OPTIONAL

}

-- TAG-MOBILITYFROMNRCOMMAND-STOP

-- ASN1STOP

MobilityFromNRCommand-IEs field descriptions

nas-SecurityParamFromNR This field is used to deliver the key synchronisation and Key freshness for the NR to LTE/EPC handovers and a part of the downlink NAS COUNT as specified in TS 33.501 [11].

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

targetRAT-Type Indicates the target RAT type.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | 3GPP TS 36.331 [10] (clause 5.4.2) | The entire DL-DCCH-Message including the RRCConnectionReconfiguration message |

| Conditional Presence | Explanation |
| --- | --- |
| HO-ToEPC | This field is mandatory present in case of inter system handover. Otherwise it is absent. |

END OF FIRST CHANGE