3GPP TSG-RAN2 Meeting #106 	R2-1906785

# Reno, USA,  13 – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 1698 | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDCP profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For supportedROHC-Profiles ROHC profiles, RFC 4995 and RFC 4996 were updated to RFC 5795 and RFC 6846 respectively in CR1462. However, for supportedUplinkOnlyROHC-Profiles, it is missing to update RFC 4995 and RFC 4996 to RFC 5795 and RFC 6846 respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For supportedUplinkOnlyROHC-Profiles, RFC 4996 is replaced by RFC 6846, and RFC 4995 is replaced by RFC 5795. Impact analysis Impacted functionality: Uplink only ROHC operation Inter-operability: The UE and the network not implementing the CR may have an interoperability issue that was erroneously introduced in RFC 4995. The UE and the network implementing the CR do not have an interoperability issue that was erroneously introduced in RFC 4995. The UE and the network not implementing the CR may have a technical issue with the SACK compression as indicated in RFC 6846. The UE and the network implementing the CR do not have a technical issue with the SACK compression as indicated in RFC 6846. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - A fix of an interoperability issue in RFC 5795 cannot be implemented in the products. - A fix of technical issue with the SACK compression in RFC 6846 cannot be implemented in the products. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.3.1.6	supportedUplinkOnlyROHC-Profiles

This field defines which ROHC profile(s) from the list below are supported in uplink-only ROHC operation by the UE.

-	0x0006 ROHC TCP (RFC 6846)

A UE that supports uplink-only ROHC profile(s) shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795).