3GPP TSG-RAN WG2 Meeting #106	 R2- R2-1907182

Reno, USA, 13th - 17th May 2019	Resubmission of R2-1904397

Agenda item:	11.2.1.1

Source:	Convida Wireless

Title:	Additional RACH Opportunities for NR-U

Document for:	Discussion & Decision

# Introduction

During the NR-U SI, various options to alleviate the impacts of LBT failures on the random access procedure were discussed.  The following text related to supporting additional transmission opportunities for the RACH messages was included in TR 38.889 [1]:

“However, in order to alleviate the impact of LBT failures further, additional opportunities for the RACH messages may be introduced, e.g. in time or frequency domain, for both 4-step and 2-step RACH.”

Additional RA Msg1 opportunities was also discussed in RAN2 email discussion [105#50][NR-U] RACH 4-step and SR [2].  Four options were considered:

- additional opportunities across different LBT sub-bands within a BWP;

- additional opportunities across different UL BWPs;

- additional opportunities across serving cells/carriers; and

- wait for RAN1 progress.

The consensus view was that RAN2 can wait for further progress from RAN1 on achieving additional opportunities for Msg1.  However, there was also support for discussing this now.

In this contribution, we discuss our views related to the introduction of additional opportunities for the RACH messages and make some observations and proposals.

# Discussion

For Msg1 transmission, additional transmission opportunities in the time domain are beneficial.  However, the channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped close together in time.  Increasing the time between the transmission opportunities helps to reduce the correlation, but also increases latency.

Observation 1:  The channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped close together in time.

For wideband carriers, the channel may be partitioned into 20 MHz sub-bands, where LBT is performed on each sub-band independently [1].  Configuring additional RACH resources across multiple LBT sub-bands can provide additional robustness without increasing latency.  Therefore, for NR-U, it should be possible to configure additional RACH resources across LBT sub-bands.

Observation 2:  Configuring additional RACH resources across multiple LBT sub-bands can provide additional robustness without increasing latency.

Proposal 1:  For NR-U, it should be possible to configure additional RACH resources across LBT sub-bands.

In R1-1901040 [3], an observation was made that frequency domain enhancements would not have much benefit for initial access.  This is because for initial access, Msg1 is transmitted on the initial active UL BWP, which is likely to be ≤ 20 MHz.  Therefore, when discussing the configuration of additional RACH resources across LBT sub-bands, RAN2 should also discuss whether this will be applicable for UEs in RRC_IDLE/RRC_INACTIVE or only for UEs in RRC_CONNECTED.

Proposal 2:  When discussing the configuration of additional RACH resources across LBT sub-bands, RAN2 should also discuss whether this will be applicable for UEs in RRC_IDLE/RRC_INACTIVE or only for UEs in RRC_CONNECTED.

# Conclusion

In this contribution, we discuss our views related to the introduction of additional opportunities for the RACH messages and make the following observations and proposals:

Observation 1:  The channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped close together in time.

Observation 2:  Configuring additional RACH resources across multiple LBT sub-bands can provide additional robustness without increasing latency.

Proposal 1:  For NR-U, it should be possible to configure additional RACH resources across LBT sub-bands.

Proposal 2:  When discussing the configuration of additional RACH resources across LBT sub-bands, RAN2 should also discuss whether this will be applicable for UEs in RRC_IDLE/RRC_INACTIVE or only for UEs in RRC_CONNECTED.

# References

- 3GPP TR 38.889, Study on NR-based access to unlicensed spectrum, V16.0.0

- R2-1903282, Summary of Email discussion [105#50][NR-U] RACH 4-step and SR, OPPO

- R1-1901040 Discussion on time/freq-domain enhancements for RACH resources, Samsung