3GPP TSG-RAN2#106			       R2-1907986

Reno, U.S.A, 13th – 17th May, 2019

Agenda Item	: 11.10.4.3	 (LTE_NR_DC_CA_enh-Core)

Source	: LG Electronics Inc.

Title	: Configuring SCG in RRC Resume message and Early Measurement reporting

Document for	: Discussion and Decision

1.	Introduction

In RAN2 #105 bis meeting, the following is agreed for the early measurements signalling.

Agreement

1	For NR IDLE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is setup will be supported.

2	For both LTE and NR, sending full idle mode measurements before security activation shall not be allowed.

FFS if some measurement information (detail TBD) related to idle mode measurements can be sent before security activation.

3	SMC and SMC complete messages will not be modified to enable the signalling of early measurements.

4	For both LTE and NR, RAN2 confirm that current specification allow that UEInformationRequest (or equivalent message to be specified in NR) can be sent by the network immediate after Security Mode Command without network having to wait for  Security Mode Complete (i.e. similar to sending of Reconfiguration after SMC)

5	For NR INACTIVE mode, the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is resumed will be supported.

6	Sending early measurement report is network controlled

7	For NR INACTIVE, the network can request early measurement report in RRCResume

8	For NR INACTIVE, early measurement reporting can be sent in RRCResumeComplete

FFS Whether agreements 7 and 8 should be applied to LTE RRCConnectionResume and RRCConnectionResumeComplete message.

For NR inactive, the followings are agreed:

Observation 1. For NR inactive mode, RAN2 agreed

- the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is resumed will be supported.

- the network can request early measurement report in RRCResume

- early measurement reporting can be sent in RRCResumeComplete

According to observation 1, there are two methods for early measurement reporting in NR inactive mode. One is to re-use the LTE rel-15 euCA early measurement reporting solution. Another is early measurement report is included in RRCResume message if the network request early measurement report in RRCResume.

Observation 2. Based on the agreements, there are two methods for early measurement reporting in NR inactive mode as follows:

Method A) to re-use the LTE rel-15 euCA early measurement reporting solution.

Method B) early measurement report is included in RRCResume message if the network request early measurement report in RRCResume.

2.	Discussion

There were proposals to configure SCG and resume stored SCG configuration via RRCResume message [1][2][3]. According to RAN2 agreement in observation 2, measurement reporting cannot be delivered to the network when Resume message is sent. Thus, if the proposals is applied, the configuration and resume via RRC resume message would be blindly performed by the network.

Observation 3. Proposals to configure SCG and resume stored SCG configuration via RRCResume message are blind operation without measurement reporting.

If measurement reporting is provided in RRCResumeComplete message, it is most likely the proposal can be supported without problems due to blind operation. However, if delivery of measurement reporting is delayed (e.g. according to Method A)), problem due to blind operation increases. For example, upon receiving the RRCResume message to configure SCG and/or resume stored SCG configuration, the UE configure/resume SCG configuration but the SCG configuration may not be valid (e.g. RSRP of the PSCell in the SCG configuration is not enough to perform transmission/reception). Then, transmission toward the SCG would be failed. It can cause radio link failure or reconfiguration with sync failure. The SCG failure information can be triggered to resolve it.

However, the probability to perform the behaviour could be increased as blind operation/configuration increases and such behaviour could be burden to the UE because the UE should perform SCG configuration/resume as well as transmission/operation including SCGFailureInformation message toward invalid SCG while performing resume procedure. Additionally, the behaviour causes the delay to receive new SCG configuration instead of invalid SCG by the network because the time is required to perform the transmission toward invalid SCG and reporting the problem to the network.

Observation 4. Blind operation via RRCResume message without measurement reporting in RRCResumeComplete message can cause unnecessary transmission/operation toward invalid SCG to UE.

Proposal 1. RAN agree to configure SCG and resume stored SCG configuration via RRCResume message only if measurement reporting is provided in RRCResumeComplete message.

Proposal 2. RAN discuss ways to resolve the problem due to blind operation if it need to be support for configuring SCG and resuming stored SCG configuration via RRCResume message without measurement reporting in RRCResumeComplete message.

3.	Conclusion

In this contribution, we provided observations and proposals about how to deal with configure SCG and resume stored SCG configuration via RRCResume message by considering early measurement reporting procedure.

Observation 1. For NR inactive mode, RAN2 agreed

- the LTE rel-15 euCA early measurement reporting solution (i.e. via UEInformationRequest and UEInformationResponse like messages) after connection is resumed will be supported.

- the network can request early measurement report in RRCResume

- early measurement reporting can be sent in RRCResumeComplete

Observation 2. Based on the agreements, there are two methods for early measurement reporting in NR inactive mode as follows:

Method A) to re-use the LTE rel-15 euCA early measurement reporting solution.

Method B) early measurement report is included in RRCResume message if the network request early measurement report in RRCResume.

Observation 3. Proposals to configure SCG and resume stored SCG configuration via RRCResume message are blind operation without measurement reporting.

Observation 4. Blind operation via RRCResume message without measurement reporting in RRCResumeComplete message can cause unnecessary transmission/operation toward invalid SCG to UE.

Proposal 1. RAN agree to configure SCG and resume stored SCG configuration via RRCResume message only if measurement reporting is provided in RRCResumeComplete message.

Proposal 2. RAN discuss ways to resolve the problem due to blind operation if it need to be support for configuring SCG and resuming stored SCG configuration via RRCResume message without measurement reporting in RRCResumeComplete message.

References

[1] R2-1904489, Configuring or resuming MR-DC in RRC Resume

[2] R2-1904491, Configuring or resuming EN-DC in RRC Resume

[3] R2-1904252, On SCG configuration with resume procedure