3GPP TSG-RAN WG2 Meeting #106	R2-1908433

Reno, USA, 13th - 17th May 2019                                                    Revision of R2-1906463

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1046 | rev | 1 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on capability of maxUplinkDutyCycle for FR2 for 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, OPPO, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-25 |
| Category: | F· |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#90 meeting in February, RAN4 agreed the LS on FR2 maxUplinkDutyCycle capability values (R4-1902489) and sent the LS to inform RAN2 on the signalling impact. The content of LS is provided as below: RAN4 has discussed the value of maxUplinkDutyCycle which is the optional signalling UE capability of maximum percentage of uplink transmission time that can be scheduled within 1s time window in order to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The optional values are {2%, 10%, 20%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. The default value is FFS.. In RAN #83 meeting in March, the issue of minimum value 2% was raised and the FR2 maximum UL duty cycle was discussed in RAN plenary meeting. According to the agreed way forward (RP-190755) RAN would like to inform RAN4 and RAN2 that it has updated the FR2 maxUplinkDutyCycle capability values as follows {2%, 10% (TBD), 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%} RAN4 is asked whether any value <= 10% should be added RAN asks - RAN2 to introduce FR2 maxUplinkDutyCycle capability signalling accordingly - RAN2 to evaluate backwards compatibility. In RAN2#106 meeting, RAN2 received LS from RAN4 on FR2 maxUplinkDutyCycle (R2-1908432 ) RAN4 has continued the discussion on maxUplinkDutyCycle-FR2 for FR2 UE capability after sending LS [1] earlier and made the following agreements: - Evaluation period for UE capability maxUplinkDutyCycle-FR2 for is 1 seconds - 15 % value should be allowed for UE capability maxUplinkDutyCycle-FR2 in addition to values what were in WF from plenary [2] i.e. possible declarable values are 15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90% and 100% - If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c - If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. - maxUplinkDutyCycle-FR2 is applicable for all power classes in FR2 With these agreements, RAN4 considers work for maxUplinkDutyCycle-FR2 to be completed. |
| Summary of change: |  | Explian the changes A UE capability of maxUplinkDutyCycle-FR2 with values {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%} is introduced to indicate the maximum percentage of uplink symbols can be scheduled within 1s so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for FR2 power class 3 UE as specified in TS38.101. Value n15 corresponds to 15%, value 20 corresponds to 20% and so on. Impact analysis Impacted functionality: The changes only impact FR2 UEs. (Standalone NR only) Inter-operability: - If the network is implemented according to the CR and the UE is not, network cannot know when UE will do power back off in order to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. - If the UE is implenented according to the CR and the network is not, network scheduler will not consider the maxUplinkDutyCycle in order to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies, and UE could do power back off when the percentage of uplink transmission within 10msec is larger than its capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No capability of maxUplinkDutyCycle for FR2 is supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 UE capability information elements, RF parameters |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS38.306 CR R2-1906464 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************************************* Beginning of changes *******************************************

6.3.3	UE capability information elements

<Partially omitted>

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                   SEQUENCE {

supportedBandListNR                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550  BandCombinationList-v1550                   OPTIONAL

]]

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                       OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                      OPTIONAL,

extendedCP                          ENUMERATED {supported}                      OPTIONAL,

multipleTCI                         ENUMERATED {supported}                      OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                      OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                   OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                          OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                      OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                      OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                      OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}             OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                      OPTIONAL,

channelBWs-DL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))               OPTIONAL

}

}                                                                               OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}           OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                      OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}                                 OPTIONAL,

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

************************************ End of changes ************************************************