3GPP TSG-RAN WG2 Meeting #107                                                 	R2-1908765

Prague, Czech Republic, 26th - 30th Aug 2019

Source:	CATT

Title:	RACH Report Optimization

Agenda Item:	11.12.4

Document for:	Discussion and Decision

Introduction

In RAN2#106 meeting, some enhancements have been agreed for RACH report [1]:

Agreements:

1	Include SSB related information consisting of SSB index and number of preambles sent for each tried SSB in the RACH information report.

Also in last RAN3 meeting, the draft requirements for RACH report optimization were merged into TR37.816 [2]:

The contents of the RACH information report comprises of the following (further check by RAN2 is needed):

- Indexes of the SSBs and number of RACH preambles sent on each tried SSB   listed in chronological order of attempts

- The frequency (NR ARFCN) of tried SSBs

- The beam quality of each tried SSB (i.e., beam level measurement during RACH attempts such as BRSRP, BRSRQ, BSINR)

- Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold

- Elapsed time from the last measurement prior to the beam selection time

- Number of RACH preambles sent on SUL

- Number of RACH preambles sent on NUL

- Total number of fallbacks between Contention Based RACH Access (CBRA) and Contention Free RACH Access (CFRA)Contention detection indication

The above RACH information report should also applied to the SN node for MR-DC case.

From the information above, we can find the requirements for RACH report optimization from RAN3 need further check by RAN2 first, so in this contribution, we’d like to confirm the RACH use case applied for RACH report optimization first and check the requirements for RACH report optimization from RAN3 and then give our suggestions for the contents of the RACH report.

Discussion

  - Check the scenario applied for RACH Report

Many use cases can trigger RACH procedure, as NR introduce more use case for RACH, it’s better to check which use case can be included in RACH report first and the discuss how to include all the applicable use case into the RACH report. To simplify the discussion, we list the possible RACH use cases according to [3]:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	Handover;

-	DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised";

-	UL data arrival during RRC_CONNECTED when there are no PUCCH resources for SR available;

-	SR failure;

-	Request by RRC upon synchronous reconfiguration;

-	Transition from RRC_INACTIVE;

-	To establish time alignment at SCell addition;

-	Request for Other SI (see subclause 7.3);

-	Beam failure recovery

Any of the above use cases can trigger RACH procedure and any RACH attempt may fail even for dedicated RACH resources, so we think all the RACH use cases listed in 38.300 are applicable for RACH report optimization.

Proposal 1: All the RACH use cases listed in 38.300 are applicable for RACH report optimization.

The next question is how these RACH use case can be included in the RACH report.

“Request for Other SI” is a new RACH use case for NR and UE only initiates RACH for SI request in IDLE/INACTIVE mode in NR R15. For detail, it can be classified into MSG1 based SI request and MSG3 based SI request. For MSG1 based SI request procedure, dedicated RACH resources are configured in SIB1 and UE will use the dedicated RACH resources to trigger SI request procedure, but for MSG3 based SI request, the common RACH resources are used to trigger RACH procedure, so there is no need to differentiate the RACH optimization between Initial access from RRC_IDLE/ Transition from RRC_INACTIVE use case and MSG3 based SI request use case.

Proposal 2: There is no need to differentiate the RACH optimization content in RACH Report between “Initial access from RRC_IDLE”/ “Transition from RRC_INACTIVE” use case and “MSG3 based SI request” use case.

The RACH report record for MSG1 based SI request is independent compared to that for Initial access from RRC_IDLE/Transition from RRC_INACTIVE use case and the RACH report record for Initial access from RRC_IDLE/Transition from RRC_INACTIVE use case will always cover the RACH report record for MSG1 based SI request if only one RACH report entry is included into the RACH report.

Even for the same BWP resources, the configured RACH configuration still can be different among different RACH scenario, for example, avoid RACH collision on the same BWP resources or some RACH scenarios has high priority than other RACH scenarios. From network perspective, the network may want to know the RACH scenario in which the UE collected the RACH report data.

So we propose the following:

Proposal 3: A list of RACH report entry can be included in one RACH report, each entry of the RACH report can be identified by a cause value.

Proposal 4: The RACH report entry for “MSG1 based SI request” use case is identified by a specific cause value.

As for “Beam failure recovery” case, this procedure is transparent to RRC if BFR is successful and a new RACH procedure, e.g. HO procedure may be triggered followed the Beam failure recovery before the network retrieves the RACH report record for “Beam failure recovery”. The network will be quite confused if the different RACH report records for “Beam failure recovery” and for “HO” case have the same cause value. In addition, dedicated RACH resources may be configured for “Beam failure recovery” case. So it will be beneficial if the RACH report record for “Beam failure recovery” has a specific cause value.

Proposal 5: The RACH report entry for “Beam failure recovery” use case is identified by a specific cause value.

The RACH report record for “To establish time alignment at SCell addition” use case is used to optimize the RACH resources in SN, so it’s nature to differentiate this RACH optimization use case with other RACH use case within MN.

Proposal 6: The RACH report entry for “To establish time alignment at SCell addition” use case is identified by a specific cause value.

For the RACH cases apart from P4~P6, we think we can follow the LTE system in which no identifier is introduced to differ the cases. So we propose:

Proposal 7: Apart from “MSG1 based SI request”, “Beam failure recovery” and “To establish time alignment at SCell addition” use cases, all the other RACH cases share the same cause value when generating a RACH report entry.

  - Check the Requirements for RACH Report Optimization from RAN3 LS

In last RAN2 meeting, RAN3 sent an LS to RAN2 [4], in which RAN3 requested RAN2 to check all the requirements on RACH Report Optimization captured in [2]. We’d like to check the requirements one by one as the following:

“Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts” is the requirement from RAN3, which is almost aligned with the agreement of RAN2 #106 meeting. So we propose:

Proposal 8: RAN2 confirms the indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts will be included in R16 RACH report.

As for the second requirement on RACH Report listed in TR37.816, we think the requirement is not clear enough. The RACH resources are configured on uplink BWP and the UL and SUL can be configured with separate uplink BWP. UE may retry the RACH procedure on UL/SUL after the RACH failure on SUL/UL. As the RACH resources for UL and SUL can be configured separately, it’s better to collect the RACH report data for UL and SUL separately.

Proposal 9: The frequency (NR ARFCN) of tried SSBs as well as the active BWP band and RACH carrier info on this BWP band for UL and/or SUL should be included in one RACH report entry.

The number of preambles sent on UL/SUL is not enough to do RACH optimization as the active BWP resources as well as the linked RACH resources are configured per UE. While in LTE, all the UEs in the same cell share the same bandwidth as well as the common RACH resources, there is no need to differ the RACH problem among UEs as the optimization object is the same.

Observation: In NR, RACH resources are configured per BWP even configured per BWP per UE; while in LTE, RACH resources are configured per cell, which are shared by all UEs in the cell.

But in NR, BWP is configured per UE, the network will be quite confused if only the number of preambles sent on UL/SUL is collected in the RACH report. Maybe for initial BWP, anyway the network can do the RACH optimization as the initial BWP is a unique BWP per cell. But for other active BWP, at most four BWPs can be configured to one UE. In the current specification, only one BWP can be activated within UE per time. The active BWP within UE can be changed by RRC or DCI for UE in connected mode. If the network retrieves the RACH report e.g. triggered by BFR, from UE after switching the active BWP on UE, the network doesn’t know on which BWP the UE collected the RACH report. So the number of preambles sent on UL/SUL as well as the related BWP carrier info is both needed in RACH report.

Proposal 10: At least the number of preambles sent on UL and/or SUL as well as the related BWP band info on UL and/or SUL should be included in one RACH report entry. It’s FFS whether the associated RACH channel resources info should be included in one RACH report entry.

BWP ID is just a BWP identifier between the network and UE, the same BWP ID corresponding to a different BWP can be configured to UE when another RRCReconfiguration message sent to UE. So the network still can’t know which BWP is behind the BWP ID.

Proposal 11: BWP ID is not included in the RACH report.

For “beam level measurement during RACH attempts of each tried SSB”, the beam level measurement of each RACH attempt within the same SSB index may differ from that of another RACH attempt, but the quality difference is usually small as the interval between two RACH attempts is not too long, we also think the beam level measurement is useful for network to optimize RACH resources, so to balance the overhead and the beam quality requirement, we think only the beam level measurement of the last failed RACH attempt or the average beam level measurement of all RACH attempts within one SSB index is included in one RACH report entry.

Proposal 12a: Only the beam level measurement of the last failed RACH attempt within one SSB index is included in one RACH report entry for the selected SSB.

Proposal 12b: Only the average beam level measurement of all RACH attempts within one SSB index is included in one RACH report entry.

For “Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold”, UE may initiate several RACH attempts within one SSB index, some of RACH attempts may be triggered when the beam quality is above rsrp-ThresholdSSB while the others may be triggered when the beam quality is below the rsrp-ThresholdSSB, so UE can just indicate how many times the beam quality is below the rsrp-ThresholdSSB among the RACH attempts on the selected SSB.

Proposal 13: The number of times when the beam quality is below the rsrp-ThresholdSSB on the selected SSB is included in one RACH report entry.

As for “Elapsed time from the last measurement prior to the beam selection time”, the requirement is not clear.

Proposal 14: Elapsed time from the last measurement prior to the beam selection time is not included in one RACH report entry.

For “Total number of fallbacks between Contention Based RACH Access (CBRA) and Contention Free RACH Access (CFRA) Contention detection indication”, we think the motivation for introducing this requirement is to optimise parameter rsrp-ThresholdSSB, proposal 13 can cover this requirement, so we propose:

Proposal 15: Total number of fallbacks between Contention Based RACH Access (CBRA) and Contention Free RACH Access (CFRA) Contention detection indication is not included in RACH report.

RACH resources can be divided into dedicated RACH resources and common RACH resources, the network may want to know whether dedicated RACH resources are configured for a specific BWP.

Proposal 16: An indicator which represents whether dedicated RACH resources are configured for UE is included in one RACH report entry.

Proposal 17: Agree the TP for RACH Optimization.

The RACH report requirements are from RAN3 LS, so RAN2 should give the feedback if having conclusion [5].

Proposal 18: Send a reply LS to RAN3 to inform them of our agreements on RACH Optimization as well as the agreements on MRO.

Conclusion

According to the analysis in section 2, we have:

Observation: In NR, RACH resources are configured per BWP even configured per BWP per UE; while in LTE, RACH resources are configured per cell, which are shared by all UEs in the cell.

And we propose:

Proposal 1: All the RACH use cases listed in 38.300 are applicable for RACH report optimization.

Proposal 2: There is no need to differentiate the RACH optimization content in RACH Report between “Initial access from RRC_IDLE”/ “Transition from RRC_INACTIVE” use case and “MSG3 based SI request” use case.

Proposal 3: A list of RACH report entry can be included in one RACH report, each entry of the RACH report can be identified by a cause value.

Proposal 4: The RACH report entry for “MSG1 based SI request” use case is identified by a specific cause value.

Proposal 5: The RACH report entry for “Beam failure recovery” use case is identified by a specific cause value.

Proposal 6: The RACH report entry for “To establish time alignment at SCell addition” use case is identified by a specific cause value.

Proposal 7: Apart from “MSG1 based SI request”, “Beam failure recovery” and “To establish time alignment at SCell addition” use cases, all the other RACH cases share the same cause value when generating a RACH report entry.

Proposal 8: RAN2 confirms the indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts will be included in R16 RACH report.

Proposal 9: The frequency (NR ARFCN) of tried SSBs as well as the active BWP band and RACH carrier info on this BWP band for UL and/or SUL should be included in one RACH report entry.

Proposal 10: At least the number of preambles sent on UL and/or SUL as well as the related BWP band info on UL and/or SUL should be included in one RACH report entry. It’s FFS whether the associated RACH channel resources info should be included in one RACH report entry.

Proposal 11: BWP ID is not included in the RACH report.

Proposal 12a: Only the beam level measurement of the last failed RACH attempt within one SSB index is included in one RACH report entry for the selected SSB.

Proposal 12b: Only the average beam level measurement of all RACH attempts within one SSB index is included in one RACH report entry.

Proposal 13: The number of times when the beam quality is below the rsrp-ThresholdSSB on the selected SSB is included in one RACH report entry.

Proposal 14: Elapsed time from the last measurement prior to the beam selection time is not included in one RACH report entry.

Proposal 15: Total number of fallbacks between Contention Based RACH Access (CBRA) and Contention Free RACH Access (CFRA) Contention detection indication is not included in RACH report.

Proposal 16: An indicator which represents whether dedicated RACH resources are configured for UE is included in one RACH report entry.

Proposal 17: Agree the TP for RACH Optimization.

Proposal 18: Send a reply LS to RAN3 to inform them of our agreements on RACH Optimization as well as the agreements on MRO.

- 4 Reference

- RAN2#106 Chairman Notes

- TR37.816

- TS38.300

- R2-1908630, LS on RACH and Mobility Robustness Optimization, 3GPP TSG-RAN WG3 Meeting #104

Reno, USA, May 13 – May 17, 2019

- R2-1908766, [Draft] Reply LS on RACH Optimization and Mobility Robustness Optimization, 3GPP TSG-RAN WG2 Meeting #107

Prague, Czech Republic, 26 – 30 August, 2019

- 5 Text Proposal for 37.816 on RACH Report Optimization

----------------------------------------------------------------------------------------------------------------------------------------

- 5.5.2 	Solution description

The setting of RACH parameters that can be optimized are:

-	RACH configuration (resource unit allocation);

-	RACH preamble split (among dedicated, group A, group B);

-	RACH backoff parameter value;

-	RACH transmission power control parameters.

As a minimum, RACH optimization is realized by UE providing RACH related information report to the NG RAN node, and by exchange of PRACH configuration of normal UL carrier and SUL carrier between NG RAN node.

For CU-DU architecture, gNB-DU should be allowed to report its RACH configuration per cell to the gNB-CU, and the gNB-CU should be allowed to signal the RACH configuration per served cell to neighbouring NG RAN nodes. This allows NG-RAN nodes to identify whether RACH configurations of neighbouring cells are optimized or whether changes are needed in order to achieve a better RACH coordination between neighbouring cells.

Upon receiving the polling message requesting RACH report, e.g., UE Information Request message, from the NG RAN node (potentially gNB-CU of the current serving cell), UE reports RACH information within a UE Information Response message. The gNB-CU and gNB-DU take into account the RACH report and other node information, to achieve an optimized RACH configuration.

The contents of the RACH information report comprises of the following:

- Indexes of the SSBs and number of RACH preambles sent on each tried SSB   listed in chronological order of attempts

- The frequency (NR ARFCN) of tried SSBs as well as the active BWP band and RACH carrier info on this BWP band for UL and/or SUL

- The average beam quality of each tried SSB (i.e., beam level measurement during RACH attempts such as BRSRP, BRSRQ, BSINR)

- The number of times when the beam quality is below the rsrp-ThresholdSSB on the selected SSB

- Number of RACH preambles sent on SUL

- Number of RACH preambles sent on NUL

- An indicator which represents whether dedicated RACH resources are configured for UE

The above RACH information report should also applied to the SN node for MR-DC case.