3GPP TSG-RAN2 Meeting #107	R2-1909192

Prague, CZ, 26th – 30th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR to 37.340 for CA/DC enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo(Rapporteur), Ericsson (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture the RAN2 agreements on LTE_NR_DC_CA_enh-Core WI: RAN2#105 agreements: - Rel-16 early measurement configuration may contain both NR and LTE configuration, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. - MCG failure can be indicated to the network via the SCG. FFS if via SCells. RAN2#105bis agreements: - When MCG failure occurs, UE follows SCG failure-like procedure: - UE does not trigger RRC connection re-establishment. - UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG. - MCG fast recovery targets the following use cases MCG leg RLF - MCG failure indication should include: - Available measurement results of MCG - MCG link failure cause - Available measurement results of SCG - Available measurement results of non-serving cells - SCG leg of the split SRB1 can be used for MCG fast recovery. RAN2#106 agreements: - FFS Whether a guard timer is needed for the MCG failure indication message - Once the MCG failure indication is triggered, the UE shall: - transmit the MCG failure indication; - suspend MCG transmission for all SRBs and DRBs; - reset MCG-MAC; - maintain the current measurement configurations from both the MN and the SN, and continue measurements based on configuration from the MN and the SN if possible. - FFS whether switch the primaryPath to SCG is needed - If SCG failure is detected while MCG is suspended then initiate RRC re-establishment procedure - Upon receiving the MCG failure indication, the MN sends reconfiguration with sync or RRC Release to the UE via SRB1. - Upon reception of reconfig with sync the UE resumes MCG transmission if suspended |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreements captured in stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 Measurements 7.7 SCG/MCG failure handling |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

7.2	Measurements

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in sub-clause 10.2, the Master node should configure the measurement to the UE.

In case of the intra-secondary node mobility described in sub-clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required.

The Secondary Node Change procedure described in sub-clause 10.5 can be triggered by both the MN (only for inter-frequency secondary node change) and the SN. For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN.

Measurements can be configured independently by the MN and by the SN (intra-RAT measurements on serving and non-serving frequencies). The MN indicates the maximum number of frequency layers and measurement identities that can be used in the SN to ensure that UE capabilities are not exceeded. If MN and SN both configure measurements on the same carrier frequency then the configurations need to be consistent (if the network wants to ensure these are considered as a single measurement layer). Each node (MN and SN) can configure independently a threshold for the SpCell quality: when the PCell quality is above the threshold configured by the MN, the UE is still required to perform inter-RAT measurements configured by the MN on the SN RAT (while it's not required to perform intra-RAT measurements); when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN.

NOTE:	The SN cannot renegotiate the number of frequency layers allocated by the MN in this version of the protocol.

When SRB3 is not configured, reports for measurements configured by the SN are sent on SRB1. When SRB3 is configured, reports for measurements configured by the SN are sent on SRB3.

Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure. Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure. Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure.

Measurement results according to measurement configuration from the MN are encoded according to SN RRC when they are provided by MN to SN in SgNB Addition Request message / SN Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to SN RRC when they are provided by MN to SN in SgNB Addition Request message / SN Addition Request message.

Per-UE or per-FR measurement gaps can be configured, depending on UE capability to support independent FR measurement and network preference. Per-UE gap applies to both FR1 (E-UTRA and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. The UE may also be configured with a per-UE gap sharing configuration (applying to per-UE gap) or with two separate gap sharing configurations (applying to FR1 and FR2 measurement gaps respectively) [8].

In the following scenarios, a measurement gap configuration is always provided in the following scenarios:

-	For UEs configured with E-UTRA inter-frequency measurements;

-	For UEs that support either per-UE or per-FR gaps, when the conditions to measure SSB based inter-frequency measurement or SSB based intra-frequency measurement as described in section 9.2.4 in TS38.300 [3] are met;

If per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, in EN-DC and NGEN-DC, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2; in NE-DC and NR-DC, the MN decides both the FR1 and FR2 gap patterns and the related gap sharing configurations.

In EN-DC and NGEN-DC, the measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN. For the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE.

In NE-DC, the MN indicates the configured per-UE or FR1 measurement gap pattern to the SN. The SN can provide a gap request to the MN, without indicating any list of frequencies.

In NR-DC, the MN indicates the configured per-UE, FR1 or FR2 measurement gap pattern and the gap purpose to the SN. The SN can indicate to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE.

When a UE is released to RRC_IDLE, RRC_IDLE with suspended RRC connection or RRC_INACTIVE, it can be configured to perform idle mode measurements. Idle mode measurements can also be configured via SIB. The idele mode measurement configuration provided n RRCRelease can be different from measurement configuration received in SIB and configuration provided in RRCRelease overrides those received in SIB (if any). The idle mode measurement configuration may contain E-UTRA and/or NR measurement configurations.

END OF CHANGES

START OF CHANGES

7.7	SCG/MCG failure handling

If radio link failure is detected for MCG, and fast MCG link recovery can be used, the UE can report the failure via SCG, if configured. Otherwise, the UE initiates the RRC connection re-establishment procedure.

The following MCG failure case triggers fast MCG link recovery:

-	MCG RLF;

During fast MCG link recovery, the UE suspends MCG transmissions for all radio bearers and reports the MCG Failure Information to the MN via the SCG, using the SCG leg of split SRB1 (if configured), instead of triggering re-establishment.

Editor's note: If MCG failure information can be sent via SRB3 is FFS.

The UE includes in the MCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN. Once the MCG failure indication is triggered, UE maintains the current measurement configurations from both the MN and the SN, and continue measurements based on configuration from the MN and the SN, if possible.

Editor's note: It is FFS whether MCG failure can be reported via SCells.

Editor's note: FFS whether a guand timer is needed for MCG Failure Indication

Editor's note: FFS whether UE needs to switch the primaryPath to SCG

Upon reception of the MCG Failure Indication, MN can send RRC Rconfiguration with sync meaasge or RRC Release message to UE, using the SCG leg of split SRB1. At reception of RRC Rconfiguration with sync meaasge, UE can resume MCG transmissions for all radio bearers, if suspended.

The following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	For EN-DC, NGEN-DC and NR-DC, SCG configuration failure (only for messages on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, SCG RRC integrity check failure (on SRB3).

Upon SCG failure, if MCG transmissions of radio bearers are not suspended, the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment. If SCG failure is detected while MCG transmissions of radio bearers are suspended, then UE will initiate RRC re-establishment procedure.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE can not maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN

END OF CHANGES

Annex (not part of the specification): RAN2 Agreements

This Annex contains the RAN2 agreements on Rel-16 WI for “DC and CA enhancements”. The agreements are provided verbatim for reference.This annex shall be removed once the WI is completed.

RAN2#105

Agreements:

For IDLE/INACTIVE

- Rel-16 early measurement configuration may contain both NR and LTE configuration, only NR configuration or only LTE configuration, to support various MR-DC and CA scenario. FFS on details.  IDLE mode and INACTIVE mode details will be discussed separately

- NR early measurement configuration should include NR specific measurement parameters configurations.

- Available beam and cell level measurement results can be included in early measurement reporting if configured.

Agreements:

•	MCG failure can be indicated to the network via the SCG. FFS if via SCells.

•	FFS how the failure is indicated, which SRBs, and which failure case the fast MCG failure recovery.

•	We will aim to have a unified solution for the failure cases that we want to address.

RAN2#105bis

Agreements for MCG fast recovery:

0	MCG fast recovery targets all MRDC architecture options

1:	When MCG failure occurs, UE follows SCG failure-like procedure:

i.	UE does not trigger RRC connection re-establishment.

ii.	UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG.

2: 	MCG fast recovery targets the following use cases MCG leg RLF

FFS: Other uses cases. Can consider in future whether the mechanism can be also be applied in the case of other MCG failures.

3	MCG fast recovery can only be triggered after AS security has been activated and the SRB2 and at least one DRB have been setup

4	MCG failure indication should include:

i.	Available measurement results of MCG

ii.	MCG link failure cause

iii.	Available measurement results of SCG

iv.	Available measurement results of non-serving cells

5: 	For MCG failure indication, new RRC message in introduced, e.g. MCGFailureInformation.

6: 	SCG leg of the split SRB1 can be used for MCG fast recovery.

FFS: If configured, SRB3 can be used for MCG fast recovery. Priority is to complete the solution based on split SRB1

7:	New SRB is not introduced for MCG fast recovery

RAN2#106

Agreements

1: Fast MCG recovery is not supported in case (intra and inter-RAT) handover failure

2: Fast MCG recovery is not supported in case of integrity check failure

3: Fast MCG recovery is not supported in case of RRC connection reconfiguration failure

Agreements

FFS Whether a guard timer is needed for the MCG failure indication message

1	Once the MCG failure indication is triggered, the UE shall:

-	transmit the MCG failure indication;

−	suspend MCG transmission for all SRBs and DRBs;

−	reset MCG-MAC;

−	maintain the current measurement configurations from both the MN and the SN, and continue measurements based on configuration from the MN and the SN if possible.

FFS whether switch the primaryPath to SCG is needed

2	If SCG failure is detected while MCG is suspended then initiate RRC re-establishment procedure

3	Upon receiving the MCG failure indication, the MN sends reconfiguration with sync or RRC Release to the UE via SRB1.

4	Upon reception of reconfig with sync the UE resumes MCG transmission if suspended