3GPP TSG RAN WG2 #107	 R2-1909325

Prague, Czech Republic, 26 – 30th August 2019                       Resubmission of R2-1906202

Agenda Item:	12.3.3.1 Execution Details of CHO

Source: 	Ericsson

Title:  	Security implications of Conditional handover in LTE

Document for:	Discussion and Decision

# Introduction

In the work item for LTE Mobility Enhancements [1], one objective is to improve the robustness at handover and for that objective the support of Conditional handover (CHO) has been agreed.

It has also been agreed that the CHO configuration consists of a trigger condition configuration and an RRCConnectionReconfiguration with mobilityControlInfo (or equivalent content) per target cell candidate. In this contribution, we discuss the security implications of CHO configurations provisions with/without security.

# Discussion

## CHO configuration

If source node prepares multiple cells for CHO, the UE needs to receive an RRC configuration for each target cell candidate. That is basically the content of an RRCConnectionReconfiguration message containing mobilityControlInfo for each target cell candidate. Associated to each of these configurations, the UE receives a triggering condition configuration (e.g. like an A3 even configuration in reportConfig). Hence, the signalling for the CHO configurations needs to consider the possibility to configure multiple target cell candidates.

Then, CHO configuration has two components, for each target candidate cell:

- Triggering condition (e.g. like an A3/A5 event);

- A dedicated RRC configuration prepared by a target candidate;

The triggering condition should be something like an Ax event configuration consisting of thresholds, measurement quantities, time-to-trigger values, etc. This would be equivalent to what exists in ReportConfig IE. And, associated to that event-like configuration, there should be a measurement object or at least a carrier frequency so that the UE knows which frequency and possibly cells to measure to monitor the fulfilment the configured conditions. Hence, one could say that the first component of the CHO matches what is currently transmitted in a measurement configuration.

The triggering conditions in CHO contains information equivalent to the ones in measurement configuration.

The dedicated RRC configurations prepared by each target candidate should be similar to a handover command i.e. an RRCConnectionReconfiguration with mobilityControlInfo, containing what is required to the UE to access a target cell fulfilling the configured CHO triggering condition.

The dedicated RRC configuration(s) in CHO contains information equivalent to the ones in an RRCConnectionReconfiguration including mobilityControlInfo.

## Security requirements

In LTE, the UE in CONNECTED state may receive a measurement configuration before security is activated e.g. when the UE performs a transition from IDLE to CONNECTED and network configures measurements before the UE sends a SecurityModeComplete in response to a SecurityModeCommand, which could be done to speed up the measurement reports. However, for privacy reasons, measurements reports shall only be transmitted after security has been activation successfully.

UE may receive a measurement configuration before security is activated.

As in the case of measurement reports, the UE in CONNECTED state may only receive a handover command (i.e. an RRCConnectionReconfiguration with mobilityControlInfo) after security has been activated.

UE may only receive a handover command after security is activated.

Hence, as the CHO configuration comprises a measurement configuration component and a handover command component, a discussion is whether the CHO configurations shall only be processed if the configuration is secure (integrity protected and/or encrypted) or whether security requirement could be relaxed. As the CHO configuration contains RRCConnectionReconfiguration message(s) the conclusion is that it needs to be security protected.

Conditional handover can only be configured when security is activated.

# Summary

RAN2 is kindly asked to discuss the following proposals:

- Conditional handover can only be configured when security is activated.

# References

- RP-181475, Work Item on Even further Mobility Enhancements in E-UTRAN, China Telecom, RAN#81, September 2018