3GPP TSG-RAN WG2 #107	 R2-1909359

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.2.2.1

Source:	Ericsson

Title:	Cell (re)selection and cell barring related to forbidden PLMNs

Document for:	Discussion, Decision

# 1	Introduction

For NR licensed, it is implicitly assumed that the UE selects its registered/equivalent PLMN, and that all cells on a frequency belong to the same PLMN. However, different operators may use the same frequencies for deploying their cells in unlicensed spectrum. Therefore, as captured in the NR-U study item report [1], modifications are needed to allow the UE to camp on a non-best cell on a carrier:

In unlicensed bands, multiple PLMNs can use the same carriers without any coordination. Therefore, the best cell found by a UE on a frequency may not belong the registered PLMN. In this case, the UE should be enabled to camp on a non-best cell on a carrier if the best cell does not belong to the registered PLMN (or E-PLMN), where the non-best cell would still be the best cell of the registered PLMN.

The issues on measurement framework and camping on non-best cell have also been captured as part of the NR-U WI objectives [2].

Relevant definition from TS 38.304, Section 4.5:

suitable cell:

A cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list and trackingAreaCode is provided for that PLMN;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2.

According to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" (TS 22.261 [12]), which belongs to a PLMN that fulfils the first bullet above.

In this contribution, we discuss necessary changes for cell (re)selection criteria in unlicensed operation. In the following, we refer to selected/registered/equivalent PLMNs as being “allowed PLMNs”, and if the cell does not belong to the registered or equivalent PLMN, the PLMN is considered a “forbidden PLMN”.

allowed PLMN: selected/registered/equivalent PLMN as described as part of suitable cell definition

forbidden PLMN: a PLMN that is not part of the registered or equivalent PLMNs

At RAN2#106, the following agreements were made:

Agreement #106-1: If highest ranked or best cell is not suitable in an unlicensed frequency due to the fact that PLMN IDs is not the RPLMN (or EPLMN), only the highest ranked or best cell is considered not candidate for cell reselection for 300s or longer. Other cells in the frequency of the highest ranked or best cell should still be considered for cell reselection. 
FFS #106-1: whether we have another limit in addition to Suitability criterion.

Agreement #106-2: To apply the cell barring and IntraFreqReselection in the MIB, the UE also has to acquire the SIB1 to check the PLMN IDs.

FFS #106-2: if the UE should only act on the cell barring and intraFreqReselection in the MIB only if the registered PLMN or selected PLMN matches one of the PLMN IDs in SIB1. [Otherwise, the UE should follow agreement #106-1]

# 2	Discussion

## Cell (re)selection and cell barring

On each frequency, the NR Rel-15 UE only needs to search for the strongest cell. However, the strongest or highest ranked cell may belong to a “forbidden” PLMN. In such a case, the whole frequency would be barred for 300 s, as described in Sections 5.1.1 and 5.4.4 of [3] and in [4]. For unlicensed operation, the UE shall not consider other cells on the same frequency as being barred, so that it can find stronger cells than on other frequencies. This is covered by the above agreement:

Agreement #106-2: To apply the cell barring and IntraFreqReselection in the MIB, the UE also has to acquire the SIB1 to check the PLMN IDs.

In our understanding, the above agreement implies that the UE does not apply the cell barring and intraFrequencyReselection bit before it has successfully read SIB1 and checked that one of the PLMN IDs match with the selected/registered/equivalent PLMN.

To enable the UE to select a non-best cell belonging to its registered/equivalent PLMN, the UE shall not consider a frequency as being barred if the strongest cell on that frequency belongs to a PLMN that does not match with the UE’s registered/equivalent PLMN. Barring for up to 300s only applies to that cell.

As described in Section 5.3.1 of [3] and in [5], cellBarring being set to barred and intraFreqReselection being set to notAllowed as indicated in the MIB bars all cells on that frequency. However, the cell/frequency barring should be handled separately for each PLMN.

cellBarring set to barred and intraFreqReselection set to notAllowed as indicated in the MIB only applies to the cell(s) on a frequency of the given PLMN(s), not to cells on that frequency belonging to other PLMNs.

In other words, only if the registered/selected/equivalent PLMN matches one of the PLMN IDs provided in SIB1, the UE acts according to the cellBarring and intraFreqReselection bits provided in the MIB.

## Potential hidden node issues

In unlicensed spectrum, many cells may be operated in an uncoordinated manner, potentially resulting in hidden node issues, where gNBs deployed by different operators may not hear each other during the listen-before-talk procedure. However, the interference from the other operator’s cells would typically impact the RSRQ level for the cell ranking.

Hidden node interference is visible in the UE’s RSRQ measurements which can be configured for cell ranking evaluations.

Existing measurement procedures can be reused for cell ranking evaluations.

# 3	Conclusion

In the previous sections we made the following observations and clarifications:

Observation 1	Hidden node interference is visible in the UE’s RSRQ measurements which can be configured for cell ranking evaluations.

- Clarification 1	To enable the UE to select a non-best cell belonging to its registered/equivalent PLMN, the UE shall not consider a frequency as being barred if the strongest cell on that frequency belongs to a PLMN that does not match with the UE’s registered/equivalent PLMN. Barring for up to 300s only applies to that cell.

Clarification 2	cellBarring set to barred and intraFreqReselection set to notAllowed as indicated in the MIB only applies to the cell(s) on a frequency of the given PLMN(s), not to cells on that frequency belonging to other PLMNs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Existing measurement procedures can be reused for cell ranking evaluations.

A corresponding CR to reflect the RAN2#106 agreements is provided in [6].

# 4	References

3GPP TR 38.889 V16.0.0, “Study on NR-based Access to Unlicensed Spectrum”, 2018-12

RP-182878, “NR-based Access to Unlicensed Spectrum”, 3GPP, RAN#82

3GPP TS 38.304 V15.2.0, “User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 15)”, 2018-12

R2-1900713, “Enabling the UE to camp on non-best cell”, Intel, RAN2#105

R2-1900142, “Cell Barring in NR-U“, Samsung, RAN2#105

R2-1909360, “Draft CR to 38.304 to handle forbidden PLMNs during cell (re)selection”, RAN2#107

# 5	Annex

Relevant text from 3GPP TS 38.304 [3]:

[5.1.1 Support for PLMN selection]

The UE shall scan all RF channels in the NR bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to.

[5.2.3 Cell Selection process]

2. On each frequency, the UE need only search for the strongest cell.

[5.2.4.4 Cells with cell reservations, access restrictions or unsuitable for normal camping]

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds.

[5.3.1 Cell status and cell reservations]

- If the field intraFreqReselection in MIB message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

- The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 second