3GPP TSG-RAN WG2 Meeting #107	R2-1909424

Prague, CZ, August 26 – 30, 2019

Agenda item:	11.8.2.4

Source: 	Qualcomm Incorporated

Title: 	Assistance Data for DL-only UE-based mode

Document for:		Discussion and Decision

# 1. 	Introduction

At RAN2#106, it was agreed that UE-based DL-only positioning is supported at least for the case of unicast assistance data [1]. The DL NR positioning techniques listed in the WID comprise DL-TDOA, DL-AoD, and E-CID [2]. The details of these positioning techniques haven’t been defined yet (e.g., RAN1 details, Stage 2 description, procedures, signalling).  However, it was also agreed at RAN2#106 [1] that the LPP procedures will be reused for NR, including the Assistance Data Transfer procedures. In this contribution we propose the assistance data details for UE-based DL-only positioning.

# 2. 	Discussion

For UE-based mode, the target device performs both positioning measurements and computation of a location estimate. Assistance data to support the positioning measurements are also needed for UE-based mode (in the same way as for UE-assisted mode) and for NR are expected to contain similar information as defined for LTE (which in case of OTDOA contains reference and neighbour cell information, including the PRS configuration details). The additional assistance data needed to support UE-based mode can be defined separately (similar as in e.g. UMTS [3]). This allows providing positioning measurement and position calculation assistance data individually (and allows a target device to request any missing assistance data for the specific mode being used (UE-assisted or UE-based) without receiving unneeded assistance data for a mode that is not being used).

Proposal 1:	The positioning measurement assistance data and position calculation assistance data should be defined in separate IEs.

The development of the details of the position calculation assistance data can make use of previous work in RAN2 (e.g., for UMTS [3], or during the LTE Rel-15 "UE Positioning Accuracy Enhancements for LTE" work item [4]). For example, the following assistance data for UE-based OTDOA are defined for UMTS ([3] section 10.3.7.103a "UE positioning OTDOA assistance data for UE-based") which are generally also applicable to NR native positioning methods:

- 	Cell (antenna) locations for the reference cell, and each neighbour cell;

-	Real (or Relative) Time Differences (RTDs) (base station synchronization difference);

-	Quality of RTDs;

-	RTD drift rate;

-	Round Trip Time (RTT) for all serving cells (active set).

OMA LPPe [5] also supports assistance data for UE-based OTDOA in LTE, which are based on the UMTS assistance data summarized above.

Proposal 2:	Use the UMTS and LPPe definition for UE-based OTDOA [3,5] as a starting point for the specification of the position calculation assistance data for NR.

Compared to e.g. LTE, the DL-PRS can be transmitted in narrow beams (for both, FR1 and FR2), as illustrated in Figure 1 below. The relevant status in RAN1 can be summarized as follows [1]: A DL-PRS Resource Set is defined as a set of DL-PRS Resources where each DL-PRS Resource has a DL-PRS Resource ID. The DL PRS Resources in a DL PRS Resource Set are associated with the same Transmission-Reception Point (TRP) (e.g. same gNB or same gNB-DU). Each DL-PRS Resource ID in the DL PRS Resource Set is associated with a specific beam. A TRP may be configured with multiple PRS Resource Sets.

Figure 1: Illustration of DL-PRS Resource Set and Spatial Direction of DL-PRS Resources.

Compared to LTE, the RSTD measurements are not made "between cells" but between "beams", where the beams (Resource IDs) may even belong to the same TRP.  In particular, a DL RSTD measurement may be identified using a DL PRS Resource ID, a subset of DL PRS Resource IDs from a single DL PRS Resource set, and/or a DL PRS Resource set [1]. Therefore, the "antenna reference point (ARP)" location (using the RTK terminology of ARP) must be per Resource ID. RSTD measurements per beam (Resource ID) of a TRP may for example allow identifying the beam which results in the earliest time of arrival, and therefore, may allow excluding DL-PRS measurements in the position calculation whose beams point to a e.g. reflecting object resulting in additional excess delay at the target device. In addition, since each beam has a specific heading (azimuth and elevation angle), it permits a direction to be determined from a TRP which can be combined with e.g. RSTD measurements to determine the target device location.  The directional information of DL-PRS Resources is also exploited in the DL-AoD method, where the target device determines the beam(s) pointing towards the target device, e.g., by performing RSRP measurements on DL-PRS Resources. Therefore, the directional information of DL-PRS should be included in the position calculation assistance data.

Proposal 3:	Include spatial direction information of the DL-PRS Resources in the position calculation assistance data (e.g., azimuth, elevation, beamwidth).

In order to reduce the number of bits required for the TRP location information, the neighbour TRP locations can be defined as delta-locations relative to the reference TRP location (similar to e.g. [3, 5]). Each TRP comprises one or more DL-PRS Resource Sets, whose ARP locations may or may not coincide with the TRP location. For example, different antenna panels may be used for different DL-PRS Resource Sets. The ARP for each Resource Set of a TRP can be defined as delta-locations relative to the TRP location. Each DL-PRS Resource in the DL-PRS Resource Set can be defined as delta relative to the corresponding DL-PRS Resource Set ARP. The ARP of each DL-PRS Resource may or may not coincide with the associated DL-PRS Resource Set ARP. For example, beams are typically created using a subset of the antenna elements, or a panel is split into subpanels as illustrated in Figure 2 below. In this way, the ARP of each DL-PRS Resource can be provided with desired resolution/granularity with minimal overhead.

Proposal 4:	Define the Antenna Reference Point (ARP) locations for each DL-PRS Resource ID as delta locations relative to a reference point. In particular:

-	define an absolute reference point location for the reference TRP, followed by a list of relative locations for each neighbour TRP;

- 	each PRS Resource Set ARP location of a TRP is defined relative to the TRP location;

- 	each PRS Resource ARP location in the PRS Resource Set is defined relative to the PRS Resource Set ARP.

NOTE: 	Proposal 3 and 4 also applies to NRPPa for UE-assisted mode.

Note that the hierarchical levels of location (TRP, resource set, resource ID) can be reduced using a convention that any delta location which is omitted is zero.

Figure 2: Illustration of a TRP antenna with multiple panels, where each panel has multiple antenna elements. Inside each panel, for each polarization, all the elements of each column may be used to form one beam, and therefore, beams of the same panel are transmitted from different physical locations on the x-axis (in this example).

At RAN2#106, it was confirmed that broadcast assistance data are supported for DL-only positioning. However, it is FFS whether this applies to UE-based mode [1]. The benefits of broadcasting location assistance data include reduced network signaling and reduced load on network elements (e.g. LMF, AMF, gNB) for a high density of recipient UEs in each cell, reduced latency in obtaining assistance data at a UE, and reduced UE signaling and power usage. As an example, IoT UEs would be able to determine and track their locations while in idle state which could reduce both UE power and load on the network. In addition, UEs supporting V2X would be able to more easily and reliably maintain high accuracy location without delay when moving between different cells and TAs.

Proposal 5:	Support broadcast of assistance data also for UE-based DL-only positioning.

For broadcast of assistance data, positioning measurement assistance data (e.g. for UE-assisted) and position calculation assistance data (for UE-based) can be provided in separate posSIBs [6]. This allows different ciphering for UE-assisted and UE-based mode, and therefore, controlled access to UE-assisted and UE-based services individually for a network operator.

Proposal 6:	For broadcast of assistance data, define separate posSIBs for UE-assisted and UE-based mode assistance data.

The UE-based specific assistance data listed above (before Proposal 2) contain rather static information (such as antenna locations) and more dynamic data (such as the RTDs). Defining separate posSIB Types for "static" and "dynamic" data would be advantageous since it would allow different broadcast periodicities for the static and more dynamic data. In addition, the "static data" may be useful for various DL-only methods which would not require the e.g. RTDs, and thus, reduces the overhead for such methods. It  also enables more control for an network operator since it allows ciphering the different data (posSIBs) separately.

Proposal 7:	Split the position calculation assistance data into two separate posSIBs, one containing the rather static TRP coordinates and one containg the more dynamic synchronization differences (RTDs).

# 3. 	Position calculation assistance data

A text proposal for the position calculation assistance data is provided in section 4 below according to the Proposals above. The following principles are used:

(1)	A single high level IE PositionCalculationAssistance is proposed which may be included in various Provide Assistance Data IEs and/or in the CommonIEsProvideAssistanceData.
The IE PositionCalculationAssistance comprises all the information required for UE-based mode individually (as optional fields) such that a subset of the information can be provided when needed. The identifiers (cell IDs/TRP IDs, Resource Set IDs, Resource IDs) are summarized in a single field (assistanceDataTRP-List), which has the advantage that the identifiers do not need to be repeated in the various fields included in the IE PositionCalculationAssistance. However, it requires that all the fields have the same number of elements in each SEQUENCE; i.e., in case of non-available or non-applicable information for a particular ID (Cell/TRP, Resource Set, Resource) an empty SEQUENCE would be included.

(2)	The IE TRP-LocationInfo provides the TRP locations for a reference TRP and a list of neighbour TRPs, and optionally the ARPs per DL-PRS Resource ID. In particular:

(a)	A ReferencePoint for the reference TRP is defined which may be a EllipsoidPointWithAltitudeAndUncertaintyEllipsoid or a HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid. This is the only GAD shape IE provided in the IE PositionCalculationAssistance.

(b)	A generic RelativeLocation IE is defined which allows geodetic locations relative to a reference point with various granularities together with uncertainty. 
Granularities of 0.00003, 0.0003, 0.003, and 0.03 (which is used in UMTS [3]) seconds of angle are proposed. These granularities correspond to approximately (at the equator):

0.00003''	~ 8.3×10-9 deg; approx. ~ 0.00092 m

0.0003''	~ 8.3×10-8 deg; approx. ~ 0.0092 m

0.003''		~ 8.3×10-7 deg; approx. ~ 0.092 m

0.03''		~ 8.3×10-6 deg; approx. ~ 0.92 m.

The delta-latitude and delta-longitude for the relative location can be provided as a "fine delta" with the granularity above together with a "coarse delta". With the proposed (rather small) value ranges, this still allows encoding a delta latitude/longitude of up to about 38 km with about centi-metre resolution.  
Similar, for the height resolution units of metre, centi-metre, and milli-metre are proposed.

(3)	The IE DL-PRS-Beam-Info includes the azimuth and elevation angles of each DL-PRS Resource (beam) together with the 3-dB beamwidth.

(4)	The IE RTD-Info includes time synchronization information for the neighbour TRPs relative to the reference TRP.  The time synchronization information comprises SFN Offset, subframe offset, and fine subframe offset as illustrated in Figure 4 below. This allows obtaining the full timing of neighbour cells, given the reference cell timing (e.g. serving cell timing). The resolution for the fine timing is proposed in time units  where Δfmax=480∙103 Hz and ; i.e. Tc = 0.509 ns [7]. A delta value for each PRS Resource Set ARP and Resource ARP for each RTD can be provided in addition.

NOTE: 	The coarse timing (SFN Offset and subframe offset) may also be included in the measurement assistance data, since probably needed to determine the PRS occasions.

(5)	A IE AuxiliaryInfo is proposed, which may contain the Round-Trip-Times (RTTs) between current target device location and TRPs (if available), similar to UMTS [3]. This could support position calculation ("hybrid") or could support OTDOA in asynchronous networks by determining the RTDs using the provided RTTs and measured RSTD: RTD = OTDOA – (RTT2 – RTT1)/2.

Figure 4: Illustration Real Time Differences (RTDs).

# 4. 	Text Proposal

## 4.1		LPP Assistance Data Transfer

#### –	PositionCalculationAssistance

The IE PositionCalculationAssistance is used by the location server to provide assistance data to enable UE-based downlink positioning.

-- ASN1START

# PositionCalculationAssistance-r16 ::= SEQUENCE {

assistanceDataTRP-List-r16			AssistanceDataTRP-List-r16		OPTIONAL,	-- Need OP

trp-LocationInfo-r16 				TRP-LocationInfo-r16			OPTIONAL,	-- Need ON

dl-prs-BeamInfo-r16					DL-PRS-Beam-Info-r16			OPTIONAL,	-- Need ON

rtd-Info-r16						RTD-Info-r16					OPTIONAL,	-- Need ON

auxiliaryInfo-r16					AuxiliaryInfo-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

PositionCalculationAssistance field descriptions

assistanceDataTRP-List This field specifies the cell IDs/TRP IDs, DL-PRS Resource Set IDs and DL-PRS Resource IDs for the assistance data reference and neighbour TRPs. If this field is absent FFS. Editor’s Note: The IDs may not always explicitely be needed in this IE; e.g. if this IE is included in the IE OTDOA-ProvideAssistanceData the IDs of the OTDOA Reference and Neighbour Cell Info would apply. FFS – Depends on how NR assistance data will be added.

trp-LocationInfo This field provides the location coordinates of the antenna reference points for the reference and neighbour TRPs identified by assistanceDataTRP-List. This field includes the same number of entries, and listed in the same order, as in the assistanceDataTRP-List. NOTE.

dl-prs-BeamInfo This field provides the spatial directions of DL-PRS Resources for the reference and neighbour TRPs identified by assistanceDataTRP-List. This field includes the same number of entries, and listed in the same order, as in the assistanceDataTRP-List. NOTE.

rtd-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs identified by assistanceDataTRP-List. This field includes the same number of entries, and listed in the same order, as in the assistanceDataTRP-List. NOTE.

auxiliaryInfo This field provides auxiliary information for UE-based positioning. This field includes the same number of entries, and listed in the same order, as in the assistanceDataTRP-List. NOTE.

NOTE: 	If the information is not available/provided for a particular entry in the assistanceDataTRP-List (TRP and/or resource set and/or resource, as applicable), the corresponding entry in this field would contain an empty sequence.

#### –	AssistanceDataTRP-List

The IE AssistanceDataTRP-List is used by the location server to provide the identities of the assistance data reference TRP and neighbour TRPs.

-- ASN1START

# AssistanceDataTRP-List-r16 ::= SEQUENCE {

referenceTRP-Id-r16			ReferenceTRP-Id-r16,

neighbourTRP-Id-List-16		NeighbourTRP-Id-List-r16,

...

}

ReferenceTRP-Id-r16 ::= SEQUENCE {

nrPhysCellId-Ref-r16			INTEGER (0..1007)					OPTIONAL,	-- Need ON

nrARFCN-Ref-r16					ARFCN-ValueNR-r15					OPTIONAL,	-- Need ON

nrCellGlobalID-Ref-r16			NCGI-r15							OPTIONAL,	-- Need ON

nrTRP-ID-Ref-r16				INTEGER (FFS)						OPTIONAL,	-- Need ON

dl-PRS-ResourceSetsRef-r16		SEQUENCE (SIZE(0..maxResourceSets-1-r16)) OF

DL-PRS-ResourceSetsElement-r16,

...

}

NeighbourTRP-Id-List-r16 ::= SEQUENCE (SIZE(0..maxTRP-1-r16)) OF NeighbourTRP-Id-Element-r16

NeighbourTRP-Id-Element-r16 ::= SEQUENCE {

nrPhysCellId-Neighbour-r16			INTEGER (0..1007)		OPTIONAL,	-- Need ON

nrARFCN-Neighbour-r16				ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

nrCellGlobalID-Neighbour-r16		CHOICE {

samePLMNasRef-r16		BIT STRING (SIZE (36)),

differentPLMN-r16		NCGI-r15

}															OPTIONAL,	-- Need ON

nrTRP-ID-Neighbour-r16				INTEGER (FFS)			OPTIONAL,	-- Need ON

dl-PRS-ResourceSetsNeighbour-r16	SEQUENCE (SIZE(0..maxResourceSets-1-r16)) OF

DL-PRS-ResourceSetsElement-r16,

...

}

DL-PRS-ResourceSetsElement-r16 ::= SEQUENCE {

dl-prs-resourceSetId-r16			INTEGER (FFS),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE(0..maxResources-1-r16)) OF

DL-PRS-ResourceID-r16,

...

}

DL-PRS-ResourceID-r16 ::= INTEGER(FFS)

maxTRP-1-r16 			INTEGER ::= 127

maxResourceSets-1-r16	INTEGER ::= FFS

maxResources-1-r16		INTEGER ::= FFS

-- ASN1STOP

AssistanceDataTRP-List field descriptions

nrPhysCellId-Ref This field specifies the physical cell identity of the reference TRP when the reference TRP corresponds to a physical cell.

nrARFCN-Ref This field specifies NR-ARFCN of the reference TRP.

nrCellGlobalID-Ref This field specifies the NR Cell Global Identifier (NCGI) of the reference TRP when the reference TRP corresponds to a physical cell.

nrTRP-ID-Ref This field specifies an identity of the reference TRP.

dl-PRS-ResourceSetsRef This field specifies the DL-PRS Resource Set IDs associated with the reference TRP.

nrPhysCellId-Neighbour This field specifies the physical cell identity of the neighbour TRP when the neighbour TRP corresponds to a physical cell.

nrARFCN-Neighbour This field specifies NR-ARFCN of the neighbour TRP.

nrCellGlobalID-Neighbour This field specifies the NR Cell Global Identifier (NCGI) of the neighbour TRP when the neighbour TRP corresponds to a physical cell.

nrTRP-ID-Neighbour This field specifies an identity of the neighbour TRP.

dl-PRS-ResourceSetsNeighbour This field specifies the DL-PRS Resource Set IDs associated with the neighbour TRP.

#### –	TRP-LocationInfo

The IE TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

# TRP-LocationInfo-r16 ::= SEQUENCE {

referenceTRP-LocationInfo-r16		ReferenceTRP-LocationInfo-r16,

neighbourTRP-LocationInfoList-16	NeighbourTRP-LocationInfoList-r16,

...

}

ReferenceTRP-LocationInfo-r16 ::= SEQUENCE {

referencePoint-r16					ReferencePoint-r16,

referenceTRP-Info-r16				TRP-Location-Element-r16,

...

}

NeighbourTRP-LocationInfoList-r16 ::= SEQUENCE (SIZE(0..maxTRP-1-r16)) OF

NeighbourTRP-LocationInfoElement-r16

Neighbour-LocationInfoElement-r16 ::= SEQUENCE {

neighbourTRP-Info-r16				TRP-Location-Element-r16,

...

}

TRP-Location-Element-r16 ::= SEQUENCE {

-- Applicable per Cell-ID/TRP-ID

trp-Location-r16					RelativeLocation-r16				OPTIONAL, -- Need OP

trp-DL-PRS-ResourceSets-r16			SEQUENCE (SIZE(0..maxResourceSets-1-r16)) OF

DL-PRS-ResourceSets-TRP-Element-r16

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

-- Applicable per Resource Set ID of the Cell-ID/TRP-ID

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL, -- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(0..maxResources-1-r16)) OF

DL-PRS-Resource-ARP-Element-r16

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

-- Applicable per Resource ID of the Resource Set ID

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL, -- Need OP

...

}

-- ASN1STOP

TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the reference TRP location.

referenceTRP-Info This field provides the antenna reference point locations of the DL-PRS Resources associated with the reference TRP and comprises the following sub-fields: - trp-Location: This field provides the location of the reference TRP relative to the referencePoint location. If this field is absent the reference TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with the reference TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the reference TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the reference TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

neighbourTRP-Info This field provides the antenna reference point locations of the DL-PRS Resources of this neighbour TRP and comprises the following sub-fields: - trp-Location: This field provides the location of this neighbour TRP relative to the reference point used to define the location of the reference TRP. If this field is absent this neighbour TRP location coincides with the reference point used to define the location of the reference TRP. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this neighbour TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of this neighbour TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of this neighbour TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	ReferencePoint

The IE ReferencePoint provides a well defined location relative to which other locations may be defined.

-- ASN1START

# ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

# RelativeLocation-r16 ::= SEQUENCE {

arc-second-units-r16 	ENUMERATED { as0-00003, as0-0003, as0-003, as0-03, ...},

height-units-r16		ENUMERATED {mm, cm, m, ...},

delta-latitude-r16		Delta-Latitude-r16,

delta-longitude-r16		Delta-Longitude-r16,

delta-height-r16		Delta-Height-r16,

locationUNC-r16			LocationUncertainty-r16					OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255),

verticalConfidence-r15					INTEGER (0..100)

}

-- ASN1STOP

RelativeLocation field descriptions

arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values as0-00003, as0-0003, as0-003, and as0-03, correspond to 0.00003, 0.0003, 0.003, and 0.03 seconds of angle, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 metre, and 1 metre, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × arc-second-units) ± (coarse-delta-Latitude × 1024 × arc-second-units) [arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × arc-second-units) ± (coarse-delta-Latitude × 1024 × arc-second-units) [arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [metres]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

#### –	DL-PRS-Beam-Info

The IE DL-PRS-Beam-Info is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

# DL-PRS-Beam-Info-r16 ::= SEQUENCE {

referenceTRP-DL-PRS-BeamInfo-r16		DL-PRS-Beam-Info-Set-r16,

neighbourTRP-DL-PRS-BeamList-16			NeighbourTRP-DL-PRS-BeamList-16,

...

}

NeighbourTRP-DL-PRS-BeamList-16 ::= SEQUENCE (SIZE(0..maxTRP-1-r16)) OF

NeighbourTRP-DL-PRS-BeamElement-16

NeighbourTRP-DL-PRS-BeamElement-16 ::= SEQUENCE {

-- Applicable per Cell-ID/TRP-ID

neighbourTRP-DL-PRS-BeamInfo-r16		DL-PRS-Beam-Info-Set-r16,

...

}

DL-PRS-Beam-Info-Set-r16 ::= SEQUENCE (SIZE(1..maxResourceSets-r16)) OF

DL-PRS-Beam-Info-ResourceSet-r16

DL-PRS-Beam-Info-ResourceSet-r16 ::= SEQUENCE {

-- Applicable per Resource Set ID of the Cell-ID/TRP-ID

dl-PRS-BeamInfoList-r16			SEQUENCE (SIZE(1..maxResources-r16)) OF

DL-PRS-BeamInfoElement-r16

...

}

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

-- Applicable per Resource ID of the Resource Set ID

dl-PRS-Azimuth-r16				INTEGER (0..719),

dl-PRS-Elevation-r16			INTEGER (-180..180)				OPTIONAL,	-- Need OP

dl-PRS-HPBW-Az-r16 				INTEGER (0..240)				OPTIONAL,	-- Need ON

dl-PRS-HPBW-El-r16 				INTEGER (0..240)				OPTIONAL,	-- Need ON

...

}

maxResourceSets-r16 INTEGER ::= FFS

maxResources-r16	INTEGER ::= FFS

-- ASN1STOP

DL-PRS-Beam-Info field descriptions

referenceTRP-DL-PRS-BeamInfo This field provides the beam information for the reference TRP.

neighbourTRP-DL-PRS-BeamList This field provides the beam information for the neighbour TRPs.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. The azimuth angle is measured clockwise from geographical North. Scale factor 0.5 degrees; range 0 to 359.5 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. The elevation angle is the angle between to the horizontal plane at the antenna reference point location and the boresight direction, measured in the vertical plane. Positive angles point in directions above the horizontal plane (upwards), negative angles point in directions below the horizontal plane (downwards). If this field is absent, the boresight direction is the same along the vertical plane. Scale factor 0.5 degrees; range -90 to +90 degrees.

dl-PRS-HPBW-Az This field specifies the half-power beamwidth (HPBW) in the horizontal plane of the beam in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. HPBW is the angle subtended by the half-power points of the mainlobe in the horizontal plane. Scale factor 0.5 degrees; range 0 to 120 degrees.

dl-PRS-HPBW-El This field specifies the half-power beamwidth (HPBW) in the vertical plane of the beam in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. HPBW is the angle subtended by the half-power points of the mainlobe in the vertical plane. Scale factor 0.5 degrees; range 0 to 120 degrees.

#### –	RTD-Info

The IE RTD-Info is used by the location server to provide time synchronization information between the reference TRP and neighbour TRPs.

-- ASN1START

# RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16			ReferenceTRP-RTD-Info-r16,

rtd-InfoList-16						RTD-InfoList-16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16			NR-MeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-16 ::= SEQUENCE (SIZE(1..maxTRP)) OF RTD-InfoElement-16

RTD-InfoElement-r16 ::= SEQUENCE {

sfn-Offset-r16				INTEGER (0..1023),

integerSubframeOffset-r16	INTEGER (0..9)					OPTIONAL,	-- Need OP

subframeOffset				INTEGER (0..1966079),

rtd-Quality					NR-MeasQuality-r16,

rtd-DriftRate				INTEGER (-256..255)				OPTIONAL,	-- Need ON

delta-rtd-set-r16			SEQUENCE (SIZE (0..maxResourceSets-1-r16)) OF

Delta-RTD-ResourceSet-r16,

...

}

Delta-RTD-ResourceSet-r16 ::= SEQUENCE {

-- Applicable per Resource Set ID of the Cell-ID/TRP-ID

delta-rtd-r16				INTEGER (-64..63)				OPTIONAL,	-- Need ON

delta-rtd-ResourceList-r16	SEQUENCE (SIZE (0..maxResources-1-r16)) OF

Delta-RTD-ResourceElement-r16,

...

}

Delta-RTD-ResourceElement-r16 ::= SEQUENCE {

-- Applicable per Resource ID of the Resource Set ID

delta-rtd-r16				INTEGER (-64..63)				OPTIONAL,	-- Need ON

...

}

maxTRP 	INTEGER ::= 128

-- ASN1STOP

RTD-Info field descriptions

refTime This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber CHOICE refers to the SFN of the assistance data reference TRP.

rtd-RefQuality This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList. Editor’s Note: The NR RSTD, etc. Quality field may be reused.

sfn-Offset This field specifies the SFN offset at the TRP antenna location between the reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP.

integerSubframeOffset This field specifies the frame boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes. If this field is absent, the integerSubframeOffset is zero.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [x]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD. Editor’s Note: The NR RSTD, etc. Quality field may be reused.

rtd-DriftRate This field specifies the drift rate of the RTD between the reference TRP and this neighbour TRP in units of 0.5 Tc per second. A positive value indicates that the reference cell clock is running at a greater frequency than the neighbour cell clock. Scale factor 0.5 Tc/sec.

delta-rtd This field provides a delta RTD value to be added to the RTD of the TRP for the corresponding DL-PRS Resource in the Resource Set. Scale factor 0.5 Tc

#### –	AuxiliaryInfo

The IE AuxiliaryInfo is used by the location server to provide additional information useful for UE-based mode.

-- ASN1START

# AuxiliaryInfo-r16 ::= SEQUENCE {

referenceTRP-Aux-Info-r16			ReferenceTRP-Aux-Info-r16,

neighbourTRP-Aux-InfoList-r16		NeighbourTRP-Aux-InfoList-r16,

...

}

ReferenceTRP-Aux-Info-r16 ::= SEQUENCE {

half-round-trip-time-r16					INTEGER (0..131071)		OPTIONAL,	-- Need ON

...

}

NeighbourTRP-Aux-InfoList-r16 ::= SEQUENCE (SIZE(0..maxTRP-1)) OF NeighbourTRP-Aux-InfoElement-r16

NeighbourTRP-Aux-InfoElement-r16 ::= SEQUENCE {

half-round-trip-time-r16					INTEGER (0..131071)		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

AuxiliaryInfo field descriptions

half-round-trip-time This field provides half of the round-trip-time between the target device and the indicated TRP. Scale factor 1 Tc. Editor’s Note: Depends on how RTT will be defined (FFS).

## 4.2	posSIB Types Element Definitions

For the posSIB elements, "assistance data reference TRP" refers to the TRP which broadcasts the corresponding posSIB type(s).

#### –	UEB-TRP-LocationData

The IE UEB-TRP-LocationData is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [x] indicates 'posSibTypeX-y'.

-- ASN1START

UEB-TRP-LocationData-r16 ::= SEQUENCE {

assistanceDataTRP-List-r16			AssistanceDataTRP-List-r16

trp-LocationInfo-r16 				TRP-LocationInfo-r16,

dl-prs-BeamInfo-r16					DL-PRS-Beam-Info-r16			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

UEB-TRP-LocationData field descriptions

assistanceDataTRP-List LPP IE AssistanceDataTRP-List as defined in sub-clause x.y.z.a.

trp-LocationInfo LPP IE TRP-LocationInfo as defined in sub-clause x.y.z.b.

dl-prs-BeamInfo LPP IE DL-PRS-Beam-Info as defined in sub-clause x.y.z.c.

#### –	UEB-TRP-RTD-Info

The IE UEB-TRP-RTD-Info is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [x] indicates 'posSibTypeX-z'.

-- ASN1START

UEB-TRP-RTD-Info-r16 ::= SEQUENCE {

rtd-Info-r16						RTD-Info-r16,

...

}

-- ASN1STOP

UEB-TRP-LocationData field descriptions

rtd-Info LPP IE RTD-Info as defined in sub-clause x.y.z.c.

# 5. 	Summary

In this contribution we proposed the assistance data details for UE-based DL-only positioning. The following proposals are made.

Proposal 1:	The positioning measurement assistance data and position calculation assistance data should be defined in separate IEs.

Proposal 2:	Use the UMTS and LPPe definition for UE-based OTDOA [3,5] as a starting point for the specification of the position calculation assistance data for NR.

Proposal 3:	Include spatial direction information of the DL-PRS Resources in the position calculation assistance data (e.g., azimuth, elevation, beamwidth).

Proposal 4:	Define the Antenna Reference Point (ARP) locations for each DL-PRS Resource ID as delta locations relative to a reference point. In particular:

-	define an absolute reference point location for the reference TRP, followed by a list of relative locations for each neighbour TRP;

- 	each PRS Resource Set ARP location of a TRP is defined relative to the TRP location;

- 	each PRS Resource ARP location in the PRS Resource Set is defined relative to the PRS Resource Set ARP.

Proposal 5:	Support broadcast of assistance data also for UE-based DL-only positioning.

Proposal 6:	For broadcast of assistance data, define separate posSIBs for UE-assisted and UE-based mode assistance data.

Proposal 7:	Split the position calculation assistance data into two separate posSIBs, one containing the rather static TRP coordinates and one containg the more dynamic synchronization differences (RTDs).

Proposal 8:	Endorse the text proposal in section 4 as baseline for further work.

# References

[1]	RP-191155, "Status report of WI: NR positioning support".

[2]	RP-190752, "New WID: NR Positioning Support".

[3]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[4]	RP-170722, "WID: UE Positioning Accuracy Enhancements for LTE".

[5]	OMA LPPe: "LPP Extensions Specification", Open Mobile Alliance.

[6]	R2-1909421, "Broadcast of Location Assistance Data by NG-RAN" , Qualcomm Incorporated.

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

TRP details (and the term TRP) remain to be agreed by RAN2 and RAN3. But it is expected that a gNB without gNB-DUs and a gNB-DU could each contain one or possibly more than one TRP. A TRP would be associated with a physical antenna (array) and be capable of transmitting DL-PRS and/or receiving and measuring UL-PRS.