3GPP TSG-RAN WG2 #107	R2-1909504

Prague, Czech Republic, 26th– 30th August 2019

Agenda Item:	11.12.2

Source:	Ericsson

Title:	UE Location Report Capability Indication for MDT

Document for:	Discussion, Decision

# 1	Introduction

In RAN2 #106, the following agreements were made about location information report for MDT:

In Rel-16, UE shall include the GNSS location information for Logged MDT, DL signal quantities measurement of Immediate MDT, RLF report, accessibility measurement and out-of-coverage logging, if the GNSS location information is available when the measurement was taken for the UEs with the GNSS receiver.

This paper discusses the location report capability indication for MDT in Rel-16.

# 2	Background

In LTE, the use cases identified for MDT [1] mainly include:

- Coverage use cases: the MDT data reported from UEs and the RAN may be used to monitor and detect coverage problems in the network. Some examples are described in TS 37.320 shown below.

  - Coverage hole

  - Weak coverage

  - Pilot Pollution

  - Overshoot coverage

  - Coverage mapping

  - UL coverage

  - Cell boundary mapping

  - Coverage mapping for pico cell in CA scenario

- QoS verification use cases:

  - Traffic Location: MDT functionality to obtain information of where data traffic is transferred within a cell.

  - User QoS Experience: MDT functionality to assess the QoS experience for a specific UE together with location information.

It is obvious that the geographical location information is essential for the use cases. The detailed usages of GNSS location information for Logged MDT, DL signal quantities measurement of Immediate MDT, RLF report, accessibility measurement and out-of-coverage logging in LTE have been described in our paper [2].

In LTE, the positioning method in MDT configuration provided by OAM is an optional parameter for immediate MDT. The positioning methods required by OAM for M1 of immediate MDT includes: E-Cell ID and GNSS. If OAM does not require specific positioning method, the best effort positioning method is used (i.e., in worst case only cell ID is provided).  For Logged MDT, UE is required to include available positioning information (ECGI of serving cell, GNSS if available and if UE supports location report and optional ECGI of neighboring cells). However, if UE does not support “locationReport”, the detailed location information, i.e., GNSS, will not be included, even the information is available when the measurements were taken.

# 3	Discussion

Including the detailed location information (e.g., GNSS) is one of the key advantages for MDT use cases described in Section 2.

In RAN2 #106, the following agreement was made about location information report for MDT:

In Rel-16, UE shall include the GNSS location information for Logged MDT, DL signal quantities measurement of Immediate MDT, RLF report, accessibility measurement and out-of-coverage logging, if the GNSS location information is available when the measurement was taken for the UEs with the GNSS receiver.

In LTE, the parameter, “locationReport”, indicates whether UE supports reporting of location information to eNB is defined in TS 36.306 and in the IE UE-EUTRA-Capability in TS 36.331 shown below.

#### 4.3.13.5 locationReport-r14 This parameter defines whether the UE supports reporting of its geographical location information to eNB.

UE-BasedNetwPerfMeasParameters-v1430 ::= SEQUENCE { locationReport-r14 ENUMERATED {supported} OPTIONAL }

The parameter, standaloneGNSS-Location, indicates whether UE is equipped with a standalone GNSS receiver is defined in TS 36.306 and in the IE UE-EUTRA-Capability in TS 36.331 shown below.

#### 4.3.13.2 standaloneGNSS-Location This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE.

UE-BasedNetwPerfMeasParameters-r10 ::= SEQUENCE { loggedMeasurementsIdle-r10 ENUMERATED {supported} OPTIONAL, standaloneGNSS-Location-r10 ENUMERATED {supported} OPTIONAL }

In LTE, even if UE is capable of GNSS (indicated by “standaloneGNSS-Location”) and GNSS location information is available when the measurements for MDT purpose were taken, the UE can still indicate that it does not support the location report.

The agreement made in RAN2 #106 clearly state that UE shall include the GNSS location information if the location information is available when the measurement was taken.

For the area-based (i.e., management-based) MDT in LTE, the MDT configurations are sent to the eNB. After receiving the MDT activation, eNB performs the UE selection based on the input information received from OAM, the device capability (MDT capable and UE capable of logging Measurements for MBSFN) and the availability of the user consent information (i.e., the Management Based MDT Allowed IE). The RAN node may not have sufficient number of UEs which can provide MDT data with detailed location information to select to perform management-based MDT if there are many UEs which do not support “locationReport”.

For area-based MDT in LTE, the number of UEs to be selected by eNB to perform MDT with location information can be limited since UEs which are in the desired area may not indicate the support of location report.

For signaling-based MDT, the UE selection is performed by OAM based on IMSI/IMEI(SV)/IMEI-TAC or IMSI/IMEI(SV)/IMEI-TAC combined with geographical area. After the IMSI, IMEISV or IMEI-TAC type user attached to the network, the MME shall forward the MDT configurations to the corresponding eNB, when the area criterion is satisfied if the area criterion is configured.

For signaling-based MDT in LTE, the UE selected to perform MDT based on IMSI/IMEI(SV)/IMEI-TAC or IMSI/IMEI(SV)/IMEI-TAC combined with geographical area may not support the location report.

The location report is optional which results that many UE do not support the location report even if the location information (e.g., GNSS information) can be derived when the UE performs logged measurement. There is a difference between the UE can report but it does not, and the detailed location information is not available so that the UE cannot report. To support the use cases of MDT stated in section 2, It is required that a UE which is equipped with a standalone GNSS receiver to report the GNSS location information (when available), given that the detailed location information is essential for the use cases.

Based on the agreement in RAN2 #106 and the above discussions, we have the following proposals:

- In Rel-16, the “locationReport” is not required in UE-based network performance measurement parameters in TS 38.306; the “locationReport” parameter is not required in UE-NR-Capability field in TS 38.331.

UE capability is one of the criteria during the UE selection in MDT (i.e., AMF selects UE for signaling-based MDT or RAN nodes choose UE for management-based MDT).

- In Rel-16, RAN2 to introduce “standaloneGNSS-Location”, as in LTE, to indicate whether UE is equipped with a standalone GNSS receiver, in TS 38.306 and TS 38.331.

The location information IE in LTE RRC is currently defined as

#### – LocationInfo The IE LocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information. LocationInfo information element -- ASN1START LocationInfo-r10 ::= SEQUENCE { locationCoordinates-r10 CHOICE { ellipsoid-Point-r10 OCTET STRING, ellipsoidPointWithAltitude-r10 OCTET STRING, ..., ellipsoidPointWithUncertaintyCircle-r11 OCTET STRING, ellipsoidPointWithUncertaintyEllipse-r11 OCTET STRING, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r11 OCTET STRING, ellipsoidArc-r11 OCTET STRING, polygon-r11 OCTET STRING }, horizontalVelocity-r10 OCTET STRING OPTIONAL, gnss-TOD-msec-r10 OCTET STRING OPTIONAL, ..., [[ verticalVelocityInfo-r15 CHOICE { verticalVelocity-r15 OCTET STRING, verticalVelocityAndUncertainty-r15 OCTET STRING } OPTIONAL ]] } -- ASN1STOP LocationInfo field descriptions. ellipsoidArc Parameter EllipsoidArc defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. ellipsoid-Point Parameter Ellipsoid-Point defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. ellipsoidPointWithAltitude Parameter EllipsoidPointWithAltitude defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. ellipsoidPointWithAltitudeAndUncertaintyEllipsoid Parameter EllipsoidPointWithAltitudeAndUncertaintyEllipsoid defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. ellipsoidPointWithUncertaintyCircle Parameter Ellipsoid-PointWithUncertaintyCircle defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. ellipsoidPointWithUncertaintyEllipse Parameter EllipsoidPointWithUncertaintyEllipse defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. gnss-TOD-msec Parameter Gnss-TOD-msec defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. horizontalVelocity Parameter HorizontalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. polygon Parameter Polygon defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. verticalVelocityAndUncertainty Parameter verticalVelocityAndUncertainty corresponds to horizontalWithVerticalVelocityAndUncertainty defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.. verticalVelocity Parameter verticalVelocity corresponds to horizontalWithVerticalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit..

As discussed in section 2, there are several positioning methods in MDT. For detailed location information, GNSS is the only choice. But GNSS information may not be always available.  In our view, RAN2 should investigate high-accuracy positioning methods, for example, UE based NR positioning [3].

- RAN2 to discuss introduction of UE-based NR positioning as one of the positioning methods in MDT for NR.

Furthermore, RAN2 has also agreed to support barometric pressure and IMU sensor measurements with MDT. Therefore, when designing the location information reporting with MDT in NR it is natural to use the ProvideLocationInformation IE in LPP [4] as template. LPP was originally designed for LTE, but has also been agreed to be used for NR positioning.

From LPP, section 6.3:

#### – ProvideLocationInformation The ProvideLocationInformation message body in a LPP message is used by the target device to provide positioning measurements or position estimates to the location server. -- ASN1START # ProvideLocationInformation ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { provideLocationInformation-r9 ProvideLocationInformation-r9-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } # ProvideLocationInformation-r9-IEs ::= SEQUENCE { commonIEsProvideLocationInformation CommonIEsProvideLocationInformation OPTIONAL, a-gnss-ProvideLocationInformation A-GNSS-ProvideLocationInformation OPTIONAL, otdoa-ProvideLocationInformation OTDOA-ProvideLocationInformation OPTIONAL, ecid-ProvideLocationInformation ECID-ProvideLocationInformation OPTIONAL, # epdu-ProvideLocationInformation EPDU-Sequence OPTIONAL, # ..., # [[ # sensor-ProvideLocationInformation-r13 # Sensor-ProvideLocationInformation-r13 # OPTIONAL, # tbs-ProvideLocationInformation-r13 TBS-ProvideLocationInformation-r13 OPTIONAL, # wlan-ProvideLocationInformation-r13 WLAN-ProvideLocationInformation-r13 OPTIONAL, # bt-ProvideLocationInformation-r13 BT-ProvideLocationInformation-r13 OPTIONAL # ]] } -- ASN1STOP

Therefore, it is relevant to start defining locationInfo in NR RRC in a generic manner.

- RAN2 to introduce a generic LocationInfo IE in NR RRC to allow multiple positioning methods including the ones already agreed.

- RAN2 to introduce, in the new LocationInfo IE in NR RRC, a CommonIEsLocationInfo attribute for position estimates, and a Sensor-LocationInfo attribute for sensor information.

- RAN2 to confirm that the CommonIEsLocationInfo and Sensor-LocationInfo attributes can be reported independent of each other.

The common location information in LPP represents a position defined using different possible means and positioning methods. Therefore, there is a separate location source specified to indicate what positioning method that has been used to estimate the location. The LTE location source is defined as

LocationSource-r13 ::= BIT STRING { a-gnss (0), wlan (1), bt (2), tbs (3), sensor (4), ha-gnss-v1510 (5) } (SIZE(1..16))

- RAN2 to introduce a locationSource IE in NR RRC to indicate the specific positioning method that has been used to determine the generic location information.

Based on the location information definition in LPP, and the current RAN2 agreements, we propose to introduce a generic IE locationInfo with position estimates and/or sensor data from barometric pressure sensors, IMUs etc. The proposal is detailed in the text proposal in the Appedix

- RAN2 to agree on the text proposal in Appendix A.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In LTE, even if UE is capable of GNSS (indicated by “standaloneGNSS-Location”) and GNSS location information is available when the measurements for MDT purpose were taken, the UE can still indicate that it does not support the location report.

Observation 2	The agreement made in RAN2 #106 clearly state that UE shall include the GNSS location information if the location information is available when the measurement was taken.

Observation 3	For area-based MDT in LTE, the number of UEs to be selected by eNB to perform MDT with location information can be limited since UEs which are in the desired area may not indicate the support of location report.

Observation 4	For signaling-based MDT in LTE, the UE selected to perform MDT based on IMSI/IMEI(SV)/IMEI-TAC or IMSI/IMEI(SV)/IMEI-TAC combined with geographical area may not support the location report.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In Rel-16, the “locationReport” is not required in UE-based network performance measurement parameters in TS 38.306; the “locationReport” parameter is not required in UE-NR-Capability field in TS 38.331.

Proposal 2	In Rel-16, RAN2 to introduce “standaloneGNSS-Location”, as in LTE, to indicate whether UE is equipped with a standalone GNSS receiver, in TS 38.306 and TS 38.331.

Proposal 3	RAN2 to discuss introduction of UE-based NR positioning as one of the positioning methods in MDT for NR.

Proposal 4	RAN2 to introduce a generic LocationInfo IE in NR RRC to allow multiple positioning methods including the ones already agreed.

Proposal 5	RAN2 to introduce, in the new LocationInfo IE in NR RRC, a CommonIEsLocationInfo attribute for position estimates, and a Sensor-LocationInfo attribute for sensor information.

Proposal 6	RAN2 to confirm that the CommonIEsLocationInfo and Sensor-LocationInfo attributes can be reported independent of each other.

Proposal 7	RAN2 to introduce a locationSource IE in NR RRC to indicate the specific positioning method that has been used to determine the generic location information.

Proposal 8	RAN2 to agree on the text proposal in Appendix A.

# References

TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT), 3GPP, V15.0.0, 2018-07

R2-1906871, Location Information Report in MDT, Ericsson, RAN WG2 #106, Reno, USA, 13 – 17 May 2019

R2-1908374, UE Based Positioning Method, Ericsson, RAN WG2 #106, Reno, USA, 13 – 17 May 2019

3GPP TS 36.355 V.15.4.0, LTE Positioning Protocol (LPP).

# Appendix A Text Proposal for generic NR RRC locationInfo

#### –	LocationInfo

The IE LocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

LocationInfo information element

-- ASN1START

# LocationInfo-r16 ::= SEQUENCE {

commonIEsLocationInfo-r16			CommonIEsLocationInfo-r16			OPTIONAL,

# sensor-LocationInfo	-r16			Sensor-LocationInfo-r16			    OPTIONAL,

}

-- ASN1STOP

[…]

#### –	CommonIEsLocationInfo

The IE CommonIEsLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonIEsLocationInfo information element

-- ASN1START

CommonIEsLocationInfo-r16 ::=	SEQUENCE {

locationCoordinates-r16					CHOICE {

ellipsoid-Point-r106									OCTET STRING,

ellipsoidPointWithAltitude-r16							OCTET STRING,

ellipsoidPointWithUncertaintyCircle-r16				    OCTET STRING,

ellipsoidPointWithUncertaintyEllipse-r16				OCTET STRING,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid-r16	OCTET STRING,

ellipsoidArc-r16										OCTET STRING,

polygon-r16												OCTET STRING,

highAccuracyEllipsoidPointWithUncertaintyEllipse-r16	OCTET STRING,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r16

OCTET STRING

},

locationSource-r16						OCTET STRING				OPTIONAL,

horizontalVelocity-r16					OCTET STRING				OPTIONAL,

gnss-TOD-msec-r16						OCTET STRING				OPTIONAL,

verticalVelocityInfo-r16		CHOICE {

verticalVelocity-r16				OCTET STRING,

verticalVelocityAndUncertainty-r15	OCTET STRING

}			OPTIONAL

}

-- ASN1STOP

LocationInfo field descriptions

ellipsoidArc Parameter EllipsoidArc defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoid-Point Parameter Ellipsoid-Point defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithAltitude Parameter EllipsoidPointWithAltitude defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid Parameter EllipsoidPointWithAltitudeAndUncertaintyEllipsoid defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithUncertaintyCircle Parameter Ellipsoid-PointWithUncertaintyCircle defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

ellipsoidPointWithUncertaintyEllipse Parameter EllipsoidPointWithUncertaintyEllipse defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

highAccuracyEllipsoidPointWithUncertaintyEllipse Parameter highAccuracyEllipsoidPointWithUncertaintyEllipse corresponds to HighAccuracyEllipsoidPointWithUncertaintyEllipse defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid Parameter highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid corresponds to HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter locationSource corresponds to LocationSource defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

gnss-TOD-msec Parameter Gnss-TOD-msec defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

horizontalVelocity Parameter HorizontalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

polygon Parameter Polygon defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

verticalVelocityAndUncertainty Parameter verticalVelocityAndUncertainty corresponds to horizontalWithVerticalVelocityAndUncertainty defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

verticalVelocity Parameter verticalVelocity corresponds to horizontalWithVerticalVelocity defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

[…]

#### –	Sensor-LocationInfo

The IE Sensor-LocationInfo is used by the target device to provide location information for sensor-based. It may also be used to provide sensor specific error reason.

-- ASN1START

Sensor-LocationInfo-r16 ::= SEQUENCE {

sensor-MeasurementInformation-r16		OCTET STRING							OPTIONAL,

sensor-MotionInformation-r16			OCTET STRING							OPTIONAL

}

-- ASN1STOP

Sensor-LocationInfo field descriptions

sensor-MeasurementInformation Parameter sensor-MeasurementInformation defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

sensor-MotionInformation Parameter sensor-MotionInformation defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.