3GPP TSG-RAN2 Meeting #107	R2-1909508

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On RRC message for logged MDT configuration in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_NR_data_collect |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2 #105, #105bis and #106 meetings, the following were agreed for Logged MDT configuration. - for logged MDT: the network sends logged measurement configuration to the UE in connected mode, and then the UE collects measurements in RRC_IDLE/INACTIVE. - Apply the Logged MDT configuration, logged measurements and reporting procedures to RRC_INACTIVE state. The RRC message for Logged MDT configuration need to be defined in TS 38.331 based on the agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include “loggedMeasurementConfiguration” and the related IEs in the RRC messages. Impact analysis Impacted functionality: Logged MDT. Inter-operability: This CR is for UE and network to implement and the proposed configuration message is sent from network to UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Logged MDT cannot be configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<         1st Change Start    <<<<<<<<<<<<<<<<<<<<<

### 6.2.2	Message definitions

* unaffected IEs are not included *

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by NR to configure the UE to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::= SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration               LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                     SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE {

traceReference-r16				TraceReference-r16,

traceRecordingSessionRef-r16	OCTET STRING (SIZE (2)),

tce-Id-r16						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16			AbsoluteTimeInfo-r16,

areaConfiguration-r16			AreaConfiguration-r16					             OPTIONAL,	--Need R

loggingDuration-r16				LoggingDuration-r16,

loggingInterval-r16				LoggingInterval-r16,

plmn-IdentityList-r16			PLMN-IdentityList3-r16			OPTIONAL,	--Need R

bt-NameList-r16					BT-NameList-r16					OPTIONAL, 	--Need R

wlan-NameList-r16				WLAN-NameList-r16				OPTIONAL, 	--Need R

nonCriticalExtension			SEQUENCE {}                                			 OPTIONAL

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

>>>>>>>>>>>>>>>>>>>>>         1st Change End     >>>>>>>>>>>>>>>>>>>>>

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [21].

PLMN-Identity information element

-- ASN1START

-- TAG-PLMN-IDENTITY-START

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC                 OPTIONAL,                   -- Cond MCC

mnc                                 MNC

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

-- TAG-PLMN-IDENTITY-STOP

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [21].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [21].

| Conditional Presence | Explanation |
| --- | --- |
| MCC | This field is mandatory present when PLMN-Identity is not used in a list or if it is the first entry of PLMN-Identity in a list. Otherwise it is optionally present, Need S. |

#### –	PLMN-IdentityList3

Includes a list of PLMN identities.

PLMN-IdentityList3 information element

-- ASN1START

PLMN-IdentityList3-r16 ::=				SEQUENCE (SIZE (1..16)) OF PLMN-Identity

-- ASN1STOP

#### –	PLMN-IdentityInfoList

<<<<<<<<<<<<<<<<<<<<<         2nd Change Start    <<<<<<<<<<<<<<<<<<<<<

### 6.3.2	Radio resource control information elements

<<<<<<<<<<<<<<<<<<<<<         2nd Change End    <<<<<<<<<<<<<<<<<<<<<

<<<<<<<<<<<<<<<<<<<<<         3rd Change Start    <<<<<<<<<<<<<<<<<<<<<

### 6.3.4	Other information elements

#### –	AbsoluteTimeInfo

The IE AbsoluteTimeInfo indicates an absolute time in a format YY-MM-DD HH:MM:SS and using BCD encoding. The first/ leftmost bit of the bit string contains the most significant bit of the most significant digit of the year and so on.

AbsoluteTimeInfo information element

-- ASN1START

-- TAG-ABSOLUTETIMEINFO-START

AbsoluteTimeInfo-r16 ::=				BIT STRING (SIZE (48))

-- TAG-ABSOLUTETIMEINFO-STOP

-- ASN1STOP

#### –	AreaConfiguration

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport.

AreaConfiguration information element

-- ASN1START

-- TAG-AREACONFIGURATION-START

AreaConfiguration-r16 ::=	CHOICE {

cellGlobalIdList-r16			CellGlobalIdList-r16,

trackingAreaCodeList-r16		TrackingAreaCodeList-r16,

trackingAreaIdentityList-r16    TrackingAreaIdentityList-r16

}

CellGlobalIdList-r16 ::=				SEQUENCE (SIZE (1..FFS)) OF CellIdentity

TrackingAreaCodeList-r16 ::=			SEQUENCE (SIZE (1..FFS)) OF TrackingAreaCode

TrackingAreaIdentityList-r16 ::=	    SEQUENCE (SIZE (1..FFS)) OF TrackingAreaIdentity-r16

TrackingAreaIdentity-r16 ::=	SEQUENCE {

plmn-Identity-r16			PLMN-Identity,

trackingAreaCode-r16        TrackingAreaCode

}

-- TAG-AREACONFIGURATION-STOP

-- ASN1STOP

#### –	EUTRA-AllowedMeasBandwidth

>>>>>>>>>>>>>>>>>>>>>         3rd Change End     >>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<         4th Change Start    <<<<<<<<<<<<<<<<<<<<<

### 6.3.4	Other information elements

#### –	BT-NameList

The IE BT-NameList is used to indicate the names of the Bluetooth beacon which the UE is configured to measure.

BT-NameList information element

-- ASN1START

BT-NameListConfig-r16 ::=		CHOICE{

release						NULL,

setup						BT-NameList-r16

}

BT-NameList-r16 ::=			SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16

BT-Name-r16 ::=		OCTET STRING (SIZE (1..248))

-- ASN1STOP

BT-NameList field descriptions

bt-Name If configured, the UE only performs Bluetooth measurements according to the names identified. For each name, it refers to LOCAL NAME defined in Bluetooth specification [X].

#### –	EUTRA-Q-OffsetRange

The IE EUTRA-Q-OffsetRange is used to indicate a cell, or frequency specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

EUTRA-Q-OffsetRange information element

-- ASN1START

-- TAG-EUTRA-Q-OFFSETRANGE-START

EUTRA-Q-OffsetRange ::=                     ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-EUTRA-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	LoggingDuration

The LoggingDuration indicates the duration for which UE is requested to perform measurement logging. Value min10 corresponds to 10 minutes, value min20 corresponds to 20 minutes and so on.

LoggingDuration information element

-- ASN1START

-- TAG-LOGGINGDURATION-START

LoggingDuration-r16 ::=			ENUMERATED {

min10, min20, min40, min60, min90, min120, spare2, spare1}

-- TAG-LOGGINGDURATION-STOP

-- ASN1STOP

#### –	LoggingInterval

The LoggingInterval indicates the periodicity for logging measurement results. Value ms1280 corresponds to 1.28s, value ms2560 corresponds to 2.56s and so on.

LoggingInterval information element

-- ASN1START

-- TAG-LOGGINGINTERVAL-START

LoggingInterval-r16 ::=			ENUMERATED {

ms1280, ms2560, ms5120, ms10240, ms20480,

ms30720, ms40960, ms61440}

-- TAG-LOGGINGINTERVAL-STOP

-- ASN1STOP

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

>>>>>>>>>>>>>>>>>>>>>         4th Change End     >>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<         5th Change Start    <<<<<<<<<<<<<<<<<<<<<

### 6.3.4	Other information elements

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type, for the identification of an RRC procedure (transaction).

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=       INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

#### –	TraceReference

The TraceReference contains parameter Trace Reference as defined in TS XX.XXX [YY].

TraceReference information element

-- ASN1START

-- TAG-TRACEREFERENCE-START

TraceReference-r16 ::=			SEQUENCE {

plmn-Identity-r16				PLMN-Identity,

traceId-r16						OCTET STRING (SIZE (3))

}

-- TAG-TRACEREFERENCE-STOP

-- ASN1STOP

#### –	WLAN-NameList

The IE WLAN-NameList is used to indicate the names of the WLAN AP for which the UE is configured to measure.

WLAN-NameList information element

-- ASN1START

WLAN-NameListConfig-r16 ::=		CHOICE{

release						NULL,

setup						WLAN-NameList-r16

}

WLAN-NameList-r16 ::=			SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF WLAN-Name-r16

WLAN-Name-r16 ::=		OCTET STRING (SIZE (1..32))

-- ASN1STOP

WLAN-NameList field descriptions

WLAN-Name If configured, the UE only performs WLAN measurements according to the names identified. For each name, it refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 [X].

## 6.4	RRC multiplicity and type constraint values

>>>>>>>>>>>>>>>>>>>>>         5th Change End     >>>>>>>>>>>>>>>>>>>>>