3GPP TSG-RAN WG2 Meeting #107			     R2-1909824

Prague, Czech Republic, 26th - 30th August 2019

Agenda Item	: 11.13.3 (NR_2step_RACH-Core)

Source	: LG Electronics Inc.

Title	: Issue on whether legacy msg2 can be reused for fallbackRAR

Document for	: Discussion and Decision

1.	Introduction

At RAN2 #106, the following agreements were made:

Agreements

- From RAN2 perspective, 2-step RACH selections can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states.  FFS if radio quality is used for 2-step RACH selection.

- From RAN2 perspective, for msgA retransmission (i.e. preamble and PUSCH) we assume that the UE retries on 2-step RACH

- FFS whether the UE can fallback to 4-step RACH after certain time.  Ask RAN1 whether the preamble transmission performance for 2-step RACH and 4-step RACH is the same.

- For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI)

- Contention resolution:

  - If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already.

  - If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly.

  - If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB.

  - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced

- For CCCH, MsgB can include the SRB RRC message.  The format should be designed for both with and without RRC message.

- For CCCH, for success or fallback RAR MsgB can multiplex messages for multiple UEs.  FFS if we can multiplex SRB RRC messages of multiple UEs.

- Network response to msgA (i.e. msgB/msg2) can include the following:

  - SuccessRAR

  - FallbackRAR

  - Backoff Indication

FFS: format of successRAR and whether successRAR is split into more than one message and format of fallbackRAR and whether legacy msg2 can be reused for fallbackRAR

- Proposal 10: The following fields can be included in the successRAR when CCCH message is included in msgA.

  - Contention resolution ID

  - C-RNTI

  - TA command

- Upon receiving the fallbackRAR, the UE shall proceed to msg3 step of 4-step RACH procedure

- FallbackRAR should contain the following fields

  - RAPID

  - UL grant (to retransmit the msgA payload).  FFS on restrictions on the grant and UE behavior if different grant and rebuilding

  - TC-RNTI

  - TA command

- From RAN2 perspective, no further offset is needed for the start of msgB monitoring window (i.e. no offset is needed to cover the RRC processing delay and/or F1 delay).

- The UE will monitor for response message using the single msgB agreed window

- MsgB containing the succcessRAR shall not be multiplexed with the legacy 4-step RACH RAR in the same MAC PDU

In this contribution, we’d like to discuss whether the legacy msg2 can be reused for fallbackRAR, and suggest to reuse the legacy msg2.

2.	Discussion

At the last meeting, there was an issue on whether the legacy msg2 can be reused as the fallbackRAR and it remains FFS. Depending on whether or not legacy msg2 is reused, we can consider the following two options for the fallbackRAR design.

- Option 1. Reuse the legacy msg2 as fallbackRAR, and msgB design only for successRAR

- Option 2. msgB design for either fallbackRAR or successRAR

For option 1, a UE may receive an msg2 addressed to RA-RNTI while RAR window is running. If the UE receives the msg2 including the RAPID which was transmitted in the msgA, the UE falls back to 4-step RACH by retransmitting the msgA payload. Or, the UE may receive the successRAR addressed to different RNTI from the RA-RNTI while the response window is running. If the UE receives the successRAR including the UE CRID matching the CCCH SDU which was transmitted in the msgA, the UE can consider the contention resolution successful. Since the UE can receive successRAR or msg2 within a same window after transmitting the msgA, the UE in option 1 should monitor both the successRAR and fallbackRAR addressed to the different RNTI within a response window. However, it is not a big burden for a UE because the current UEs in RRC_CONNECTED monitor several RNTIs, e,g, C-RNTI, CS-RNTI, SI-RNTI, and P-RNTI. In addition, according to the agreements at RAN2 #106, the contents of the fallbackRAR are totally the same as the msg2 of 4-step RACH. If we reuse the legacy msg2 for fallbackRAR, we can focus on the msgB design only for the successRAR.

Observation 1. If we reuse the legacy msg2 for fallbackRAR, the UE should monitor both the successRAR and fallbackRAR addressed to the different RNTI within a response window, but it is not a big burden for a UE.

On the other hand, the UEs in option 2 monitor msgB for either successRAR or fallbackRAR addressed to one RNTI. This means that one or more successRAR(s) and fallbackRAR(s) for multiple UEs can be multiplexed into one MAC PDU. The UEs which transmitted a preamble on the same RO monitor the same msgB. At the last meeting, we agreed that the successRAR can include SRB RRC message. If the successRAR contains SRB RRC message and it can be multiplexed with the fallbackRARs, the size of msgB will be too large. Because of the larger msgB, the gNB may want to send the split msgBs multiple times. For this reason, if the larger msgB is sent in multiple MAC PDUs, it would be more effective to send fallbackRAR and successRAR to different MAC PDUs. Even considering only the successRAR, if we reuse the RA-RNTI for successRAR monitoring, the UE might decode the many separate successRARs for other UEs. The smaller the granularity of UEs multiplexing successRAR, the lower the successRAR decoding overhead of the UE. From this point of view, it is beneficial to design a new msgB RNTI suitable for the successRAR while the fallbackRAR reuses the legacy msg2 addressed to RA-RNTI [1].

Observation 2. If we design the new msgB for either successRAR or fallbackRAR, it makes the msgB size too large.

Also, the option 2 is needed to an additional indicator for distinguishing the successRAR from BI or fallbackRAR. Given that the msgB is designed based on the legacy msg2, we can consider to use one of the reserved bits of the legacy msg2. But, it seems quite complicated because there is not enough room. In the results, it leads to more specification efforts than the option 1. Several companies suggested to include the RAPID for efficient parsing of the successRAR, but we think the processing complexity of a UE can be reduced by separating successRAR and fallbackRAR into different MAC PDUs rather than including RAPID in successRAR or defining a new indicator.

Observation 3. MsgB design for either successRAR or fallbackRAR requires more specification efforts than reusing the legacy msg2 as well as leads the high decoding overhead due to msgB of large size.

Therefore, we propose to reuse the legacy msg2 for the fallbackRAR.

Proposal. FallbackRAR reuse the legacy msg2.

3.	Conclusion

In this contribution, we discuss details on fall back procedure to 4-step RACH, and our proposals are as follows:

Observation 1. If we reuse the legacy msg2 for fallbackRAR, the UE should monitor both the successRAR and fallbackRAR addressed to the different RNTI within a response window, but it is not a big burden for a UE.

Observation 2. If we design the new msgB for either successRAR or fallbackRAR, it makes the msgB size too large.

Observation 3. MsgB design for either successRAR or fallbackRAR requires more specification efforts than reusing the legacy msg2 as well as leads the high decoding overhead due to msgB of large size.

Proposal. FallbackRAR reuse the legacy msg2.

4.	References

[1] R2-1909826	“RNTI design for msgB”, LG Electronics Inc.