3GPP TSG-RAN WG2#107 meeting	Tdoc R2-1909949

# Prague, Czech Republic, 26– 30 August 2019

Agenda Item:		11.12.2

Souce:	Samsung

Title:	Immediate MDT for early measurments

Document for:		Discussion and decision

# Introduction

This contribution discusses Immediate MDT for early measurements. I.e. the contribution proposes to support the option for network to indicate that UE should include location info when reporting results of early measurements, alike for regular RRM.

# Discussion

During the last meeting RAN2 reached several agreements regarding MDT, including agreements regarding immeditate MDT. There was however so far no discussion about immediate MDT for the early measurements that a UE in idle or inactive may be configured to report upon moving to connected, if network requests.

The early measurements can be provided either within RRCResumeComplete or in UEInformationResponse. As for RRM measurements, immediate MDT for early measurements would just introduce the option that network can request the UE to include location information together with the early measurement information. The latter request can either be provided together with early measurement configuration (i.e. at RRCRelease) or when results are retrieved. We have no strong view, but merely a slight preference for the first option (i.e. as this is similar to what we do for RRM).

Proposal	Introduce immediate MDT for early measurments by:

  - Adding a field by which network can request the UE to include location information in RRCRelease (within the early measurement configuration i.e. together with the general parameters set by MN e.g. timer)

  - Adding a (single) field for detailed location information in RRCResumeComplete and in UEInformationResponse

Note	If UE was in DC, it seems appropriate for SN to configure early measurements for SN-controlled non-camping frequencies. We think that in DC, the MN can set the includeLocationInfo as UE will just provide a single detailed location independent of which frequencies the UE provides results for. Note that in DC, we assume the MN collects and forwards the MDT results.

# Conclusion & recommendation

This contribution discusses Immediate MDT for early measurements. I.e. the contribution proposes to support the option for network to indicate that UE should include location info when reporting results, alike for regular RRM. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal	Introduce immediate MDT for early measurments by:

  - Adding a field by which network can request the UE to include location information in RRCRelease (within the early measurement configuration i.e. together with the general parameters set by MN e.g. timer)

  - Adding a (single) field for detailed location information in RRCResumeComplete and in UEInformationResponse

# References

[1] TS 38.331 Radio Resource Control