3GPP TSG-RAN WG2 #107	R2-1910137

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.4.2

Source:	Ericsson

Title:	SL handling during handover in NR

Document for:	Discussion, Decision

# 1	Introduction

During the RAN2#105 meeting, companies have discussed the handling of SL transmission in MR-DC scenario and the following agreement has been made:

Agreements on resource allocation/configuration:

1-11: SN is not allowed to control/configure SL resources in MRDC.

However, the case of how the SL transmissions are handled during handover in NR is still not clear. For this reason, in this contribution we address the SL configuration and resource coordination during HO between NG-RAN nodes. In doing this, we analyse the possible coordination needed between the source and target NG-RAN node in order to guarantee regular SL operations and no degradation of the performance.

# 2	Discussion

In NR Uu, intra-NR and inter-RAT mobility generally involve a coordination between the source and target NG-RAN node. Such a coordination is done by using messages exchanged via the X2/Xn signalling and require explicit RRC signalling to be triggered, i.e., handover request. As an example, the inter-gNB handover is illustrated in Figure 1.

Figure 1 - Inter-gNB handover procedure

During this procedure, the source gNB send a HANDOVER REQUEST message via X2/Xn signaling to the target gNB that reply in a later moment with a HANDOVER REQUEST ACKNOWLEDGE message. Further, one of the important information that is exchanged (i.e., within the Handover Request message) is the UE context information that allows the NG-RAN node, where the UE RRC Connection has been requested to be established, to know all the relevant information of the new UE, but also enables the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer. The UE context information, indeed, is a block of information associated to one active UE that generally contains the necessary information required to maintain the network services towards the active UE and to allocate correctly the radio resources by the target NG-RAN node.

In NR Uu, during the handover procedure the UE context is exchanged from the source to the target NG-RAN node in order to maintain the network services towards the active UE and to allocate correctly the radio resources by the target NG-RAN node.

When considering SL transmission during handover procedures in NR Uu, at the moment how SL should be handled has not been discussed so far. In fact, according to the current specification, when an handover is performed the SL transmission needs to be release (or transmission mode needs to be changed) since the target gNB is not aware of the ongoing SL transmission and the SL configuration (and radio resources) previously configured by the source NG-RAN node may be node valid anymore. This of course implies a SL connectivity interruption and an additional signalling overhead due to the fact that the target NG-RAN needs to configure a new SL transmission.

According to current specification, during a handover in NR, since no SL UE context is exchanged between the source and target NG-RAN node, the SL connectivity needs to be release with a consequent connectivity interruption.

In order to avoid such problem, a straightforward solution would be to follow the LTE/NR Uu principles and to allow the source gNB to forward the SL UE context to the target gNB. The benefits of this solution are primarily that the target NG-RAN can allocate in a more efficient way the SL radio resource for the upcoming UE, but also that may guarantee the network services towards without forcing the UE to release the SL transmission. Therefore, we propose:

During handover, the SL UE context information is sent from the source NG-RAN to the target NG-RAN node.

However, there is still an open question about how the SL UE context information exchange should happen. Looking at the content of the HANDOVER REQUEST message in 3GPP TS 38.423 [1], it includes the following fields:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |

An important field of the UE context information structure contained in the HANDOVER REQUEST message is the RRC Context. As a matter of fact, this field is just a container that carries the entire HandoverPreparationInformation message as defined LTE or NR RRC specification (i.e., 3GPP TS 36.331 [3] for LTE and 3GPP TS 38.331 [2] for NR) depending the RAT of the target NG-RAN node. In particular, this RRC message is used to transfer relevant information for the target NG-RAN node such as UE capability information, the radio resource configuration as used in the source cell, relevant RRC configurations, and available measurements.

The HandoverPreparationInformation message generated by the RRC is used to transfer the relevant information for the target NG-RAN node such as UE capability information, the radio resource configuration as used in the source cell, relevant RRC configurations, and available measurements.

The HandoverPreparationInformation message is embedded in a container within the HANDOVER REQUEST message in the X2/Xn signalling.

According to this, would be good to apply the same principle also for the SL UE context information and thus adding all the relevant SL information that may be useful at the target NG-RAN node within the HandoverPreparationInformation message as defined in TS 3GPP 38.331 [2]. Therefore, we propose:

During handover, the SL UE context information is exchanged via inter-node RRC messages (i.e., within the HandoverPreparationInformation message). FFS what information to include in the SL UE context information.

Once the target NG-RAN node is aware of the SL UE context information, it can reserve radio resource accordingly and can generate (or just re-use) a SL configuration to be forwarded to the UE in order to guarantee the SL session continuity. In order to do so, a possible solution would be to allow the target NG-RAN node to forward the decided SL configuration and radio resource scheduling via the HandoverCommand message as defined in the 3GPP TS 38.331 [2]. The HandoverCommand message is then embedded into the HANDOVER REQUEST ACKNOWLEDGE (i.e., similarly as we have for the HandoverPreparationInformation embedded into the HANDOVER REQUEST) via the X2/Xn interface. Thus:

During handover, the SL configuration and radio resource scheduling decided by the target NG-RAN node are sent to the source NG-RAN node via inter-node RRC messages (i.e., within the HandoverCommand message).

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	In NR Uu, during the handover procedure the UE context is exchanged from the source to the target NG-RAN node in order to maintain the network services towards the active UE and to allocate correctly the radio resources by the target NG-RAN node.

Observation 2	According to current specification, during a handover in NR, since no SL UE context is exchanged between the source and target NG-RAN node, the SL connectivity needs to be release with a consequent connectivity interruption.

Observation 3	The HandoverPreparationInformation message generated by the RRC is used to transfer the relevant information for the target NG-RAN node such as UE capability information, the radio resource configuration as used in the source cell, relevant RRC configurations, and available measurements.

Observation 4	The HandoverPreparationInformation message is embedded in a container within the HANDOVER REQUEST message in the X2/Xn signalling.

Based on the discussion in the previous sections we propose the following:

Proposal 1	During handover, the SL UE context information is sent from the source NG-RAN to the target NG-RAN node.

Proposal 2	During handover, the SL UE context information is exchanged via inter-node RRC messages (i.e., within the HandoverPreparationInformation message). FFS what information to include in the SL UE context information.

Proposal 3	During handover, the SL configuration and radio resource scheduling decided by the target NG-RAN node are sent to the source NG-RAN node via inter-node RRC messages (i.e., within the HandoverCommand message).

# 5	References

3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), 3GPP, 15.4.0, July 2019

3GPP TS 38.331, NR; Radio Resource Control (RRC); Protocol specification, 3GPP, 15.6.0, July 2019

3GPP TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, 3GPP, 15.6.0, July 2019.