3GPP TSG RAN WG2 Meeting #107	R2-1910144

Prague, Czech Republic, 26th – 30th August 2019	is revision of R2-1907156

Source:	Ericsson

Title:	MAC CE signaling impact of enhanced TCI indication framework

Agenda Item:	11.16

Document for:	Other

# Introduction

In the incoming LS of [1], RAN2 is informed of the following which was agreed in RAN1 as part of the Rel-16 NR MIMO WI:

For single PDCCH based multi-TRP/panel transmission,

- TCI indication framework shall be enhanced in Rel-16 at least for eMBB. Each TCI code point in a DCI can correspond to 1 or 2 TCI states. When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1 and FFS design for DMRS type 2. MAC-CE can be enhanced to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2.

According to the above agreement, each codepoint in the Transmission Configuration Indication (TCI) field in DCI can be mapped to either 1 or 2 TCI states.  This can be interpreted as follows:

A DCI in PDCCH schedules a PDSCH where a first set of layers is associated with a first TCI state and a second set of layers is associated with a second TCI state (if 2 TCI states are indicated by the TCI field).  Additionally, according to the above agreement, at least for DMRS type 1, PDSCH DMRS associated with one TCI state is contained within one DMRS CDM group.

In this paper, we provide our views on the MAC-CE signaling impact of the Rel-16 TCI indication framework.

# Discussion of enhancements

In NR Rel-15, up to 8 TCI states can be activated by TCI States Activation/Deactivation for UE-specific PDSCH MAC CE. The 3-bit TCI field in DCI is used to point to one of the activated TCI states.  There is a one-to-one mapping between the activated TCI states and the codepoints of the TCI field in DCI. The MAC CE for activating/deactivating TCI states for UE specific PDSCH in NR Rel-15 as specified in 3GPP TS 38.321 includes the following fields:

- A Serving Cell ID field that indicates the identity of the serving cell for which the MAC CE applies

- A BWP ID field that indicates the downlink bandwidth part for which the MAC CE applies

- A variable number of Ti fields each of which indicates the activation/deactivation status of one of the TCI states.  The codepoint to which a TCI state is mapped is determined by the ordinal position among all the TCI States which are activated.

- A Reserved bit R: this bit is set to ‘0’ in NR Rel-15.

In the NR Rel-15 MAC CE for TCI States Activation/Deactivation for UE-specific PDSCH, a single codepoint of the TCI field can only be mapped to a single TCI State.  The NR Rel-15 MAC CE cannot be used for Rel-16 NR enhanced TCI indication framework where one codepoint in the TCI field needs to be mapped to either 1 or 2 TCI states.

Furthermore, it is beneficial to dynamically switch between receiving from one or two TRPs.  Hence, it is beneficial to support TCI indication framework in Rel-16 that allows different TCI field codepoints to indicate different number of TCI states.  That is, we should support the possibility that some TCI field codepoints indicate a single TCI state and some other TCI field codepoints indicate multiple TCI states.

Proposal 1:  Rel-16 NR allows different TCI field codepoints to indicate different number of TCI states.  That is, we should support the possibility that some TCI field codepoints indicate a single TCI state and some other TCI field codepoints indicate multiple TCI states.

One aspect that will impact the MAC CE signalling is how to map up to two TCI states a single codepoint in the TCI field.   One straightforward approach is to first RRC configure a list of TCI trigger states where each TCI trigger state may contain either a single TCI state or two TCI states.  Then, up to 8 of the TCI trigger states can be activated via MAC CE and mapped to the codepoints of the TCI field.  Note that this approach is similar to CSI-AperiodicTriggerStateList from which one of the CSI-AperiodicTriggerState can be associated via MAC CE with a codepoint of the ‘CSI request’ field in DCI.

Proposal 2:  For Rel-16 enhanced TCI indication, consider the approach of RRC configuring a list of TCI trigger states and associating one of the TCI trigger states to a codepoint in the TCI field in DCI

The number of TCI states in a TCI trigger state can be either 1 or 2.

# Conclusion

In this paper, we provide our views on the MAC-CE signaling impact of the Rel-16 TCI indication framework.  Based on the discussion in this contribution these proposals are made:

Proposal 1:  Rel-16 NR allows different TCI field codepoints to indicate different number of TCI states.  That is, we should support the possibility that some TCI field codepoints indicate a single TCI state and some other TCI field codepoints indicate multiple TCI states.

Proposal 2:  For Rel-16 enhanced TCI indication, consider the approach of RRC configuring a list of TCI trigger states and associating one of the TCI trigger states to a codepoint in the TCI field in DCI

The number of TCI states in a TCI trigger state can be either 1 or 2.

# References

[1]	R2-1903010, ‘LS on support of Enhancements on multi-TRP/panel transmission (R1-1903697)’ RAN1 WG1