3GPP TSG-RAN WG2 # 107	R2-1910681

Prague, CR, 26-30, August, 2019

Agenda Item:	11.13.5

Source:	Huawei, HiSilicon

Title:	Measurement gap and 2-step RACH

Document for:	Discussion, Decision

Introduction

MsgA transmission and the measurement gap has been discussed in [6], this contribution further discusses the issues related to the MsgA transmission and MsgB Reception.

Discussion

msgA transmission and measurement gap

In 38.321, the measurement gap is considered during the RACH procedure.

5.1.2 Random Access Resource selection 1> else if an SSB is selected above: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB). #### 5.4.2.2 HARQ process To generate a transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer; or 1> if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer: 2>instruct the physical layer to generate a transmission according to the stored uplink grant.

In R15, UE may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion, and UE shall transmit the Msg3 PUSCH regardless of the occurrence of a measurement gap.

For 2-step RACH, the similar principle can be reused. In Figure 1, we illustrate two possible cases of collision between msgA time and frequency resources and measurement gap: (a) PRACH occasion collides with measurement gap; (b) PUSCH occasion collides with measurement gap.

Figure 1: Collision between 2-step RA and measurement GAP

For the first case, following LTE and R15 NR, UE should transmit the MsgA payload in the PUSCH resource during a measurement gap if the corresponding preamble is already transmitted in a preceding RACH occasion, it is illustrated as Case A in Figure 1.

Proposal 1: UE is allowed to transmit the MsgA payload even during a measurement gap.

For the second case, UE may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion for the 2-step RACH occasion for case B. If the PRACH occasion overlaps the measurement GAP, it is up to the implementation of the UE to either skip this this 2-step RACH resource, it is illustrated as case B in Figure 1. Note that skipping the PRACH resource and its corresponding PUSCH resource will delay the 2-step transmission.

Proposal 2: UE take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion

msgB reception and measurement gap

In R15, once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, UE shall monitor the PDCCH for the RAR reception while the RAR timer is running. Once Msg3 is transmitted, UE shall monitor the PDCCH for the contention resolution reception while the Contention Resolution Timer is running regardless of the possible occurrence of a measurement gap. This is shown in the following text in the R15 MAC spec.

5.1.4 Random Access Response reception Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall 1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission; 2> monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running. 5.1.5 Contention Resolution Once Msg3 is transmitted, the MAC entity shall: 1> start the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission; 1> monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap;

For the 2-step RACH, after reception of msgA, UE monitor PDCCH to receive msgB instead of msg2 and msg4. Similar to the LTE behaviour, we think the UE should monitor the PDCCH for the MsgB Reception regardless of the measurement gap.

Proposal 3: UE shall monitor the PDCCH for the MsgB Reception regardless of the measurement gap during the MsgB timer is running.

Conclusion

Base on the analysis above, we propose the following:

Proposal 1: UE take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion

Proposal 2: UE is allowed to transmit the MsgA payload even during a measurement gap.

Proposal 3: UE shall monitor the PDCCH for the MsgB Reception regardless of the measurement gap during the MsgB timer is running.

References

RP-182894, “New work item: 2-step RACH for NR,” ZTE Corporation, Sanechips, RAN#82.