3GPP TSG-RAN WG2 Meeting #106	R2-1910694

Prague,  Czech Republic, 26 – 30 August 2019

Agenda item:	11.6.4.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Pre-trigger based mobility for NTN

WID/SID:	FS_NR_NTN_solutions - Release 16

Document for:	Discussion and Decision

# 1	Introduction

The following general challenges for non-terrestrial mobility have been identified and captured in TR 38.821 [1], based on the outcome of a previous email discussion [2]:

For GEO NTN, mobility management procedures require adaptations to accommodate large propagation delay. In particular radio link management may require specific configuration. For LEO NTN, mobility management procedures should be enhanced to take into account satellite movement related aspects such as measurement validity, UE velocity, movement direction, large and varying propagation delay and dynamic neighbour cell set.

Furthermore, the [106#73][NR/NTN ] “Mobility issues and solutions” [3] email discussion is meant to capture and discuss mobility challenges and possible solutions for non-terrestrial networks proposed so far in RAN2.

Following are among the main challenges identified for mobility in LEO NTN scenarios:

- Latency associated with mobility signalling

- Measurement validity and need for signalling latency reduction

- Handover for large number of UEs (cell level mobility)

- Conditional Handover (enhancements)

This contribution proposes a measurement reporting and mobility signalling mechanism which minimises the impact the large signalling latency. The proposed mechanism can be used with ‘normal’ hand-over procedures or in condition hand-over setups.

# 2	Radio mobility aspects in NTN

The general radio mobility problem in a LEO NTN scenario is how to avoid long handover procedure times (measurement, preparation, execution) to a new satellite cell, and at the same time reduce the risk of handover failures considering the low signal levels at which these cell changes happen (extreme link budget).

Previous 3GPP solutions for LTE and 5G NR were developed to reduce hand-over times in terrestrial networks (make-before-break, conditional handover, dual-connectivity based handover); these might not be optimal, or not possible to be used, directly for NTN scenarios due to the largely different radio propagation conditions, fast movements of the target and source cell(s) and ultimately large differences between the signals received at the UE depending on its relative location w.r.t. to the source and target cell(s).

Furthermore, most of the handover enhancements developed for terrestrial networks assume that data can be forwarded from the source cell to the candidate target satellite cells in timely manner. This is a strong assumption in NTN scenarios and might be even impossible to achieve when a very large number of UEs are to perform handover (satellite cell coverage areas are in the order of 200km to 500km radius).

For NTN it has been proposed to use the ephemeris data [1][3] of the satellites in combination with UE geo-location information in addition to traditional radio measurements to trigger handovers between NR cells from different satellites (inter-satellite) or between NR cells from the same satellite (intra-satellite). However, when the UE geo-location is not known – the current baseline in TR38.821 – the radio handovers must still rely on UE radio measurements and reporting mechanism, even if the ephemeris data of the satellites is available.

A typical scenario where a radio hand-over is triggered according to currently specified mechanisms TR38.331, is depicted in Figure 1 below:

Figure 1 - Example of mobility measurements and handover event trigger

When the configured trigger conditions are fulfilled for timeToTrigger time duration the UE reports the event to the source cell (SpCell) and the network starts the handover procedure.

In an NTN LEO scenario, due to large RTT values, there is a large delay until the handover to the target cell (NCell1) is executed. During this time period the channel conditions of the UE can change, especially for the SpCell corresponding to a LEO satellite moving fast below the minimum ‘visibility’ elevation angle.

Secondly, the slope of the RSRP signal level curves (as experienced by the UE) shown in Figure 1 depend on the antenna patterns, relative movement of the satellite gNB and UE. This further means that the same two cells, SpCell and NCell1, could be perceived with different RSRP levels and changing rates depending on the UE location on Earth w.r.t. to the two satellites’ coverage areas. Therefore, using the same configuration values for the radio measurements and mobility events for the two (or more, or all) UE locations can lead to very different outcomes in terms of successful handover (as an example).

Thirdly, at larger time scale and lower accuracy, the signal level curves can be pre-estimated for any geographical location/area due to the deterministic movement of the satellites. This removes some of the uncertainties in the possible signal level changes at the UE, as compared to the case of terrestrial networks. Though, the NTN channel conditions will still vary at smaller scale due to fast fading, body/building/vehicle shadowing, vegetation shadowing, atmospheric conditions, etc.

Observation 1: In NTN the currently specified mobility mechanism could lead to HO failure/RL failure due to the large signalling delay during the hand-over decision/preparation/execution time; the channel conditions of the UE can change, especially for the serving cell corresponding to a LEO satellite moving fast below the minimum ‘visibility’ elevation angle

# 3	Pre-triggered mobility event

One main part of the large signalling latency could be mitigated in case the UE is able to report the event earlier compared to the case depicted in Figure 1. In case the UE is able to estimate (with certain accuracy) how the RSRP signal levels of the serving and neighbouring cells will change in the next x10ms time frame, then the radio handover could be predicted, and the event be triggered in advance. However, predicting accurately the radio channel condition is a notoriously difficult problem and would likely rely on implementation specific solutions (UE algorithms). For example, in [4] a gradient based scaling method has been proposed to estimate the future change of the RSRP values, where the gradient was defined as G = ({RSRPTargetCell(t) – RSRPSourceCell(t)} – {RSRPTargetCell(t-ΔT) – RSRPSourceCell(t-ΔT)})/ ΔT.

A more practical solution could rely instead on a configurable (by the RAN) mechanisms where the UE needs only to select among a few possible ‘trends’ in the signal levels (e.g. RSRP), and these signal trends are pre-calculated, updated and signalled to the UE(s), by the RAN. This would allow for RAN adaptation, so that not all NTN cells use the same set of signal trend configurations (possibly also changing vs. time) and, also accommodate the UE specific radio channel conditions. Note that precise geographical area of the UE would not be needed, because in NTN the signal trends are very similar for larger geographical areas. Furthermore, the network could also include the estimated movement of the UE in the expected signal trends, when this information is available.

Observation 2: In NTN scenarios, the average radio channel conditions exhibit a more ‘predictable’ behaviour compared to terrestrial networks and could allow for mechanism which are triggered in advance to reduce the risk of HO failure/RL failures.

Figure 2 shows the proposed pre-triggered mobility mechanisms. The pre-trigger event is configured as a condition based on the relative RSRP (RSRP-diff) values between the SpCell and NCell1 along with a corresponding timeToPreTrigger value.

Figure 2 - Proposed pre-triggered mobility event (example)

Solution details:

- The network provides the UE with the expected trends of the radio measurements (RSRP, RSRQ) based on the area the UE is located in or based on the coverage footprint of the satellite and its neighbor satellites.

- To minimize the signaling of the signal trend information, these trends can be provided for example in the form of timeToPreTrigger vs. gradient RSRP-diff, or RSRP-diff vs. time. (Further refinement is possible, for example by including different trend information for different neighbor cell ID and/or different target timeToPreTrigger values.)

- The UE compares its radio measurements with the expected trend and once they match with certain accuracy (also configurable by the network), it reports back to the Network, that there is a match together with the corresponding timeline (timeToPreTrigger and/or neighbor cell ID), such that the network can anticipate and prepare for a handover to one or more target cells.

Some of the main advantages of this proposed solution are:

- Earlier (in advance) and potentially more accurate measurement reports available at the SpCell, which allow to perform better hand-over decisions

- Avoid potential HO failures due to significant SpCell signal degradation during the HO decision, preparation and signalling time (including large RTT to/from satellite)

- Can be used as condition/trigger for Conditional Handover including prioritization of target cells

- No need to rely on the use of precise UE location information and the knowledge of satellite constellation ephemeris data at the network is enough to configure the pre-trigger event

Proposal 1: RAN2 to consider and study the feasibility of mobility events which use pre-estimated average radio channel behaviour and introduce “pre-triggering” events to mitigate the large propagation delays

Proposal 2: RAN2 to agree to capture the below TP in the TR 38.821

# 4	Conclusion

We conclude with the following observations and proposal:

Observation 1: In NTN the currently specified mobility mechanism could lead to HO failure/RL failure due to the large signalling delay during the hand-over decision/preparation/execution time; the channel conditions of the UE can change, especially for the serving cell corresponding to a LEO satellite moving fast below the minimum ‘visibility’ elevation angle

Observation 2: In NTN scenarios, the average radio channel conditions exhibit a more ‘predictable’ behaviour compared to terrestrial networks and could allow for mechanism which are triggered in advance to reduce the risk of HO failure/RL failures.

Proposal 1: RAN2 to consider and study the feasibility of mobility events which use pre-estimated average radio channel behaviour and introduce “pre-triggering” events to mitigate the large propagation delays.

Proposal 2: RAN2 to agree to capture the below TP in the TR 38.821

# References

3GPP TR 38.821 V0.7.0 Solutions to support non-terrestrial networks (NTN)

R2-1900547 Report of email discussion [104#54] [NR – NTN]

Email discussion [106#73][NR/NTN ] Mobility issues and solutions (InterDigital)

R2-132597, “Gradient based scaling”, 3GPP TSG-RAN2#83 meeting

# Text Proposal for TR 38.821

<<TP start>>

### 8.3.8	Solution options for LEO NTN mobility scenarios

### 8.3.8.1	Pre-triggered mobility event to minimize HO failures and RL failures

One main part of the large signalling latency could be mitigated in case the UE is able to report the event earlier compared to the case depicted in Figure 1. In case the UE is able to estimate (with certain accuracy) how the RSRP signal levels of the serving and neighbouring cells will change in the next x10ms time frame, then the radio handover could be predicted, and the event be triggered in advance. However, predicting accurately the radio channel condition is a notoriously difficult problem and would likely rely on implementation specific solutions (UE algorithms). For example, in [4] a gradient based scaling method has been proposed to estimate the future change of the RSRP values, where the gradient was defined as G = ({RSRPTargetCell(t) – RSRPSourceCell(t)} – {RSRPTargetCell(t-ΔT) – RSRPSourceCell(t-ΔT)})/ ΔT.

A more practical solution could rely instead on a configurable (by the RAN) mechanisms where the UE needs only to select among a few possible ‘trends’ in the signal levels (e.g. RSRP), and these signal trends are pre-calculated, updated and signalled to the UE(s), by the RAN. This would allow for RAN adaptation, so that not all NTN cells use the same set of signal trend configurations (possibly also changing vs. time) and, also accommodate the UE specific radio channel conditions. Note that precise geographical area of the UE would not be needed, because in NTN the signal trends are very similar for larger geographical areas. Furthermore, the network could also include the estimated movement of the UE in the expected signal trends, when this information is available.

Below figure shows the proposed pre-triggered mobility mechanisms. The pre-trigger event is configured as a condition based on the relative RSRP (RSRP-diff) values between the SpCell and NCell1 along with a corresponding timeToPreTrigger value.

Figure x - Proposed pre-triggered mobility event (example)

Solution details:

- The network provides the UE with the expected trends of the radio measurements (RSRP, RSRQ) based on the area the UE is located in or based on the coverage footprint of the satellite and its neighbor satellites.

- To minimize the signaling of the signal trend information, these trends can be provided for example in the form of timeToPreTrigger vs. gradient RSRP-diff, or RSRP-diff vs. time. (Further refinement is possible, for example by including different trend information for different neighbor cell ID and/or different target timeToPreTrigger values.)

- The UE compares its radio measurements with the expected trend and once they match with certain accuracy (also configurable by the network), it reports back to the Network, that there is a match together with the corresponding timeline (timeToPreTrigger and/or neighbor cell ID), such that the network can anticipate and prepare for a handover to one or more target cells.

Following are the main advantages of this proposed solution:

- Earlier (in advance) and potentially more accurate measurement reports available at the SpCell, which allow to perform better hand-over decisions.

- Avoid potential HO failures due to significant SpCell signal degradation during the HO decision, preparation and signalling time (including large RTT to/from satellite).

- Can be used as condition/trigger for Conditional Handover including prioritization of target cells.

- No need to rely on the use of precise UE location information and the knowledge of satellite constellation ephemeris data at the network is enough to configure the pre-trigger event.

<<TP end>>