3GPP TSG-RAN WG2 Meeting #107	 R2-1911559

# Prague, Czech Republic, 26 – 30 August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for the introduction of NR mobility enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for NR mobility enhancement into stage 2 specification. : |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture agreements for NR mobility enhancement into stage 2 specification: RAN2#107: - Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CHO configuration to UE. - UE is not required to continue evaluating the triggering condition of other candidate cell(s) during CHO execution. - For the scenario of multiple CHO cells being triggered the cell selected by the UE considering beams and beam quality. We will not specify normative requirements for the selection process but can be captured in an informative note in stage 3 spec. - For FR1, we will leave it up to UE implementation to select the target cell if more than one candidate cell meets the triggering condition (same as for FR2). - S1/N2-based CHO is not supported for Release 16. RAN3 impacst to be discussed in RAN3. If we want to support this, the WID has to be updated and SA2 impacts identified. - A RRC complete message is required for UE to confirm receipt and proper comprehension of CHO configuration (execution condition, FFS target cell configuration) to the source eNB/gNB. - Allow having multiple triggering conditions (using “and”) for CHO execution of a single candidate cell. Only single RS type per CHO candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultnaeously. FFS on UE capability. - Editorial changes; RAN2#106: -The source cell adds the condition for the execution of CHO to the RRC message sent to UE. - Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates) - On cell level A3/A5-like CHO execution condition shall be specified (other events will not be specified without clear justifications) RAN2#105b: - CHO is defined as UE having network configuration for initiating access to a target cell based on configured condition(s). - Usage of conditional handover is decided by network. UE evaluates when the condition is valid. - Support configuration of one or more candidate cells for conditional handover; - The baseline operation for Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met. - The baseline operation for Conditional HO assumes the source RAN remains responsible for RRC until UE successfully sends RRC Reconfiguration Complete message to target RAN. - Ax events (entry condition) are used for CHO execution condition and A3/5 as baseline. FFS: on other events |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR moiblity enhancement is missing in stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 ... CR TBD ... Others TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MU-MIMO	Multi User MIMO

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NR	NR Radio Access

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Cell-Defining SSB: an SSB with an RMSI associated.

Conditional Handover (CHO): A handover procedure that is executed only when the configured execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Xn: network interface between NG-RAN nodes.

- Next change

### 9.2.3	Mobility in RRC_CONNECTED

#### 9.2.3.1	Overview

Network controlled mobility applies to UEs in RRC_CONNECTED and is categorized into two types of mobility: cell level mobility and beam level mobility.

Cell Level Mobility requires explicit RRC signalling to be triggered, i.e. handover. For inter-gNB handover, the signalling procedures consist of at least the following elemental components illustrated in Figure 9.2.3.1-1:

Figure 9.2.3.1-1: Inter-gNB handover procedures

1.	The source gNB initiates handover and issues a Handover Request over the Xn interface.

2.	The target gNB performs admission control and provides the RRC configuration as part of the Handover Acknowledgement.

3.	The source gNB provides the RRC configuration to the UE in the Handover Command. The Handover Command message includes at least cell ID and all information required to access the target cell so that the UE can access the target cell without reading system information. For some cases, the information required for contention-based and contention-free random access can be included in the Handover Command message. The access information to the target cell may include beam specific information, if any.

4.	The UE moves the RRC connection to the target gNB and replies the Handover Complete.

NOTE:	User Data can also be sent in step 4 if the grant allows.

The handover mechanism triggered by RRC requires the UE at least to reset the MAC entity and re-establish RLC. RRC managed handovers with and without PDCP entity re-establishment are both supported. For DRBs using RLC AM mode, PDCP can either be re-established together with a security key change or initiate a data recovery procedure without a key change. For DRBs using RLC UM mode and for SRBs, PDCP can either be re-established together with a security key change or remain as it is without a key change.

Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed when the target gNB uses the same DRB configuration as the source gNB.

Timer based handover failure procedure is supported in NR. RRC connection re-establishment procedure is used for recovering from handover failure.

CHO is supported in NR, the details are specified in section 9.2.3.x.

Beam Level Mobility does not require explicit RRC signalling to be triggered. The gNB provides via RRC signalling the UE with measurement configuration containing configurations of SSB/CSI resources and resource sets, reports and trigger states for triggering channel and interference measurements and reports. Beam Level Mobility is then dealt with at lower layers by means of physical layer and MAC layer control signalling, and RRC is not required to know which beam is being used at a given point in time.

SSB-based Beam Level Mobility is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, Beam Level Mobility can only be performed based on CSI-RS.

#### 9.2.3.2	Handover

##### 9.2.3.2.1	C-Plane Handling

The intra-NR RAN handover performs the preparation and execution phase of the handover procedure performed without involvement of the 5GC, i.e. preparation messages are directly exchanged between the gNBs. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB. The figure below depicts the basic handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover

0.	The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the UE, based on MeasurementReport and RRM information.

3.	The source gNB issues a Handover Request message to the target gNB passing a transparent RRC container with necessary information to prepare the handover at the target side. The information includes at least the target cell ID, KgNB*, the C-RNTI of the UE in the source gNB, RRM-configuration including UE inactive time, basic AS-configuration including antenna Info and DL Carrier Frequency, the current QoS flow to DRB mapping rules applied to the UE, the SIB1 from source gNB, the UE capabilities for different RATs, PDU session related information, and can include the UE reported measurement information including beam-related information if available. The PDU session related information includes the slice information (if supported) and QoS flow level QoS profile(s).

NOTE:	After issuing a Handover Request, the source gNB should not reconfigure the UE, including performing Reflective QoS flow to DRB mapping.

4.	Admission Control may be performed by the target gNB. Slice-aware admission control shall be performed if the slice information is sent to the target gNB. If the PDU sessions are associated with non-supported slices the target gNB shall reject such PDU Sessions.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

6.	The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE, containing the information required to access the target cell: at least the target cell ID, the new C-RNTI, the target gNB security algorithm identifiers for the selected security algorithms. It can also include a set of dedicated RACH resources, the association between RACH resources and SSB(s), the association between RACH resources and UE-specific CSI-RS configuration(s), common RACH resources, and system information of the target cell, etc.

7.	The source gNB sends the SN STATUS TRANSFER message to the target gNB.

8.	The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

9.	The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

10.	5GC switches the DL data path towards the target gNB. The UPF sends one or more "end marker" packets on the old path to the source gNB per PDU session/tunnel and then can release any U-plane/TNL resources towards the source gNB.

11.	The AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

12.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

The RRM configuration can include both beam measurement information (for layer 3 mobility) associated to SSB(s) and CSI-RS(s) for the reported cell(s) if both types of measurements are available. Also, if CA is configured, the RRM configuration can include the list of best cells on each frequency for which measurement information is available. And the RRM measurement information can also include the beam measurement for the listed cells that belong to the target gNB.

The common RACH configuration for beams in the target cell is only associated to the SSB(s). The network can have dedicated RACH configurations associated to the SSB(s) and/or have dedicated RACH configurations associated to CSI-RS(s) within a cell. The target gNB can only include one of the following RACH configurations in the Handover Command to enable the UE to access the target cell:

i)	Common RACH configuration;

ii)	Common RACH configuration + Dedicated RACH configuration associated with SSB;

iii)	Common RACH configuration + Dedicated RACH configuration associated with CSI-RS.

The dedicated RACH configuration allocates RACH resource(s) together with a quality threshold to use them. When dedicated RACH resources are provided, they are prioritized by the UE and the UE shall not switch to contention-based RACH resources as long as the quality threshold of those dedicated resources is met. The order to access the dedicated RACH resources is up to UE implementation.

##### 9.2.3.2.2	U-Plane Handling

The U-plane handling during the Intra-NR-Access mobility activity for UEs in RRC_CONNECTED takes the following principles into account to avoid data loss during HO:

-	During HO preparation, U-plane tunnels can be established between the source gNB and the target gNB;

-	During HO execution, user data can be forwarded from the source gNB to the target gNB;

-	Forwarding should take place in order as long as packets are received at the source gNB from the UPF or the source gNB buffer has not been emptied.

-	During HO completion:

-	The target gNB sends a path switch request message to the AMF to inform that the UE has gained access and the AMF then triggers path switch related 5GC internal signalling and actual path switch of the source gNB to the target gNB in UPF;

-	The source gNB should continue forwarding data as long as packets are received at the source gNB from the UPF or the source gNB buffer has not been emptied.

For RLC-AM bearers:

-	For in-sequence delivery and duplication avoidance, PDCP SN is maintained on a per DRB basis and the source gNB informs the target gNB about the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet (either from source gNB or from the UPF).

-	For security synchronisation, HFN is also maintained and the source gNB provides to the target one reference HFN for the UL and one for the DL i.e. HFN and corresponding SN.

-	In both the UE and the target gNB, a window-based mechanism is used for duplication detection and reordering.

-	The occurrence of duplicates over the air interface in the target gNB is minimised by means of PDCP SN based reporting at the target gNB by the UE. In uplink, the reporting is optionally configured on a per DRB basis by the gNB and the UE should first start by transmitting those reports when granted resources are in the target gNB. In downlink, the gNB is free to decide when and for which bearers a report is sent and the UE does not wait for the report to resume uplink transmission.

-	The target gNB re-transmits and prioritizes all downlink data forwarded by the source gNB (i.e. the target gNB should first send all forwarded PDCP SDUs with PDCP SNs, then all forwarded downlink PDCP SDUs without SNs before sending new data from 5GC), excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the UE.

NOTE:	Lossless delivery when a QoS flow is mapped to a different DRB at handover, requires the old DRB to be configured in the target cell. For in-order delivery in the DL, the target gNB should first transmit the forwarded PDCP SDUs on the old DRB before transmitting new data from 5GCN on the new DRB. In the UL, the target gNB should not deliver data of the QoS flow from the new DRB to 5GCN before receiving the end marker on the old DRB from the UE.

-	The UE re-transmits in the target gNB all uplink PDCP SDUs starting from the oldest PDCP SDU that has not been acknowledged at RLC in the source, excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the target.

For RLC-UM bearers:

-	The PDCP SN and HFN are reset in the target gNB;

-	No PDCP SDUs are retransmitted in the target gNB;

-	The target gNB prioritises all downlink SDAP SDUs forwarded by the source gNB over the data from the core network;

NOTE:	To minimise losses when a QoS flow is mapped to a different DRB at handover, the old DRB needs to be configured in the target cell. For in-order delivery in the DL, the target gNB should first transmit the forwarded PDCP SDUs on the old DRB before transmitting new data from 5GCN on the new DRB. In the UL, the target gNB should not deliver data of the QoS flow from the new DRB to 5GCN before receiving the end marker on the old DRB from the UE.

-	The UE does not retransmit any PDCP SDU in the target cell for which transmission had been completed in the source cell.

##### 9.2.3.2.3	Data Forwarding

The following description depicts the data forwarding principles for intra-system handover.

The source NG-RAN node may suggest downlink data forwarding per QoS flow established for a PDU session and may provide information how it maps QoS flows to DRBs. The target NG-RAN node decides data forwarding per QoS flow established for a PDU Session.

If "lossless handover" is required and the QoS flows to DRB mapping applied at the target NG-RAN node allows applying for data forwarding the same QoS flows to DRB mapping as applied at the source NG-RAN node for a DRB and if all QoS flows mapped to that DRB are accepted for data forwarding, the target NG-RAN node establishes a downlink forwarding tunnel for that DRB.

For a DRB for which preservation of SN status applies, the target NG-RAN node may decide to establish an UL data forwarding tunnel.

The target NG-RAN node may also decide to establish a downlink forwarding tunnel for each PDU session. In this case the target NG-RAN node provides information for which QoS flows data forwarding has been accepted and corresponding UP TNL information for data forwarding tunnels to be established between the source NG-RAN node and the target NG-RAN node.

As long as data forwarding of DL user data packets takes place, the source NG-RAN node shall forward user data in the same forwarding tunnel, i.e.

-	for any QoS flow accepted for data forwarding by the target NG-RAN node and for which a DRB DL forwarding tunnel was established for a DRB to which this QoS flow was mapped at the source NG-RAN node, any fresh packets of this QoS flow shall be forwarded as PDCP SDUs via the mapped DRB DL forwarding tunnel.

-	for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE.

NOTE:	The SN of forwarded PDCP SDUs is carried in the "PDCP PDU number" field of the GTP-U extension header.

-	for any QoS flow accepted for data forwarding by the target NG-RAN node for which a DL PDU session forwarding tunnel was established, the source NG-RAN node forwards SDAP SDUs as received on NG-U from the UPF.

As long as data forwarding of UL user data packets takes place for DRBs for which preservation of SN status applies the source NG-RAN node either:

-	discards the uplink PDCP PDUs received out of sequence if the source NG-RAN node has not accepted the request from the target NG-RAN node for uplink forwarding or if the target NG-RAN node has not requested uplink forwarding for the bearer during the Handover Preparation procedure; or

-	forwards to the target NG-RAN node the uplink PDCP SDUs with their SN corresponding to PDCP PDUs received out of sequence if the source NG-RAN node has accepted the request from the target NG-RAN node for uplink forwarding for the bearer during the Handover Preparation procedure.

Handling of end marker packets:

-	The source NG-RAN node receives one or several GTP-U end marker packets per PDU session from the UPF and replicates the end marker packets into each data forwarding tunnel when no more user data packets are to be forwarded over that tunnel.

-	End marker packets sent via a data forwarding tunnel are applicable to all QoS flows forwarded via that tunnel. After end marker packets have been received over a forwarding tunnel, the target NG-RAN node can start taking into account the packets of QoS flows associated with that forwarding tunnel received at the target NG-RAN node from the NG-U PDU session tunnel.

#### 9.2.3.X	CHO

##### 9.2.3.x.1	General

Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. UE starts evaluating the execution condition upon receiving the CHO configuration, and stops evaluating the execution condition during the CHO execution once the execution condition(s) is met.

The following principles apply to CHO:

-	The CHO command contains the configuration of CHO candidate cell(s) generated by candidate gNB (s) and execution condition(s) generated by source gNB.

-	An execution condition consists of Ax (A3/5) measurement events. Only single RS type is supported and at most two different execution quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evalution of CHO execution condition of a single candidate cell.

-	Reception of HO command (without conditional component), as described in clause 9.2.3.2 overrides any configured HO command for CHO.

While executing CHO, UE will not monitor source cell.

N2 based CHO is not supported in this release.

##### 9.2.3.2a.1	C-plane handling

As in intra-NR RAN handover, in intra-NR RAN CHO, the preparation and execution phase of the conditional handover procedure is performed without involvement of the 5GC; i.e., preparation messages are directly exchanged between gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target gNB. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.2.1-1a: Intra-AMF/UPF Conditional Handover

0/1.	Same as step 0, 1 in Figure 9.2.3.2.1-1 of section 9.2.3.2.1.

2.	The source gNB decides to use CHO.

3.	The source gNB issues a CHO Request message to  one or more candidate gNBs.

Editor’s note: FFS on the details of CHO Request message, and whether multiple candidate gNBs are allowed. RAN3 scope.

4.	Same as step 4 in Figure 9.2.3.2.1-1 of section 9.2.3.2.1.

5.	The candidate gNB sends CHO response including configuration of CHO candidate cell to the source gNB.

Editor’s note: FFS on the details of CHO response message. RAN3 scope.

6.	The source gNB sends a RRCReconfiguration message to the UE, containing CHO the configuration of CHO candidate cell(s) and CHO execution condition(s).

7.	UE sends an RRCReconfigurationComplete message to the source gNB.

8.	UE maintains connection with source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell and synchronises to that candidate cell.

9.	Same as step 8 in Figure 9.2.3.2.1-1 of section 9.2.3.2.1.

Editor’s note: FFS how to perform data forwarding, RAN3 scope.

Editor’s note: FFS whether the procedure can be combined with ‘simultaneous connectivity handover.

##### 9.2.3.2a.2	U-plane handling

Editor’s note: FFS based on RAN3 decisions

#### 9.2.3.3	Re-establishment procedure

A UE in RRC_CONNECTED may initiate the re-establishment procedure to continue the RRC connection when a failure condition occurs (e.g. radio link failure, reconfiguration failure, integrity check failure…).

The following figure describes the re-establishment procedure started by the UE:

Figure 9.2.3.3-1: Re-establishment procedure

1.	The UE re-establishes the connection, providing the UE Identity (PCI+C-RNTI) to the gNB where the trigger for the re-establishment occurred.

2.	If the UE Context is not locally available, the gNB, requests the last serving gNB to provide UE Context data.

3.	The last serving gNB provides UE context data.

4/4a. The gNB continues the re-establishment of the RRC connection. The message is sent on SRB1.

5/5a. The gNB may perform the reconfiguration to re-establish SRB2 and DRBs when the re-establishment procedure is ongoing.

6.	If loss of DL user data buffered in the last serving gNB shall be prevented, the gNB provides forwarding addresses.

7/8. The gNB performs path switch.

9.	The gNB triggers the release of the UE resources at the last serving gNB.

End Text Proposal Change

# RAN2 agreements on NR mobility enhancement

## General:

### RAN2#105

Agreements

1	Solution proposals should consider at least the following evaluation criteria:

- Mobility robustness

- Interruption time

2	Other criteria to be considered are:

- Applicable deployment scenarios

- Signalling overhead

- Specification effort

- UE/network complexity

Agreements

1	Rely on chairman’s notes to capture agreements in the initial phase of the work item. Rapporteur will start a running stage-2 CR when work item phase starts.

Agreements

1	The UE ability to simultaneously receive and transmit to/from the source and target cells is to be considered in the study on NR mobility enhancements.

2	We prioritize on intra-NR handovers in this WID.

## Reduction in user data interruption during handover or SCG change:

### RAN2#107:

Agreements

1	Introduce a solution for HO interruption time reduction based on dual active protocol stack.

Agreements

1	We will not work on RACHless HO any further in Rel16 (Can be revisited if CFRA is not agreed to be part of 2 Step RACH in Rel-16)

### RAN2#106:

Agreements

1:	Mobility interruption time means the shortest time duration supported by the system during which a user terminal is not able to exchange user plane packets with any base station during transitions.

2:	RAN2 common understanding is to reduce interruption time at radio (i.e. air interface) level during mobility (i.e. handover) to improve user experience at service/application layer.

3: 	RAN2 aim to develop protocol design to achieve strict 0ms (if feasible) else close to 0ms interruption time on radio level during handover considering UE capabilities and deployment scenarios.

4: 	For achieving the aim of agreement 3, RAN2 targets a single solution

5: 	Interruption time reduction in DL to be prioritized, but UL will still be considered.

Agreements

1	PDCP packet duplication does not need to be supported in combination with the HO interruption solution (but doesn't preclude that it might be possible to support it and it may be beneficial in some cases)

2	Simultaneous UL PUSCH transmission does not need to be supported for the HO interruption solution.

3	There is a point in time where the UL PUSCH switches from source to target.

### RAN2#105bis:

Agreements

1	The solutions to be introduced for handover interruption time reduction will only address cases where UE is able to receive simultaneously from source and target cells (both within FR1). (This is based on the assumption that RAN1/4 indicate that simultaneous rx is available in the majority of FR1 deployment scenarios)

2	We will identify the key aspects of the solutions that are common and that are different. The aspects that are different can then be considered in the decision process.

3	We will define an interruption time definition that we can use in our evaluation of different solutions (starting point is to use one of the definitions that is already available in 3GPP, e.g. 38.913, RAN4, etc).

### RAN2#105:

=> 	We will consider DC-based solutions in study phase. Proponents are encouraged to come up with joint solutions and evaluation using the agreed criteria.

=>	We will consider non-DC-based solutions in study phase. Proponents are encouraged to come up with joint solutions and evaluation using the agreed criteria.

## Handover robustness improvements:

### RAN2#107:

Agreements

1	For the scenario of multiple CHO cells being triggered the cell selected by the UE considering beams and beam quality. We will not specify normative requirements for the selection process but can be captured in an informative note in stage 3 spec.

2	No additional optimizations are introduced to improve RACH performance for CHO completion with multi-beam operation.

Agreements

1  For FR1, we will leave it up to UE implementation to select the target cell if more than one candidate cell meets the triggering condition (same as for FR2).

2  Do not introduce “bye” message from UE to the source cell for CHO.

3  If UE receives conventional handover command, it will execute the handover command regardless of stored (configured) conditional handover command. This applies if the HO cmd is received before any CHO triggering condition is satisfied. FFS how HO failure is handled.

4 	The UE can’t receive and perform RRC configuration from source cell while executing CHO command (which means from the time when the UE starts synchronization with target cell).

FFS whether simultaneous connectivity and CHO can work simultaneously.

5	UE is not required to continue evaluating the triggering condition of other candidate cell(s) during CHO execution.

6	We will not change cell selection procedure due to CHO (T310 expiry, T304(-like) expiry, etc.)

7	CHO is optional feature for UEs and networks.

Agreements

1	As part of CHO configuration to be sent to the UE, RRC container is used to carry target cell configuration and source cell is not allowed to alter any content of configuration from the target cell.

2	Use add/mod list + release list to configure multiple CHO candidate cells. CHO execution condition can be updated by modifying the existing CHO configuration, Target cell configuration can be updated by modifying the existing CHO configuration.

3	Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CHO configuration to UE.

4	A RRC complete message is required for UE to confirm receipt and proper comprehension of CHO configuration (execution condition, FFS target cell configuration) to the source eNB/gNB.

FFS whether the UE is required to check the compliance of the target cell configuration within CHO configuration upon reception or whether it is allowed to check upon execution.

FFS whether different RRC processing requirements are defined for the reconfiguration with CHO command.

5	After CHO configuration has been sent to the UE, source configuration can be updated.

FFS whether CHO commands need to be updated after source reconfiguration.

6	Delta configuration for CHO commands is based on latest source configuration

7	Allow having multiple triggering conditions (using “and”) for CHO execution of a single candidate cell. Only single RS type per CHO candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultnaeously. FFS on UE capability.

8	TTT is supported for CHO condition (as per legacy configuration)

Agreements

- S1/N2-based CHO is not supported for Release 16. RAN3 impacst to be discussed in RAN3. If we want to support this, the WID has to be updated and SA2 impacts identified.

Agreements

1: 	Support conditional NR PSCell addition/change and reusing the conditional HO solution being developed. Supported for any architecture option with NR PSCell.

2	From RAN2 perspective conditional NR PSCell change can be supported for both intra-SN and inter-SN

### RAN2#106:

Agreements

2	The source cell decides on the condition for the execution of CHO.

3	The source cell adds the condition for the execution of CHO to the RRC message sent to UE.

4	Multiple CHO candidate cells can be sent in either one or multiple RRC messages. FFS on signalling details. FFS how CHO execution is handled.

5	CHO execution does not trigger measurement report.

6	On cell level A3/A5-like CHO execution condition shall be specified (other events will not be specified without clear justifications)

Agreements

1:	Separate CHO execution condition(s) can be configured for each individual candidate cells.

2	Define a CHO execution condition by the measurement identity which identifies a measurement configuration. (FFS to be addressed in stage 3 which parts of the measurement configuration are used for the CHO triggering)

3	As a baseline CHO can be triggered based on a condition consisting of a single event, single RS type, singe quantity.

3.1	The single trigger quantity can be configured to be RSRP, RSRQ or RS-SINR

3.2	The single RS type can be configured to be SSB or CSI-RS

FFS Whether multiple triggering conditions are required.

Agreements

1	Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates)

2	Baseline that configuration of all CHO candidates are released after successful (any) handover completion (sending complete message to the target cell).

FFS if it might be possible to keep CHO candidates after the HO.

Agreements

1	UE shall not stop T310 and shall not start T304 when it receives configuration of a CHO candidate

2.	The timer T310 is stopped and timer T304-like is started when the UE begins execution of a conditional handover for a target cell. (Stage 3 detail whether we reuse T304 or define a new timer)

Working assumption (to be confirmed next meeting after checking further details)

3	At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed

4	At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed

### RAN2#105bis

0:	CHO is introduced in NR to solve robustness/reliability issue.

1:The LTE agreements below are applicable for NR:

a/ CHO is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

b/ Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

c/ Support configuration of one or more candidate cells for conditional handover;

=>	FFS how many candidate cells (UE and network impacts should be clarified).

=>	FFS how to include the CHO conditions in UE configuration

d/ The baseline operation for Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

e/ The baseline operation for Conditional HO assumes the source RAN remains responsible for RRC until UE successfully sends RRC Reconfiguration Complete message to target RAN.

f/ 	RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be suitable for CHO when there are multiple candidate target cells. Early packet forwarding can also be considered. Detailed decisions require RAN3 study.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

4	Ax events (entry condition) are used for CHO execution condition and A3/5 as baseline

FFS: on other events

5	Trigger quantity for CHO execution condition(RSRP, RSRQ or RS-SINR) is configured by network.

FFS: on multiple quantities.

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

### RAN2#105

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

2	We should consider how solutions work in FR2.

## Fast handover failure recovery

### RAN2#107

Agreements

1	Support T312 mechanism (similar to LTE). At least applicable for PCell

FFS Whether this is applicable for PSCell.