3GPP TSG RAN WG2 Meeting #107 	R2-1911748

Prague, CZ, 26-30 Aug 2019																	rev of R2-1909053

Title:	Revision to Non-Terrestrial Networks reference scenarios parameters

Source: 	Thales, HNS, Inmarsat

Type:	pCR

Document for:	Agreement

Agenda Item:	11.6.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at revising the TR 38.821

- the Table 4.2-2: Reference scenario parameters of chapter 4.2

- The Table 7.1-1: NTN scenarios versus delay constraints, Source [2] of chapter 7.1.1

# Discussion

In this paper we propose to revise the following parameters of the reference scenarios currently studied:

- Max channel bandwidth (service link): is currently set to 400 MHz for band > 6 GHz while the satellite communication industry suggests setting it to 1 GHz based on state of the art and in orbit satelite resources

- User equipment motion on the earth: The max speed is set to 1000 km/h (e.g. aircraft). Actually, the max aircraft should be increased to 1200 km/h to cover all commercial aircrafts

- User equipment Tx power : It is defined as up to 4 W for terminal with Directive antenna. Actually, this limit should be increased to 20 W to cover aircraft mounted devices

- Beam diameter should be replaced by beam size because the beam footprint shape may be distorted with elevation. Hence beam size corresponds to the max distance from edge to edge in a beam

- Maximum differential delay is revisited to take into account larger beams for GEO and LEO and their projected foot print at low elevation

- It shall be clarified that “Max channel bandwidth (service link)” refers to a satellite transponder capability.

Furthermore, it should be clarified that in table 7.1-1, that the Round Trip Delay below refers actually to the propagation delay contribution to the RTD

- Maximum Round Trip Delay on the radio interface between the gNB and the UE:

- Minimum Round Trip Delay on the radio interface between the gNB and the UE:

# Conclusion

Proposal 1: Reflect the above proposals in the TR 38.821 (See TP below)

# Text Proposal for TR 38.821 v0.7.0

START OF CHANGES

## 4.2	Non-Terrestrial Networks reference scenarios

…

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A : Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) - 25.77 ms (600km) - 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) - 12.89 ms (600km) - 20.89 ms (1200km) |
| Max delay variation within a beam (earth fixed user equipment) | 16ms | 4.44ms (600km) 6.44ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 msand 3.18 ms for respectively 600km and 1200km altitude |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print size

NOTE 4:	Speed of light used for delay calculation is 299792458 m/s.

NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation).

NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It doesn’t preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above.

END OF CHANGES

START OF CHANGES

### 7.1.1 	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| Maximum delay difference within a NTN beam as seen by the UE (note 3) | 16 ms (Worst case) |  | 4.44 ms |  |  |  |
| Max rate of hand-over (FFS) |  |  |  |  |  |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: The delay difference compares the delay (function of UE-satellite-NTN gateway distance) experienced by two different UEs served by the same beam at a given time NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

END OF CHANGES