3GPP TSG RAN WG2 #107-Bis	R2-1912421

Chongqing, China; Oct 14 – Oct 18, 2019

Agenda Item:	7.6

Source:	Polaris Wireless

Title:	Sensor Provide Location Information Element Correction

Document for:	Discussion, Decision

# Introduction

Support for barometric pressure based altitude determination was introduced in LPP (36.355) [1] in Rel-13 and updated in Rel-14 (addition of barometric pressure assistance data to support UE-Based barometric altitude determination) and Rel-15 (barometric pressure assistance data enhancements).

Reporting of measured barometric pressure by the UE to the Location Server (E-SMLC) as part of LPP Provide Location Information is currently limited to uncompensated barometric pressure i.e. the raw pressure measurement of the UE’s barometric pressure sensor.

# Discussion

In order for a Location Server to determine the altitude of a UE based on the UE’s reported barometric pressure sensor measurement, the Location Server must know not only the “uncompensated” barometric pressure, but also the UE’s barometric pressure sensor’s bias. The bias in this case is the difference between the measured and reported raw sensor pressure and the actual “true” pressure. Unfortunately, the current version of LPP only allows reporting of the uncompensated barometric pressure:

This deficiency was corrected in LPPe:

This contribution proposes to add an IE representing the UE’s barometric pressure sensor’s bias to Sensor-MeasurementInformation (part of LPP Provide Location Information).

Observation 1: 	Currently LPP only supports reporting of the UE’s barometric pressure sensor’s uncompensated measurements (i.e. the raw barometric pressure sensor reading) but not reporting of the UE’s barometric pressure sensor’s bias.

Observation 2:	For accurate altitude determination based on measured barometric pressure at the UE, a Location Server not only needs to know the uncompensated barometric pressure as measured by the UE’s barometric pressure sensor, but also any sensor internal bias (the difference between the sensor’s pressure measurement and the true actual pressure).

Observation 3:	LPPe has corrected this deficiency and introduced an “adjustment” parameter representing the UE’s barometric pressure sensor’s bias.

Observation 4:	Only very minor changes (adding one single IE) are required to LPP in order to enable the UE to report its barometric pressure sensor’s bias to the Location Server.

# Changes required in LPP

The barometric pressure measurement IE is defined in Rel-15 of 36.355 as:

To enable reporting of barometric pressure sensor bias, a single IE representing the sensor bias needs to be added to Sensor-MeasurementInformation.

Observation 5:	Adding one single IE representing the barometric pressure sensor bias to Sensor-MeasurementInformation is required to enable the UE to report its barometric pressure sensor’s internal bias to the Location Server.

It is recommended to adopt the IE naming and encoding from LPPe:

# Conclusion

In order for a Location Server to determine the altitude of a UE based on the UE’s reported barometric pressure sensor measurements, the Location Server must know not only the “uncompensated” barometric pressure, but also the UE’s barometric pressure sensor’s bias.

While currently not supported in LPP, this can be achieved by simply adding a single IE representing the UE’s barometric pressure sensor’s bias to Sensor-MeasurementInformation.

It is recommended that RAN2 consider this proposal for approval. The proposed CR against LPP (36.355) is provided as a separate contribution.

Proposal 1:	Agree to add IE representing the UE’s barometric pressure sensor’s bias to Sensor-MeasurementInformation in LPP.

# References

TS 36.355: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)”