3GPP TSG-RAN WG2 Meeting #107bis	R2-1913042

Chongqing, China, 14-18 Oct., 2019

Agenda item:	6.21

Source: 	Huawei, HiSilicon

Title: 	Further discussions on on-demand SI in RRC_CONECTED

Document for:	Discussion and Decision

Introduction

In the last RAN2 meeting, the following agreement on on-demand SI in RRC_CONNECTED state were made.

Agreements: 1 On demand SI procedure in RRC_Connected is supported in Rel-16 (for potential use by any WI) 2 Define new UL-DCCH message for SI request in RRC_CONNECTED. FFS Whether the request is per SIB or per SI (we can consider whether different WIs have specific requirements) 3 UE in RRC_CONNECTED does not use existing SI request procedure based on RACH. 4 If common search space to receive the system information is configured on the active BWP, the UE tries to receive the on-demand SI through broadcast after transmitting the SI request as in RRC_IDLE/INACTIVE. FFS Whether the UE has feedback from the request to know whether the SI will be sent via broadcast before it tries to receive the SI. 5 If common search space to receive the system information is configured on the active BWP, the UE in RRC_CONNECTED checks whether the required on-demand SI is being broadcasted by reading SIB1 before transmitting the SI request, and transmits the SI request only when the required on-demand SI is not being broadcasted, as in RRC_IDLE/INACTIVE.

The two FFS has been covered by E-mail discussion [107#42] and we have shown our opinion on them in e-mail discussion. In this contribution, we focus on SI update in RRC_CONNECTED.

Discussion

Remaining issues for SI reception from the last meeting

According to the agreement of the last meeting, in RRC_CONNECTED

If common search space to receive the system information is configured on the active BWP, the UE tries to receive the on-demand SI through broadcast after transmitting the SI request as in RRC_IDLE/INACTIVE.

Another aspect not handled by this agreement is the UE behaviour when the active DL BWP is not configured with CSS. This agreement implies that the UE receive the on-demand SI by dedicated RRC signalling if there is no CSS configuration to receive system information on the active BWP. We kindly ask RAN2 to confirm this.

Proposal 1: RRC_CONNECTED UE receives the requested SI by dedicated RRC signalling if there is no CSS configuration to receive system information.

SI update in RRC_CONNECTED

In Rel-15, NR specifies UE how to obtain updated SIB1 and SI message in RRC_CONNECTED state. UE in RRC_CONNECTED could obtain updated SIB1 by receiving broadcast if its active DL BWP is configured with common search space searchSpaceSIB1 and pagingSearchSpace and it has received an indication about change of system information, or by receiving dedicated signalling from network. For OSI, UE is in RRC_CONNECTED could obtain updated SI message by receiving broadcast if its active DL BWP is configured with common search space searchSpaceOtherSystemInformation and pagingSearchSpace and SIB1 indicates this SI change, or by receiving dedicated signalling from network (only supporting SIB6-SIB8).

Observation 1: Rel-15 has two mechanism to support SI update in RRC_CONNECTED: 1) system information change notification in paging if there is CSS configuration in the DL active BWP; 2) Network transmits updated system information directly to UE by dedicated signalling.

In the following, we discuss SI update in RRC_CONNECTED in Rel-16 by considering two scenarios with distinguishing whether there is CSS configuration to receive paging and system information on the active BWP or not.

SI update with CSS configuration

The first scenario is that there is CSS configuration in the active DL BWP for the UE. In this scenario, CSS configuration for paging, SIB1 and OSI is configured in the active DL BWP. When SIB1 or OSI changes, the network would transmit SI change notification by paging, and UE could receive the updated SIB1 and updated OSI by receiving SI broadcasting. Therefore, the current SI update mechanism could work for scenario. Therefore, it is proposed that the current SI update mechanism could be reused if there is CSS configuration in the DL active BWP

Proposal 2: If there is CSS configuration in the active DL BWP, the SI update mechanism, in which UE obtains updated SIB1 and updated SI by receiving SI broadcast after receiving SI change notification in paging, could be reused to handle SIB1 and OSI change.

SI update without CSS configuration

Another important scenario is the case when the CSS is not configured. Since on the PCell of the UE, multiple DL BWPs can be configured to UE but common search space of paging and system information may not be configured in its active DL BWP. In this scenario, UE only can obtain updated SI message by dedicated signalling due to no CSS configuration. SI update can be discussed in the two cases of SIB1 change and OSI change. In this section, we discuss this two cases, respectively.

For the first case of SIB1 change, if the content of SIB1 changes (not including the case of valueTag change caused by the change of OSI content), the network could transmit updated SIB1 to all connected UEs by dedicated RRC signalling, i.e., the current SI update mechanism could work for SIB1 update. Hence, the mechanism of transmitting updated SIB1 by dedicated signalling could reused in Rel-16 to handle SIB1 change.

Proposal 3: For SIB1 update, if there is no CSS configuration in the active DL BWP, mechanism of transmitting updated SIB1 by dedicated signalling could reused.

For the second case of OSI change, we take SIBx as an example to analyse the change of OSI with considering two cases:

- Case 1: UE has stored SIBx before entering RRC_CONNECTED state;

- Case 2: UE has stored SIBx by on-demand SI in RRC_CONNECTED state.

Both in case 1 and case 2, UE will not know whether its stored SIBx has changed or not. Although the network could transmit updated SIBx to some UEs who ever requested SIBx by network implementation in case 2, network doesn’t know when to stop this. Actually, UE may just need SIBx for a certain time. Furthermore, the network transmitting updated SIBx to UE may leads to that UE stored valueTag of SIBx doesn’t align with valueTag of SIBx in networkd side.

Observation 2: UE will not know whether its stored OSI has changed or not if there is no CSS configuration in the current active BWP and network implementation could not solve it.

To deal with this issue, we discuss the OSI change notification and updated OSI delivery, respectively.

OSI change notification

A OSI change notification method was proposed in the last meeting [1]. This method delivers updated SIB1 to all connected UEs by dedicatedSIB1-Delivery every time SI change to notify UE SI change by valueTag of OSI in SIB1. After receiving updated SIB1, UE could know which SIBs change according to the valueTags in SIB1. This method could work without large specification modification.

However, considering that the size of SIB1 may be up to 3000bits and the number of SIBs will be large in Rel-16, it is very resource-consuming to transmit entire SIB1 to all connected UEs every time SI change. A possible alternative is to transmit updated SIB1 to UEs only when the content of SIB1 changes and just transmit a valueTag list of OSI to UEs when the content of OSI changes. After receiving valueTag list, UE could know which SIBs change. In this way, the resources could be saved.

Based on the above discussion, the SI change notification mechanism can be summarized as follows:

- Method 1: delivers updated SIB1 to all connected UEs by dedicatedSIB1-Delivery every time SI change.

- Method 2: transmit a valueTag list of OSI to all connected UEs when the content of OSI changes.

Table 1 summaries the advantages and disadvantages of the three methods.

Table 1 Analysis of OSI change notification methods when CSS is not configured

|  | Resource efficiency | Specification impact |
| --- | --- | --- |
| Method 1 | - Transmit entire SIB1 to all connected UE every time SI change - Transmit updated OSI to UE after UE request | - No ASN.1 impact - Some procedure impact |
| Method 2 | - Transmit just the valueTag list of OSI to all connected UEs when the content of OSI changes - Transmit updated OSI to UE after UE request | - Define a message or reuse other message to carry valueTag list - Some procedure impact |

Comparing these two methods, we think method 2 is a better option. Hence, we propose the following:

Proposal 4: For SI change notification for OSI, if there is no CSS configuration under the active DL BWP, a RRC message including the valueTag information for the OSI is sent to the UE.

Updated OSI delivery mechanism

By the two methods, UE could find whether interested OSI has changed and update it by transmitting SI request via dedicated signalling, which we have agreed during the last meeting. Then, the network delivery updated OSI to UE by dedicated signally as a response. The next question is how the updated OSI is delivered.

One approach (Method I) is that whenever SI changes, and network sends the SIB change indication to the UE. Then, with the SI change indication, the UE request its interested SI and the network delivers the SI to the UE via dedicated signalling. And after this, whenever there is a change in SI, this interchange of signalling needs to be performed again. This approach is illustrated in the following figure 1.

Figure 1, SI delivery method I

In fact, after receiving SI request from UE, gNB could know which SIBs those UE concern. Once those SIBs change again, the network could delivery updated SIBs to those UE directly without transmitting updated SIB1 or valueTag list again. By this, the signalling of transmitting SIB1 or valueTag list to the UE and UE transmitting SI request to gNB could be saved.

In the broadcast of positioning system information in LTE, valueTag is added to each positioning SIB such that when receiving the SIB, the UE can know what the current version of SIB is. When the network transmit updated OSI to UE, the new valueTags also should be delivered to UE together with corresponding SIBs to ensure the alignment of valueTag of a SIB between network and UE. Furthermore, considering UE may just need some OSI for a certain time, it will be very wasting to transmit updated OSI to UE always even UE hasn’t needed them. So, a SI stop indication could be used to tell network which SIBs it does not need any more. This could be called subscribing/unsubscribing SIB mechanism, as shown in Fig.2. This mechanism has higher resource efficiency the current mechanism of delivering updated OSI.

Figure 2, OSI delivery Method II

With the above discussion, we make the following proposal:

Proposal 5: After knowing which SIBs UE concerns, network could transmit updated SIBs and new valueTags to UE directly without transmitting OSI change notification.

Proposal 6: A SI stop indication message should be sent to the network when the UE no long needs some SIBs anymore after the SI request.

Conclusion

In this contribution, we further discussed on-demand SI in RRC_CONNECTED, and we get the following observations proposals:

Observations:

Observation 1: Rel-15 has two mechanism to support SI update in RRC_CONNECTED: 1) system information change notification in paging if there is CSS configuration in the DL active BWP; 2) Network transmits updated system information directly to UE by dedicated signalling.

Observation 2: UE will not know whether its stored OSI has changed or not if there is no CSS configuration in the current active BWP and network implementation could not solve it.

Observation 3: after knowing which SIBs change by OSI change notification, UE could obtain updated OSI by requesting them via dedicated signalling.

Proposals:

Proposal 1: RRC_CONNECTED UE receives the on-demand SI by dedicated RRC signalling if there is no CSS configuration to receive system information.

Proposal 2: if there is CSS configuration in the active DL BWP, the SI update mechanism, in which UE obtains updated SIB1 and updated SI by receiving SI broadcast after receiving SI change notification in paging, could be reused to handle SIB1 and OSI change.

Proposal 3: For SIB1 update, if there is no CSS configuration in the active DL BWP, mechanism of transmitting updated SIB1 by dedicated signalling could reused.

Proposal 4: For SI change notification for OSI, if there is no CSS configuration under the active DL BWP, a RRC message including the valueTag information for the OSI is sent to the UE.

Proposal 5: After knowing which SIBs UE concerns, network could transmit updated SIBs and new valueTags to UE directly without transmitting OSI change notification.

Proposal 6: A SI stop indication message should be sent to the network when the UE no long needs some SIBs anymore after the SI request.

References

[1] R2-1911408, “Request and reception of on-demand SI in RRC CONNECTED”, RAN2#107, LG Electronics Inc.