3GPP TSG-RAN2 Meeting #107bis	R2-1913285

# Chongqing, China, 14th - 18th Oct, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38331 | CR | 1299 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on delta signalling for handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the LTE spec, when PUCCH and SRS resources are released, the UE would apply the default congfiguration for PUCCH and SRS as the basis for delta signalling during handover. While at handover failure, the UE would revert to the default PUCCH and SRS configuration instead of the configuration in source SpCell, which has already been released at handover intiation due to MAC reset and TA handling. However, the current text is not clear on this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Clarify that at reconfig with sync failure, the UE should revert to the default configuration for CSI/SR/SRS instead of the configuration in the source SpCell 2/ Clarify that the UE should apply the default configuration for SRS/CSI/SR at PUCCH and SRS release. 3/ Editorial correction : change PCell to SpCell Impact analysis: Impacted 5G architecture options: SA, (NG)-ENDC, NE-DC, NR-DC Impacted functionality: RRC reconfiguration Inter-operability analysis: 1/ If the network implements according to the CR while the UE is not, the network and UE will have different understanding on the basis for delta signalling (if any) for RRC re-establishment. 2/ if the UE implements according to the CR while the network is not, the UE behavior is undefined and network will not know what is the UE behavior in this case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot revert to the previous CSI/SR/SRS configuration because it is already released and the UE behavior is undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.8.3, 5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================FIRST CHANGE====================================

5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	revert back to the UE configuration used in the source SpCell;

NOTE 1:	The UE should revert back to the default configuration of CSI/SR/SRS at reconfiguration with sync failure. For the other radio resource configurations, the UE reverts back to the configuration used in the source SpCell

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

========================SECOND OF CHANGE==========================================

5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

1>	apply the default physical channel configuration for SchedulingRequestResourceConfig as specified in 9.2;

1>	apply the default physical channel configuration for CSI-ReportConfig as specified in 9.2;

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config;

1>	apply the default physical channel configuration for SRS-Config as specified in 9.2.

=======================END OF CHANGES=============================================