3GPP TSG-RAN2 Meeting # 107b	R2-193986

# Chongqing, China, 14th – 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0772 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using t4is form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the handling of stored AS context upon CN type change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2019-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RRC Suspend/Resume, in current TS 36.304, the following use case is defined for UE discarding the stored AS context information (see subclause 5.2.4.11): - Inter-RAT cell reselection (from LTE to NR) in RRC_IDLE However, for the following use case, there is currently no corresponding requirement: - The UE is in RRL_IDLE and AS context is stored due to RRC Suspend, and there is a CN type change from EPC to 5GC In this case the UE should also discard the stored AS context information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.2.4.11 describes UE behaviours for cell reselection when stroing UE AS context, so this section is extended to apply to the following use case: - The UE is in RRL_IDLE and AS context is stored due to RRC Suspend, and there is a CN type change from EPC to 5GC Impact analysis Impacted functionality: LTE connected to 5GC Inter-operability: There is no inter-operability issue as this CR has no network impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not discard the stored AS context upon CN type change, resulting in potential RRC Resume failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.4.11	Cell reselection or CN type change when storing UE AS context

For UEs storing UE AS context and resumeIdentity as specified in TS 36.331 [3], upon cell reselection to another RAT or upon reselecting to another CN type, the UE shall discard the stored UE AS context and resumeIdentity.