3GPP TSG-RAN WG2#108                                	 R2-1914372

Reno, Nevada, USA, 18th – 22th Nov, 2019	                               Revision of R2-1912184

Source:	vivo

Title:	Further Discussion on the Whitelist in NR-U

Agenda item:	6.2.3.2

Document for:	Discussion and Decision

1	Introduction

The paper is the revision of R2-1912184. The proposals which are captured in the running CR[1] are removed.

In the RAN2#107 meeting, it was agreed that whitelist is supported.

We will support whitelist

Whitelist is helpful for UE to perform cell selection and re-selection on unlicensed carriers. In this paper, we discuss the details of introducing the whitelist for NR-U.

2 Discussion

2.1 Whitelist in NR system information

Whitelist is already included in NR SIB3 and SIB4, according to the endorsed running CR[1]. However, the maximum number of whitelisted cells per frequency is still FFS. In NR, the maximum number of blacklisted cells on each frequency is 16.

We believe the same maximum number can be adopted for whitelisted cells.

Proposal 1: The maximum number of whitelisted cells on each frequency is 16.

2.2 Whitelist in RRCRelease

After the reception of RRCRelease message, one RRC_CONNECTED UE needs to go to RRC_IDLE or RRC_INACTIVE and perform cell selection. According to current mechanism, RRC_CONNECTED UE is not required to acquire the SIB3/4. Hence, it is very likely that UE has no valid whitelist while leaving the RRC_CONNECTED state. It means UE has to perform the cell selection afterwards without whitelist, which may result in long cell search delay for the UE. To solve this issue, it is proposed that whitelist is also included in the RRCRelease message.

Observation1: Include whitelist in the RRCRelease message is helpful for UE to perform cell selection after leaving RRC_CONNECTED state

Proposal 2:  Whitelist can be included in the RRCRelease message.

2.3 Whitelist in RAN sharing scenario

There may be cells belonging to different PLMNs on one unlicensed frequency. As illustrated in the Figure 1, cell1, cell2 and cell3 are deployed on the same frequency. Cell1 belong to PLMN1 and cell2 belongs to PLMN2. Cell3 is a RAN sharing cell, which belongs to both PLMN1 and PLMN2. To enable cell re-selection from cell3 to cell1 or cell2, cell3 indicates camping UEs that both cell1 and cell2 are in the whitelist.

Figure 1 Issue of RAN Sharing

UE1 which is registered to PLMN1 detects cell1 and cell2 when it leaves cell3. The UE considers that both two cells are potential target for cell re-selection as both cells are in the whitelist from cell3. In case the UE1 re-selects to cell2, it will find the cell2’s PLMN (i.e. PLMN2) is not identical with its registered PLMN (i.e. PLMN1). Then, UE1 has to perform cell re-selection evaluation again on cell1. This will delay the cell re-selection and may lead to paging missing and more UE power consumption.

Figure 2 Solution of RAN Sharing Scenario

One solution for the issue is that whitelist is provided for each PLMN in RAN sharing cells. As illustrated in the figure2, the cell3 broadcasts two whitelists, one for PLMN1 and the other for PLMN2. UE follows the whitelist corresponding to its registered PLMN while performing cell re-selection.

Proposal 3: The whitelist is configured per PLMN.

2.4 Whitelist in LTE

In the WID of NR-U [2], one of the objectives is following:

- Mobility: For non-standalone NR-U deployments, connected mode mobility is supported on licensed spectrum using the baseline mobility procedure specified for the concerned licensed radio access technology (LTE or NR). For standalone NR-U deployments, specify mechanisms for inter-cell handover between NR-U and NR-U, inter-cell handover between NR-U and NR, and Inter-RAT handover between NR-U and LTE (connected to either EPC of 5GC). Extensions of mobility-related measurement reporting for unlicensed operation including channel occupancy indication and RSSI measurements.

According to the above description highlighted in yellow, inter-RAT handover between NR-U and LTE should be specified. And it can be deduced that inter-RAT cell re-selection between NR-U and LTE may also happen. Therefore, enhancing LTE cell re-selection procedure to support whitelist can bring benefits when the IDLE/INACTIVE UE performs cell re-selection from LTE cell to NR-U cell.

Proposal 4: Enhance LTE to provide NR whitelist, which will improve the performance of cell re-selection from LTE to NR-U.

3 Conclusion

This contribution discusses details of introducing the whitelist for NR-U and concludes with the following observation and proposals:

Proposal 1: The maximum number of whitelisted cells on each frequency is 16.

Observation1: Include whitelist in the RRCRelease message is helpful for UE to perform cell selection after leaving RRC_CONNECTED state

Proposal 2:  Whitelist can be included in the RRCRelease message.

Proposal 3: The whitelist is configured per PLMN.

Proposal 4: Enhance LTE to provide NR whitelist, which will improve the performance of cell re-selection from LTE to NR-U.

4 References

- R2-1914001 Running RRC CR for NR-based Access to Unlicensed Spectrum, Rapporteur (Qualcomm Incorporated)

- RP-190706 Revised WID on NR-based Access to Unlicensed Spectrum Qualcomm Inc.