3GPP TSG-RAN WG2 Meeting #108                                                        R2-1914414

Reno, USA, 18th – 22nd Nov., 2019                                          Revision of R2-1912212

Source:	CATT

Title:	Equal-priority Handling

Agenda Item:	6.7.3.2

Document for:	Discussion and Decision

Introduction

In RAN2#107, the following agreement was achieved regarding equal-priority handling in intra-UE prioritization:

The case of highest priorities of two conflicting grants are equal is handled according to the following: for CG DG conflict, DG is prioritized, other cases FFS to what extent to specify.

In this contribution, we address the above leftover issue of equal priority handling between CG/CG as well as SR/PUSCH.

Discussion

CG/CG

We believe that, compared to Rel-15, one of the goals of Rel-16 IIoT is to properly serve co-existing URLLC services. And, since the number of supported CG configurations per BWP has been increased in Rel-16 specifically in support of IIoT, a typical case of CG resources overlap will be when two such TSN traffic transmissions collide. Table 5.2-1 of TS22.104 [1] provides a list of performance requirements associated with various periodic deterministic communication services, an extract of is given below. Some may co-exist for a given application e.g. the first three rows of the table are for three different usecases of the Motion Control application. Moreover, the 5GS UE may be attached to a device supporting simultaneously a sensor and a motion controller, hence concurrent motion control and control-to-control communication services. On the other hand, 802.1Q supports only 8 priority levels that 5GS should also map onto 8 priority levels for fair QoS serving and enforcement. And compared to other less stringent services coexisting in the UE (eMBB, periodic reports, etc) such critical flows are expected to be mapped onto a high(est) priority of the 8 levels of 802.1Q. Then, equal priority handling will not be a marginal case. However, as can be observed from the table, there are significant variations in both the reliability (service availability) and latency (transfer interval) requirements of these high-priority flows. Therefore, we believe the most secure scheduling policy in case two configured grants with equal priority have colliding allocations, is to prioritize the grant carrying the LCH with tightest PUSCH duration and/or reliability mapping restriction(s). However, latest RAN1/RAN2 agreements considering the latter parameter converged on a 2-level indication primarily aiming at distinguishing eMBB from URLLC. Hence it is of no use for equal-priority handling where the critical issue is when both are URLLC LCHs, as discussed above. Hence we propose using the PUSCH duration for discriminating two LCHs with equal priority.

Observation 1: Multiple CG configurations typically target serving concurrently different TSN services in Rel-16.

Observation 2: 802.1Q supports only 8 priority levels that 5GS should also map onto 8 priority levels for fair QoS serving and enforcement.

Observation 3: With only 8 priority levels supported in 802.1Q, it can be a typical case where co-existing TSN services are mapped onto the same priority, but have different latency and/or reliability requirements (e.g. sensor and a motion controller per Table 5.2-1 of TS22.104).

Proposal 1: In case of equal-priority, but LCHs with different LCP restrictions in the different grants, the grant carrying the LCH with LCP mapping restriction with shortest PUSCH duration is prioritized.

Extract of Table 5.2-1 of [5]: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 3) | Remarks |
| 99,999 % to 99,99999 % | ~ 10 years | < transfer interval value | – | 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | – | 40 | 1 ms | 1 ms | ≤ 75 km/h | ≤ 50 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | – | 20 | 2 ms | 2 ms | ≤ 75 km/h | ≤ 100 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| … |  |  |  |  |  |  |  |  |  |  |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value |  | 1 k | ≤ 10 ms | 10 ms | - | 5 to 10 | 100 m x 30 m x 10 m | Control-to-control in motion control (A.2.2.2); (note 9) |

SR/PUSCH

In case of equal-priority between SR and PUSCH, it is reasonable to always prioritize the PUSCH, since it carries more than just 1-bit information which is thus more spectral efficient.

Proposal 2: In case of equal-priority between SR and PUSCH, the PUSCH is prioritized.

However, it is interesting to look at the implications of this rule.

In [2] we propose that the priority of a PUSCH transmission overlapping with an SR is determined with the same rule as for data/data prioritization, i.e. involving also the priorities of the MAC CEs multiplexed in the PDU. This actually makes sense when the MAC CE is a BSR MAC CE in the SR/PUSCH prioritization scenario. Indeed, it could be a typical case that:

- New data arrives for an LCH, which triggers an SR because the next UL-SCH does not meet the LCP mapping restriction of the LCH triggering the SR (e.g. a CG not allowed for that LCH via configuredGrantType1Allowed)

- When the SR resource comes, it overlaps with the above-mentioned UL-SCH which cannot include any data from the triggering LCH but can include the triggered BSR.

It is obvious that, if the LCP restriction is not related to the latency or the reliability of the LCH, as for example a CG-restriction (configuredGrantType1Allowed) or Cell-restriction (allowedServingCells), then there is no reason to prioritize the SR since the BSR can directly and timely bring more information. Hence, if BSR MAC CE priority (set to the priority of its triggering LCH) is accounted for in the priority determination of the UL-SCH, both SR and PUSCH will have the same priority and the prioritization rule of proposal 1 will prioritize the PUSCH, which is indeed the good solution for that case.

Proposal 3: The priority of the BSR MAC CE (set to the priority of its triggering LCH) is accounted for in determining the PUSCH priority.

On the other hand, if the LCP restriction of the triggering LCH is related to the latency of the LCH, the above rule will prioritize the PUSCH although the SR, faster, would have been more appropriate for that LCH. Thus, the solution to distinguish both cases is to apply the latency (maxPUSCH-Duration) LCP mapping restrictions to the regular and periodic BSR MAC CEs.

Proposal 4: The latency (maxPUSCH-Duration) LCP mapping restrictions is applicable to regular and periodic BSR MAC CEs.

Conclusion

This contribution discussed equal-priority handling of CG/CG and SR/PUSCH, resulting in the following observations and proposals:

Observation 1: Multiple CG configurations typically target serving concurrently different TSN services in Rel-16.

Observation 2: 802.1Q supports only 8 priority levels that 5GS should also map onto 8 priority levels for fair QoS serving and enforcement.

Observation 3: With only 8 priority levels supported in 802.1Q, it can be a typical case where co-existing TSN services are mapped onto the same priority, but have different latency and/or reliability requirements (e.g. sensor and a motion controller per Table 5.2-1 of TS22.104).

Proposal 1: In case of equal-priority, but LCHs with different LCP restrictions in the different grants, the grant carrying the LCH with LCP mapping restriction with shortest PUSCH duration is prioritized.

Proposal 2: In case of equal-priority between SR and PUSCH, the PUSCH is prioritized.

Proposal 3: The priority of the BSR MAC CE (set to the priority of its triggering LCH) is accounted for in determining the PUSCH priority.

Proposal 4: The latency (maxPUSCH-Duration) LCP mapping restrictions is applicable to regular and periodic BSR MAC CEs.

Reference

- TS 22.104 v16.1.0, Service requirements for cyber-physical control applications in vertical domains, Stage 1 (Release 16), 3GPP

- R2-1914413, MAC PDU priority determination, CATT