3GPP TSG-RAN WG2 Meeting #108	R2-1914523

# Reno, USA, 18th – 22nd November, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 running CR for support of UE Power Saving in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav |  |  |  |  |  |  | Date: |  | 2019-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the enhancements specified as part of the Work Item on UE Power Saving in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add PS-RNTI in abbreviation section 3.1 - Add a new usage of PDCCH in Section 5.2.3 in support of power saving. - Add in Section 7.9 support for preferred DRX parameters and wish to transition out of DRX state in UE assistance information. - Add PS-RNTI to UE identities in Section 8.1. - Section 11: add description of the DCI with CRC scrambled by PS-RNTI procedure for controlling PDCCH monitoring in DRX on-duration. Also captures basic principles of RRM relaxation and per-BWP maximum MIMO Layers configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements agreed as part of WI on UE Power Saving in NR are not specified in stage-2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.2.3, 7.9, 8.1, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MU-MIMO	Multi User MIMO

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NR	NR Radio Access

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

Next Modified Subclause

### 5.2.3	Physical downlink control channels

The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes:

-	Downlink assignments containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to DL-SCH;

-	Uplink scheduling grants containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to UL-SCH.

In addition to scheduling, PDCCH can be used to for

-	Activation and deactivation of configured PUSCH transmission with configured grant;

-	Activation and deactivation of PDSCH semi-persistent transmission;

-	Notifying one or more UEs of the slot format;

-	Notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE;

-	Transmission of TPC commands for PUCCH and PUSCH;

-	Transmission of one or more TPC commands for SRS transmissions by one or more UEs;

-	Switching a UE's active bandwidth part;

-	Initiating a random access procedure;

-	Indicating the UE(s) to monitor the PDCCH during the next occurrence of the DRX on-duration.

A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations.

A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCE. Different code rates for the control channels are realized by aggregating different number of CCE. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET.

Polar coding is used for PDCCH.

Each resource element group carrying PDCCH carries its own DMRS.

QPSK modulation is used for PDCCH.

Next Modified Subclause

## 7.9	UE Assistance Information

When configured to do so, the UE can signal the network through UEAssistanceInformation if:

-	it prefers an adjustment in the connected mode DRX cycle length for the purpose of delay budget reporting, or

-	it prefers certain DRX parameter values, a certain number of maximum secondary component carriers, a maximum aggregated bandwidth and a number of maximum MIMO layers for power saving purpose, or

-	it expects not to send or receive any more data in the near future, and in this case, it can request to transition out of RRC_CONNECTED and with this indication may express its preferred RRC state, or

-	 it is experiencing internal overheating, and in this case, can express a preference for temporarily reducing the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of maximum MIMO layers.

In all cases, it is up to the gNB whether to accommodate the request.

Editor’s note: Exact wording capturing the UE assistance parameters for power saving “number of maximum secondary component carriers, maximum aggregated bandwidth and number of maximum MIMO layers” can be adjusted after stage-3 details are finalized

Editor’s note: Final wording regarding UE reporting “certain DRX parameter values” can be adjusted once RAN2 decides whether (1) UE provides its preference based on any value of the corresponding parameter range, or option (2) UE provides its preference based on pre-defined values or set of configurations that were previously conveyed to the UE by the network

Next Modified Subclause

# 8	NG Identities

## 8.1	UE Identities

In this clause, the identities used by NR connected to 5GC are listed. For scheduling at cell level, the following identities are used:

-	C-RNTI: unique UE identification used as an identifier of the RRC Connection and for scheduling;

-	CS-RNTI: unique UE identification used for Semi-Persistent Scheduling in the downlink or configured grant in the uplink;

-	INT-RNTI: identification of pre-emption in the downlink;

-	MCS-C-RNTI: unique UE identification used for indicating an alternative MCS table for PDSCH and PUSCH;

-	P-RNTI: identification of Paging and System Information change notification in the downlink;

-	SI-RNTI: identification of Broadcast and System Information in the downlink;

-	SP-CSI-RNTI: unique UE identification used for semi-persistent CSI reporting on PUSCH.

For power and slot format control, the following identities are used:

-	SFI-RNTI: identification of slot format;

-	TPC-PUCCH-RNTI: unique UE identification to control the power of PUCCH;

-	TPC-PUSCH-RNTI: unique UE identification to control the power of PUSCH;

-	TPC-SRS-RNTI: unique UE identification to control the power of SRS.

During the random access procedure, the following identities are also used:

-	RA-RNTI: identification of the Random Access Response in the downlink;

-	Temporary C-RNTI: UE identification temporarily used for scheduling during the random access procedure;

-	Random value for contention resolution: UE identification temporarily used for contention resolution purposes during the random access procedure.

For NR connected to 5GC, the following UE identities are used at NG-RAN level:

-	I-RNTI: used to identify the UE context in RRC_INACTIVE.

For Power Saving purpose during DRX, the following identity is used:

-	PS-RNTI: UE identification for indicating a UE to monitor PDCCH on the next occurrence of the connected mode DRX on-duration.

Next Modified Subclause

# 11	UE Power Saving

The PDCCH monitoring activity of the UE in RRC connected mode is governed by DRX, BA, and DCI with CRC scrambled by PS-RNTI.

When DRX is configured, the UE does not have to continuously monitor PDCCH. DRX is characterized by the following:

-	on-duration: duration that the UE waits for, after waking up, to receive PDCCHs. If the UE successfully decodes a PDCCH, the UE stays awake and starts the inactivity timer;

-	inactivity-timer: duration that the UE waits to successfully decode a PDCCH, from the last successful decoding of a PDCCH, failing which it can go back to sleep. The UE shall restart the inactivity timer following a single successful decoding of a PDCCH for a first transmission only (i.e. not for retransmissions);

-	retransmission-timer: duration until a retransmission can be expected;

-	cycle: specifies the periodic repetition of the on-duration followed by a possible period of inactivity (see figure 11-1 below);

-	active-time: total duration that the UE monitors PDCCH. This includes the "on-duration" of the DRX cycle, the time UE is performing continuous reception while the inactivity timer has not expired, and the time when the UE is performing continuous reception while waiting for a retransmission opportunity.

Figure 11-1: DRX Cycle

When BA is configured, the UE only has to monitor PDCCH on the one active BWP i.e. it does not have to monitor PDCCH on the entire DL frequency of the cell. A BWP inactivity timer (independent from the DRX inactivity-timer described above) is used to switch the active BWP to the default one: the timer is restarted upon successful PDCCH decoding and the switch to the default BWP takes place when it expires.

In addition, the UE may be indicated, when configured accordingly, whether it is required to monitor or not the PDCCH during the next occurrence of the on-duration by a DCI with CRC scrambled by PS-RNTI received on the active BWP. By default, if it does not detect a DCI with CRC scrambled by PS-RNTI on the active BWP, the UE does not monitor the PDCCH during the next occurrence of the on-duration. However, it can also be configured to have the opposite behaviour in this case, i.e. monitor the PDCCH during the next occurrence of the on-duration.

Editor’s note: The term “DCI with CRC scrambled by PS-RNTI” is expected to be replaced with an abbreviation (e.g. PSI, DCP, …), to be finalized by RAN2

A UE can only be configured to monitor such DCI with CRC scrambled by PS-RNTI when connected mode DRX is configured, and at occasion(s) at a configured offset before the on-duration. More than one monitoring occasion can be configured before the on-duration. The UE does not monitor the DCI with CRC scrambled by PS-RNTI on occasions occurring during active-time, in which case it monitors the PDCCH during the next on-duration. If no CORESET and search space is configured for DCI with CRC scrambled by PS-RNTI in the active BWP, UE follows normal DRX operation.

When CA is configured, the DCI with CRC scrambled by PS-RNTI is only configured on the PCell.

One DCI with CRC scrambled by PS-RNTI can be configured to control PDCCH monitoring during on-duration for one or more UEs independently.

Power saving in RRC_IDLE and RRC_INACTIVE can also be achieved by UE relaxing neighbour cells RRM measurements when it meets the criteria determining it is in low mobility and/or not at cell edge.

UE power saving may be enabled by adapting the DL maximum number of MIMO layers by BWP switching.

# Annex	- collection of RAN1 and RAN2 agreements on UE Power Saving in NR WI

RAN1 agreements:

Agreements made in RAN1#96bis meeting (Xi’an, China, 08th – 12th  April 2019):

Agreements:

- The PDCCH-based power saving signal/channel is UE-specifically configured.

- The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios):

  - Alt 1: triggering a single UE only

  - Alt 2: triggering UE(s) within a group

    - FFS whether to always trigger all UEs in a group or a subset of it

  - Alt 3: Alt 1 & Alt 2

Agreements:

The assumptions of the DCI design of the PDCCH-based power saving signal/channel include:

- No increase of DCI format size budget

  - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time

- Working assumption: no increase of UE BD/non-overlapping CCE limit

Agreements:

Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios):

- New DCI format(s)

  - The size of new DCI format may or may not be the same size as the existing DCI size

- Enhancement of existing DCI. E.g.,:

  - Additional new field(s)

  - Using the existing DCI format for the power saving purpose

    - Re-purpose field(s) in the DCI

    - The detection of existing DCI format as the indication for the power saving technique

Agreements:

The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide:

- Indication of one or more power saving associated operations.

- Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format

Agreements:

Potential DCI contents in DCI format(s), to be further investigated:

- Power saving technique associated with C-DRX–

  - Essential for UE function for the C-DRX

    - Wakeup –

      - UE is indicated to transition from outside Active Time to Active Time

      - UE is indicated to stay at Active Time

    - Go to sleep–

      - UE is indicated to transition from Active Time to outside Active Time.

      - UE is indicated to stay outside Active Time

    - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time.

- Cross-slot scheduling

- Triggering RS transmission

- CSI report

- Single vs. multi-cell operation

- BWP /SCell

  - BWP & SCell together

  - BWP and SCell have separated fields

- MIMO layer adaptation/number of Anenna adaptation

  - May further depend on RAN4’s work

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding

- PDCCH monitoring periodicity

- PDCCH skipping (skipping duration)-

  - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time

Note that

- For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2.

- For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far

The following candidates may be discussed after RAN2’s SI is completed:

- Skipping number of DRX monitoring

- SPS activation

- DRX configuration

Agreements:

- The design of the DCI format(s) and size needs to account for one or more of the following aspects

  - Within or outside Active Time

  - DCI format size for the power saving signal/channel to fit the DCI format size budget

    - Including aspects whether or not it is necessary to align it with existing DCI format size

  - Other aspects are not precluded

Agreements:

- Support at least one CORESET with configured search space(s) for the power saving signal/channel

  - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable)

Agreements:

- For PDCCH-based power saving signal/channel,

  - The set of AL(s) is configured

  - The number of PDCCH candidate(s) for each AL is configured

Agreements:

- Regarding aperiodic CSI-RS triggering, at least if a UE is operated with cross-slot scheduling based power saving,

  - If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, specification allows the aperiodic CSI-RS triggering offset to be set to a non-zero value.

Agreements:

- For an active DL and an active UL BWP, a UE can be indicated via signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured) where the signalling type is to be down-selected from:

  - Alt 1: MAC-CE based

  - Alt 2: L1 based

- FFS: How to determine the minimum applicable value if explicit value is not provided.

Agreements:

Possible candidate indication methods to adapt the minimum applicable value of K0 (or K2) for an active DL (or UL) BWP, where the indication method is to be selected from:

- Alt 1: Indication of a subset of TDRA entries, e.g., bit-map based indication

- Alt 2: Indication of one active table from multiple configured TDRA tables

- Alt 3: Indication of the minimum applicable value

- Note: Other option is not precluded

Note: PDCCH monitoring case 1-1 is prioritized for the design.

FFS: Whether and how the minimum applicable K0 (or K2) value of the active DL (or UL) BWP is also applied to cross-BWP scheduling

Agreements:

Possible candidate indication methods to adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, where the indication method is to be selected from:

- Alt 1: Implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated

- Alt 2: Indication of the minimum applicable value

- Note: Other option is not precluded

Note: PDCCH monitoring case 1-1 is prioritized for the design.

Agreements:

- The adaptation on the minimum applicable value of K0 does not apply to at least the following cases:

| RNTI | PDCCH search space |
| --- | --- |
| SI-RNTI | Type0 common |
| SI-RNTI | Type0A common |
| RA-RNTI, TC-RNTI | Type1 common |
| P-RNTI | Type2 common |

Conclusion:

Companies are encouraged to further investigate whether to apply the adaptation for at least the following cases:

- K0 related:

| RNTI | PDCCH search space |
| --- | --- |
| C-RNTI, CS-RNTI |  |
| MCS-C-RNTI |  |

- K2 related:

| RNTI | PDCCH search space |  |
| --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in subclause 8.2 of TS 38.213 |  |  |
| C-RNTI, TC-RNTI, CS-RNTI |  |  |
| MCS-C-RNTI |  |  |

Agreements made in RAN1#97 meeting (Reno, US, 13th  – 17th  May 2019):

Agreements:

The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON

- “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space

- FFS: The value and the range of offset

Agreements:

For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC

- FFS whether the DCI is in UESS or CSS or both

- FFS detailed configuration of the power saving information

- FFS the new DCI format

- Note: the same DCI may carry power saving information for one or more UEs

Agreements:

The indication of at least one power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time.

- It applies to UE-specific search space.

  - It is FFS for the common search space.

- The at least one power saving technique(s) includes at least “Cross-slot scheduling”

- FFS: Which existing DCI formats includes the power saving information

  - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0)

  - Use of non-scheduling DCI formats.

- It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used.

- FFS: New DCI format with size aligned with existing DCI format

Agreements:

A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured

- FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

Agreements:

Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time

- Following the principle of Rel-15 search space configuration

- FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time

- FFS whether UE can further monitor the search space set(s) inside Active Time

Agreements:

- The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring

  - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring)

  - FFS whether or not to increase the number of CORESETs relative to that in Rel-15

  - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel

R1-1907939 LS to RAN2 on RAN1 agreements on the PDCCH-based power saving signal/channel design  RAN1 (CATT)

Agreements:

- For outside Active Time, up to [3] CORESETs per BWP is supported for the power saving signal/channel outside Active Time with each CORESET associated with its TCI state and QCL assumption

  - FFS details

  - FFS whether any other additional handling is necessary for FR2 w.r.t. TCI state, and if so, how

Agreements:

Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s)

- FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements

Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update):

- Indicating UE to use the aperiodic RS

  - Aperiodic CSI-RS

  - Aperiodic SRS

  - TRS

- Triggering aperiodic CSI report

- Cross-slot scheduling

- Rel-15 DCI-based BWP switching

The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

Agreements:

For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured).

Agreements:

To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported.

- FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication.

- FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following:

  - Excluding the invalid TDRA entries

  - Re-interpret the selected K0 (K2) value

Agreements:

When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from:

- An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s)

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure.

- FFS: Exclude contention-free RACH

- FFS: Exclude PUSCH scheduled with TC-RNTI

Agreements:

- To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported.

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to:

- PUSCH scheduled by RAR UL grants for contention-free RACH procedure

- PUSCH scheduled with TC-RNTI

Conclusion:

Companies are encouraged to check the following proposal for the application delay:

For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot n+X⋅2μPDSCH2μPDCCH for K0, or slot n+X⋅2μPUSCH2μPDCCH for K2, where

- X = max(Y, Z)

- Y is the minimum applicable K0 value prior to the indicated change

- Z = [1]

  - Z is the smallest feasible non-zero application delay that may depend on DL SCS

  - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP

- FFS: Cross-carrier scheduling with different numerologies

- FFS: interruption time, if any

Agreements made in RAN1#98 meeting (Prague, 26th – 30th August 2019):

Working assumption:

- More than one monitoring occasion can be configured within a slot or multiple slots before the DRX ON

Agreements:

- The new DCI format for power saving signal/channel is configured to be monitored at least in CSS.

  - FFS for UESS

Agreements:

- The CRC of new DCI format for power saving signal/channel is scrambled by PS-RNTI outside active time

Agreements:

- The maximum number of CORESETs for PDCCH-based power saving signal/channel outside Active Time is no larger than the max number that can be configured inside Active Time.

Working assumption:

- The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s).

Working assumption:

UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

Agreements:

- Scheduling DCI format(s), 1-1 and/or 0-1 are enhanced to include the configurable additional field of 1-bit dynamic indication of cross-slot scheduling in the Active Time in Rel-16.

Agreements:

- Power saving information using DCI formats 0_0/1_0 is not supported

Conclusion:

For next meeting, down select the following two alternatives: The configuration of the offset of the PDCCH-based power saving signal/channel

- Alt1: Dedicated configuration with offset relative to the beginning of DRX ON

- Alt2: The Offset is based on search space configuration

- FFS: whether this applies to long DRX only or long/short DRX.

Agreements:

Scheduling DCI format(s), 1-1 and/or 0-1, to indicate the minimum applicable value of K0 (K2) for active DL (UL) BWP during Active Time is supported

- FFS: Whether and how other scheduling DCI format(s) during Active Time can be used

- FFS whether to have joint or separate indication for DL & UL

Agreements:

To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted, the following is supported:

- One or two RRC configured values for restriction to the active TDRA table

- RRC configuration is per BWP

- If there are one or two RRC configured values for a BWP, 1-bit indication to indicate one value from two candidate values

  - For the case of one RRC configured value, the 1-bit indication further indicates whether or not there is no restriction to the active TDRA table

Agreements:

For an activated BWP without the 1-bit indication received in DCI for adapting the minimum applicable value of K0(K2) for the BWP when there are one or two RRC configured values for the BWP, e.g., due to BWP switching triggered by BWP timer expiration, etc., the value applied for the BWP is determined by (to down-select one of them):

- Option 1: No restriction if one value is RRC configured; The lowest-indexed RRC configure value if two values are RRC configured

- Option 2: The configured value if one value is RRC configured; The lowest-indexed RRC configured value if two values are RRC configured;

- Option 3: No restriction

Agreements:

    - The 1-bit indication in DCI format 1_1 or format 0_1 is used to jointly determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 value for the active UL BWP, which are to be applied at least after the application delay.

Agreements:

- For an active DL and/or an active UL BWP, after UE is indicated to change the minimum applicable values of K0 and/or K2 and before the change indication takes effect,

- UE can be scheduled data with restriction based on current active minimum applicable values of K0 and/or K2

Agreements:

- Support per-DL-BWP configuration of maximum number of DL MIMO layers

  - Signalling details up to RAN2

Agreements made in RAN1#98bis meeting (Chongqing, 14th – 22nd October 2019):

      - PDCCH-based power saving signal/channel

Agreements:

The following working assumption is confirmed:

- More than one monitoring occasion can be configured within a slot or multiple slots before the DRX ON

Agreements:

The following working assumption is confirmed:

- UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

Agreements:

Confirm the following working assumption:

- The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s).

Agreements:

UE follows legacy DRX operation to wake up and start the OnDurationTimer at the next DRX ON at least for the case when the active BWP is not configured with the PDCCH –based power saving signal/channel

- FFS other cases (e.g., the monitoring occasion at the PDCCH-based power saving signal/channel is not valid for the UE,  e.g., collision with other procedures)

Agreements:

- The PDCCH-based power saving signal/channel can be configured for long DRX for a UE

  - (Working assumption) If there is also short DRX cycle configured for the UE, the PDCCH-based power saving signal/channel is not applicable for the short DRX cycle

Agreements:

For a UE, the following information can be configured to be included in the new DCI for the WUS PDCCH scrambled by PS-RNTI

- Indication to wake up or not to wake up

- L1 based mechanism for transitioning from ’dormancy-like’ to ’non-dormancy like’ behaviour on activated Scells,  as agreed in MR CA/DC

- FFS: Triggering -CSI-report

- FFS: whether or not the bitwidths of some or all of the above information fields can be zero

Agreements:

CORESET(s) for PDCCH-based power saving signal/channel outside Active Time is not distinguished from those CORESETs inside Active Time.

Agreements:

In Active Time, a Rel-16 UE is not expected to monitor the new DCI format for the WUS PDCCH scrambled by PS-RNTI.

Agreements:

UE starts monitoring the PDCCH with CRC scrambled by PS-RNTI  at the configured PS_offset before the start of DRX ON until the end of the configured range of monitoring.

- The “PS_offset” is a new higher layer parameter

- (Working assumption) the range of monitoring is derived based on existing parameters, e.g., duration and  monitoringSymbolsWithinSlot, in SearchSpace IE without any change

  - Note: the value of “range” is smaller than that of PS_offset, e.g., to ensure UE with sufficient time for preparing for PDCCH monitoring at the DRX ON

Search space is configured for the indication of monitoring DCI with CRC scrambled by PS-RNTI.

- Reuse the existing search space configuration with all or subset of parameters, e.g., the periodicity and the offset

- FFS: The restriction in the  supported periodicity, the aggregation level and the number of blind decoding for the new DCI with CRC scrambled by PS-RNTI

Conclusion:

With the above agreement, it is understood that the search space(s) for WUS PDCCH reuses the existing SS IE (i.e., no new SS IE).

Agreements:

The new DCI of PDCCH-based power saving signal/channel outside Active Time is designed with UE-specific configured power saving information for one or more UEs .

- The new DCI format supports multiplexing of one or more UEs

- FFS: Whether the starting position and the location of the indication in the new DCI format for a specific UE is based on higher layer configuration

- FFS: Details of the DCI format design for a specific UE

  - Whether new DCI format is similar to that of DCI format 2_x

Agreements:

A new DCI format 3_0 is introduced for providing the indication of power saving information used outside Active Time in Rel-16

Agreements:

Terminology of PDCCH-based power saving signal/channel outside Active Time is DCI with CRC scrambled by PS-RNTI

- Note PS-RNTI is monitored only outside Active Time in Rel-16

Agreements:

- TCI state and update mechanism for the CORESET(s) used for DCI format 3_0 follow Rel-15 procedures

Agreements:

If a DCI format 3_0 outside Active Time is not detected by a UE, “UE wakeup or not” is configured by the higher layer signalling to address this case

  - The default is “not wake up”

Agreements:

If UE detects DCI format 3_0 in the monitoring occasion(s), UE follows the indication of wakeup or not from the corresponding field.

      - Procedure of cross-slot scheduling power saving techniques

Agreements:

For an activated BWP without the 1-bit indication received in DCI for adapting the minimum applicable value of K0 (K2) for the BWP when there are one or two RRC configured values for the BWP, e.g., due to BWP switching triggered by BWP timer expiration, etc., the value applied for the BWP before the 1-bit indication is received within the BWP is determined by

- Option 2: The configured value if one value is RRC configured; The lowest-indexed RRC configured value if two values are RRC configured

FFS Value zero is a valid configuration for the minimum applicable K0/K2 value for the case when two RRC values are configured for the BWP

Agreements:

Value zero is a valid configuration for the minimum applicable K0/K2 value for the case when two RRC values are configured for the BWP

- Detail RRC configuration design is up to RAN2.

Agreements:

For the purpose RRC configuration design, the configured minimum applicable K0/K2 value(s) are a subset of the possible values for those of the existing K0/K2 parameters

- FFS the detailed subset of values

Agreements:

UE higher layer signalling (detailed mechanisms up to RAN2) of suggested minimum applicable values for K0/K2 (one for each) for applying cross-slot scheduling is supported:

- For each of the all possible SCSs, the values are reported separately

- For same-carrier scheduling, each suggested value is in the range from 1 to

  - 15kHz/30kHz SCS: [2-4] slots

  - 60kHz/120kHz SCS: [4-8] slots

- FFS how to apply the values to the cross-carrier scheduling case in terms of minimum applicable value

Draft LS to RAN2 to inform the above agreements – R1-1911586 (Weide, MTK)

Agreements:

For the RRC configuration, the configured minimum applicable K0/K2 value(s) take integer value(s) in the range from 0 to [16]

Agreements:

- With application delay, X, for adaptation to the indicated minimum applicable K0/K2 value(s) for a scheduled cell triggered by the 1-bit indication of a DCI format 1-1 or 0-1 with in the scheduling cell,

  - UE receives DCI of the change indication in slot n of the scheduling cell

  - UE can be scheduled with the indicated minimum applicable K0/K2 value(s) for PDSCH/PUSCH on the scheduled cell in a DCI in slot (n + X) of the scheduling cell

- For same-carrier scheduling and at least for PDCCH monitoring case 1-1,

  - X = max(Y, Z)

  - Y is the active minimum applicable K0 value of the active DL BWP prior to the change indication

  - Z is ([1], [1], [2], [2]) for DL SCS of (15, 30, 60, 120) KHz, respectively

- FFS: Cross-carrier scheduling

- FFS: PDCCH monitoring case 1-2 and case 2

- FFS: Whether and how to add a delay for adaptation from same-slot scheduling to cross-slot scheduling before potential data retransmission(s) is finished

- FFS whether or not/how to define the upper bound for the application delay

- FFS whether/how to define UE behavior in case of miss detection

      - UE adaptation to maximum number of MIMO layers

Conclusion:

- No additional spec impact to support adaptation of UL Max #MIMO layers for UL NCB and CB MIMO in Rel-16 power saving WI

Agreements:

- The configured per-BWP DL max MIMO layer value is expected to be less than or equal to the per-cell configured DL Max MIMO layer value (if configured).

Agreements:

- DL-SCH TBSLBRM is invariant across all the DL BWPs, when DL max MIMO layer adaptation is per BWP

  - FFS details

Agreements:

- No spec change for TS38.212 is needed for determining DL-SCH TBSLBRM when downlink max MIMO layer adaptation is configured.

  - Note: DL-SCH TBSLBRM is not dependent on the per-BWP configured maximum number of DL MIMO layers value.

- When maximum number of DL MIMO layers per BWP is configured for all configured DL BWPs for a serving cell,

  - At least one BWP is configured with per-BWP configured maximum number of DL MIMO layers value equal to the per-cell configured maximum number of DL MIMO layers value (if configured).

RAN2 agreements:

Agreements made in RAN2#107 meeting (Prague, 26th – 30th August 2019):

Agreements:

- The PDCCH-WUS triggers a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence of the drx-onDurationTimer.

- The PDCCH-WUS is considered jointly with DRX i.e. it is only configured when DRX is configured.

- The PDCCH-WUS is monitored at occasions located at a configured offset before the start of the drx-onDurationTimer. The offset is part of physical layer design.

- On a PDCCH-WUS occasion that a UE is monitoring, if the UE is indicated to wake-up to monitor the PDCCH during the next occurrence of the drx-onDurationTimer, the UE starts the drx-onDurationTimer at its next occasion. Otherwise it does not.

- From RAN2 point of view the UE does not monitor WUS during active time.

- If UE is in DRX Active Time during a PDCCH-WUS occasion, it starts the drx-onDurationTimer at its next occasion as in legacy.

- The WUS is configured on the PCell with CA and SpCell with DC (i.e. PCell on MCG and PSCell on SCG)

- RLM and RRM measurements are not impacted by WUS design (i.e. the UE continues to measure the required reference signals as per RRM requirements)

Agreements

-	UE indicates the preferred state using RRC signalling.   The information is provided using UE assistance.  The network uses legacy release to transition the UE. It is up to the network whether and which state the UE should transition to.  UE autonomous release is not supported.

-	The network can control the rate in which the UE transmits the release request.  At least a prohibit timer is introduced.

Agreements:

-	UE assistance reporting follows Rel-15 UE assistance information procedure as a baseline.  The network can configure the UE to be able to report C-DRX UE assistance.  The UE reports only the configured UE assistance information that triggered the report.

-	C-DRX configuration will be provided in UE assistance.   Include the following DRX configuration parameters in UE assistance: long DRX cycle, short DRX cycle, DRX inactivity timer, short DRX cycle timer. FFS DRX on duration and DRX start offset

-	BWP and SCells UE assistance there is not enough consensus.

-	LTE PPI is excluded from UE assistance

Agreements - Measurement relaxation criteria can consider both low mobility and UE location in the cell (e.g. whether the UE is in cell-edge). - UE may activate relaxed measurement criteria if at least any of the following conditions are met: a) Serving Cell measurement does not change more than a relative threshold during a time period - LTE relaxed monitoring criteria in 36.304 is considered as a baseline. Additional enhancements to address aspects that are specific to NR can be considered. b) UE is not a cell edge, meaning that serving cell/beam RSRP/RSRQ/SINR is above a threshold FFS: Whether neighbour cell RSRP should also be considered.

Agreements made in RAN2#107bis meeting (Chongqing, 14th – 20th October 2019):

Agreements

=>	From RAN2 perspective, it is desirable to support WUS for both short and long DRX and it can be configurable by the network.  However, RAN2 will follow the final RAN1 decision.

=>	Ask RAN1 if there are technical feasibility concerns to support WUS for short DRX

=>	Ask RAN1, what is the assumption regarding whether the UE considers also PDCCH-WUS during WUS occasion(s) to determine when/whether to report P/SP SRS and CSI (i.e. CSI/SRS are performed only during active time).

Agreements:

- UE assistance information for release request is network configurable.

- Preferred state is optionally included in Release Request in UE assistance information

- Triggering condition of Release Request in the UE assistance is up to UE implementation and we will specify what we expect from the UE.

- Specify that UE may signal via UE assistance that it prefers to be released when the UE may expect not to send or receive more data on near future (like NB-IoT).

- Introduce a RRC prohibit timer for the release request.

- A prohibit timer value = 0 can be configured for UE assistance information for release indication.

- A prohibit timer with value infinity can be configured and large values don’t have to be introduced.  FFS on the additional values and try to keep the values shorter

Agreements:

- UE assistance of C-DRX configuration will not include UE’s preference for “DRX on duration” and “DRX start offset”.

- FFS how UE provides “UE’s preferred C-DRX configuration” information: option (1) UE provides its preference based on any value of the corresponding parameter range, or option (2) UE provides its preference based on pre-defined values or set of configurations that were previously conveyed to the UE by the network

- Prohibit timer will be used as the general framework for UE assistance information and configured per UE assistance type.

- FFS UEAssistanceInformation message can be sent without including “UE’s preferred C-DRX configuration” (i.e. if the UE doesn’t have a preference anymore)

Agreements:

- The network should be able to distinguish from the received message whether it is for overheating or power saving purpose.

- The maximum aggregated bandwidth DL/UL (FR1 and FR2) from Rel-15 overheatingAssistance I.E is re-used for power saving.

- The total number of DL/UL SCells from Rel-15 overheatingAssistance I.E is re-used for power saving

- The maximum number of MIMO layers DL/UL (FR1 and FR2) from Rel-15 overheatingAssistance I.E is re-used for power saving.

- Introduce a new I.E. in UEAssistanceInformation message including the above selected fields from overheatingAssistance I.E.

- The new type of UE assistance information in support of power saving has its own prohibit timer

Agreements:

- RAN2 confirms that the UE power saving in NR network in MR-DC scenarios are within the scope of the WID.

- The solution on how to transfer UE assistance information to NR SN should be discussed together with other UE assistance information (e.g. overheating) in main session.

Agreements:

- Network can configure the triggering criteria independently (i.e. either cell-edge or low mobility or both)

- Cell-edge criteria will not consider neighbour cell measurements