3GPP TSG-RAN WG2 Meeting #108		R2-1914525

Reno, USA, 18th–22nd November 2019

Source:	CATT

Title:	Impacts of PDCCH-WUS on RRC

Agenda Item:	6.11.2

Document for:	Discussion and Decision

Introduction

In previous RAN1 and RAN2 meetings, further agreements on power saving signal procedure were achieved. RAN1 also agreed higher layer parameters for UE Power Saving in NR [1]. In this document, we analyze some issues related to stage-3 impacts (RRC) brought by PDCCH-WUS, including impact on RRC procedures, impact on inter-node messages, and impact on ASN.1 extension.

Discussion

The list of higher layer parameters related to PDCCH-WUS is extracted below from [1]:

| Parameter Name | RAN1 specification / section | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | RAN2 specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PS-RNTI | 38.213 | RNTI-value | new | Per UE | dedicated | Power saving RNTI used for CRC scrambling of PDCCH based power saving signal/channel | 38.331 |
| PS_offset | 38.213 | [1-x slots] | new | Per CG | dedicated | The starting searching time of DCI format 3_0 with CRC scrambled by PS-RNTI relative to the start of DRX ON | 38.331 |
| PS_WakeUpOrNot | 38.213 | 0,1 |  | Per UE | dedicated | When DCI format 3_0 outside active time is not detected, this parameter indicates whether UE wakes up or UE does not wake up. Default is not wake up. | 38.331, 38.321 |
| dci-Format3-0 | 38.212, 38.213 |  | Extension of Rel-15 | Per search space | dedicated | DCI format for the power saving indication in the search space Configuration | 38.331 |

In the following sub-sections, we will discuss three parts, including impact on RRC procedures, impact on inter-node messages, and impact on RRC IE extension.

  - Impact on RRC procedures

In order to configure PDCCH-WUS to a UE, it is straightforward to introduce higher layer parameters of PDCCH-WUS in RRC Reconfiguration procedures, i.e. configuring PDCCH-WUS configurations in RRC reconfiguration message. Then the network can configure/reconfigure PDCCH-WUS configurations in any time during RRC connected.

In order to fast resume a suspended RRC connection from RRC inactive mode, the network can contain MCG configurations in RRC resume message during RRC resume procedure. And in DC and CA enhancement WI, it was also agreed that the network can contain SCG configurations in RRC resume message. Then we think it is also beneficial and straightforward to allow the network to configure PDCCH-WUS in RRC resume message. For example: When a UE enters RRC connected mode from RRC inactive mode, the network can configure an SCG with PDCCH-WUS to the UE via RRC resume message if there is no much data transmitted via SCG and power saving is required.

Proposal 1: Configuring PDCCH-WUS in RRC reconfiguration message is supported.

Proposal 2: Configuring PDCCH-WUS in RRC resume message is supported.

  - Impact on inter-node messages

- Background for DRX coordination in MR-DC

In MR-DC scenarios, separate DRX configurations are supported by MN and SN. In MR-DC scenario, MN and SN may share the same RF chain. If the DRX active time of MN is aligned with that of SN as much as possible, UE power consumption can be reduced. Hence, in order to align DRX configurations between MN and SN, the following coordination for DRX configurations were agreed:

- The MN provides the MCG DRX configurations to the SN via inter-node message (i.e. CG-Configinfo).

- The SN provides the SCG DRX configurations to the MN via inter-node message (i.e. CG-Config).

- Whether to align the SCG DRX configuration with the MCG DRX configuration is left up to SN implementation.

- Whether to align the MCG DRX configuration with the SCG DRX configuration is left up to MN implementation.

Observation 1: In order to align DRX configurations between MN and SN for reducing UE power consumption, DRX configurations are exchanged between inter-nodes via inter-node message.

- Coordination for PDCCH-WUS in MR-DC

It was agreed that PDCCH-WUS can be configured on the PCell on MCG and PSCell on SCG in MR-DC scenario. Similar as coordination for DRX configurations, UE power consumption can be reduced if configurations for PDCCH-WUS can exchanged between MN and SN.

For example:

-	PS_offset is introduced to define the starting searching time of DCI format 3_0 with CRC scrambled by PS-RNTI relative to the start of DRX ON. If the parameter is exchanged between MN and SN, the starting searching time of DCI format 3_0 with CRC scrambled by PS-RNTI between MN and SN can be aligned. Then the UE can wake up to monitor PDCCH-WUSs sent by MN and SN at the same time.

-	PS_WakeUpOrNot is introduced to indicate whether UE wakes up or UE does not wake up if it does not detect PDCCH-WUS. If the parameter is exchanged between MN and SN, the same consistent behavior in that case can be achieved in both CGs of MR-DC.

Hence, in order to align configurations for PDCCH-WUS between MN and SN, parameters for PDCCH-WUS need to be exchanged between MN and SN. Whether to align configurations of PDCCH-WUS with the other node and which parameters to align are left up to RAN node implementation.

For simplify, the whole parameters for DRX configuration were exchanged between inter-nodes. We also can reuse the similar principle to coordination for PDCCH-WUS. However, it is still open on the RRC extension for PDCCH-WUS. We can discuss further which parameters for PDCCH-WUS needs to be exchanged via inter-node message after RRC extensions for PDCCH-WUS were roughly achieved.

Proposal 3: In order to align configurations of PDCCH-WUS between MN and SN:

-	The MN provides related parameters of PDCCH-WUS in MN to the SN via inter-node RRC message;

-	The SN provides related parameters of PDCCH-WUS in MN to the MN via inter-node RRC message.

  - Impact on RRC IE extension

- PS-RNTI

This first parameter in the above RAN1 list of PDCCH-WUS higher layer parameters was agreed in RAN1#97 meeting [2]:

Agreements: A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured - FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

It shows that one new RNTI will be introduced in power saving. In NR, RNTIs are all signalled in IE PhysicalCellGroupConfig. The same principle can also be applied in PS-RNTI. The corresponding TP is provided in Annex 5.1.

Proposal 4: Specify PS-RNTI in IE PhysicalCellGroupConfig.

- PS_offset

This second parameter in the above RAN1 list of PDCCH-WUS higher layer parameters determines when to start to monitor the PDCCH-WUS occasions. It was agreed in RAN1#98bis meeting [3]:

Agreements: UE starts monitoring the PDCCH with CRC scrambled by PS-RNTI at the configured PS_offset before the start of DRX ON until the end of the configured range of monitoring. - The “PS_offset” is a new higher layer parameter - (Working assumption) the range of monitoring is derived based on existing parameters, e.g., duration and monitoringSymbolsWithinSlot, in SearchSpace IE without any change - Note: the value of “range” is smaller than that of PS_offset, e.g., to ensure UE with sufficient time for preparing for PDCCH monitoring at the DRX ON

In the email discussion [5] to this meeting on MAC running CR, a majority of companies supported that the MAC entity indicates the start of PDCCH-WUS monitoring based on the PS_offset, which means it is visible in MAC. Then, it is straightforward to specify this parameter in MAC. Considering it is per CG configured [1], it should be defined in the IE MAC-CellGroupConfig (where DRX parameters are also configured). The corresponding TP is provided in Annex 5.2.

Proposal 5: Specify PS-offset in IE MAC-CellGroupConfig.

- PS_WakeupOrNot

This third parameter in the above RAN1 list of PDCCH-WUS higher layer parameters was agreed in RAN1#98bis meeting:

Agreements: If a DCI format 3_0 outside Active Time is not detected by a UE, “UE wakeup or not” is configured by the higher layer signalling to address this case - The default is “not wake up”

Whether the UE wakes up or not when it does not detect PDCCH-WUS outside Active Time depends on RRC configuration of this parameter PS_WakeUpOrNot. It is per UE configured. But since PDCCH-WUS is only configured on SpCell, it makes sense that this parameter is also configured per CG. Upon notification from lower layers that PDCCH-WUS was received or not, MAC will use this parameters to decide if it starts or not the drx-onDurationTimer. Hence it needs to be visible to MAC and, similar to PS_offset it should be defined in the IE MAC-CellGroupConfig. The corresponding TP is provided in Annex 5.2.

Proposal 6: Specify PS_WakeUpOrNot in IE MAC-CellGroupConfig .

- DCI format 3-0

This fourth parameter in the above RAN1 list of PDCCH-WUS higher layer parameters is for the DCI format which is used for PDCCH-WUS, and agreements were achieved in RAN1#98bis meeting as copied in following. It is one kind of signalling extension of current search space. Then, it makes sense to be defined in the SearchSpace IE. Taking into account the RAN1 agreements, i.e. the new DCI is monitored at least in CSS, DCI format 3-0 can be configured in the common search space. However, there is no extension field for common in SearchSpace. Therefore, it is suggested that dci-Format3-0 is specified in IE PDCCH-Config. The corresponding TP can be found in Annex 5.3.

RAN1#98 meeting [4]:

Agreements: - The new DCI format for power saving signal/channel is configured to be monitored at least in CSS. - FFS for UESS

RAN1#98bis meeting [3]:

Agreements: A new DCI format 3_0 is introduced for providing the indication of power saving information used outside Active Time in Rel-16

Proposal 7: Specify dci-Format3-0 in IE PDCCH-Config.

Finally, it is proposed that:

Proposal 8: Capture the TPs in the Annexes into the running RRC CR.

Conclusion

This contribution discusses the PDCCH-WUS parameters exchange in inter-node message and ASN.1 impacts on RRC. And we provide observation and proposals below:

Observation 1: In order to align DRX configurations between MN and SN for reducing UE power consumption, DRX configurations are exchanged between inter-nodes via inter-node message.

Proposal 1: Configuring PDCCH-WUS in RRC reconfiguration message is supported.

Proposal 2: Configuring PDCCH-WUS in RRC resume message is supported.

Proposal 3: In order to align configurations of PDCCH-WUS between MN and SN:

Proposal 4: Specify PS-RNTI in IE PhysicalCellGroupConfig.

Proposal 5: Specify PS-offset in IE MAC-CellGroupConfig.

Proposal 6: Specify PS_WakeUpOrNot in IE MAC-CellGroupConfig .

Proposal 7: Specify dci-Format3-0 in IE PDCCH-Config.

Proposal 8: Capture the TPs in the Annexes into the running RRC CR.

Reference

- R1-1911733, List of Higher Layer Parameters  for UE Power Saving in NR, CATT, 3GPP TSG RAN WG1 Meeting #98bis, Chongqing, China, Oct 14th – 20th , 2019;

- RAN1#97 meeting, chairman notes;

- RAN1#98bis meeting, chairman notes;

- RAN1#98 meeting, chairman notes;

- [107bis#71][PowerSaving] Running CR 38.321 (Huawei)

- RAN#107 meeting, chairman notes;

Annex

  - PS-RNTI

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

ps-RNTI                             RNTI-Value                                                      OPTIONAL,   -- Cond SpCellOnly

]]

}

PDCCH-BlindDetection ::=                INTEGER (1..15)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clause 9.1.2.1).

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2).

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the MCS-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

pdcch-BlindDetection Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3).

ps-RNTI RNTI for power saving signal (see TS 38.213, clause x). The UE monitors PDCCH scrambled by PS-RNTI to detect whether to start drx-onDurationTimer (see 38.321, clause 5.7).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.101-3 [34]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB. This field is only used in EN-DC.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in EN-DC as defined in TS 38.101-3 [34]. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need R, in the ps-RNTI of an SpCell. It is absent otherwise. |

  - PS_offset, PS_WakeUpOrNot

- -	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                                BOOLEAN                                                     OPTIONAL,   -- Need M

dataInactivityTimer                     SetupRelease { DataInactivityTimer }                        OPTIONAL    -- Cond MCG-Only

]],

[[

ps_offset                               SetupRelease { PS_Offset }                                  OPTIONAL   -- Cond SpCellOnly

]],

[[

ps-WakeUpOrNot                          BOOLEAN                                                     OPTIONAL   -- Cond SpCellOnly

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

PS_Offset ::=                   ENUMERATED {s1, s2, …sx}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

ps_offset Used to configure the offset between start of WUS monitoring occasion(s) and the start of drx-OnDurationTimer as specified in TS 38.321 [3].

ps-WakeUpOrNot When PDCCH-WUS outside active time is not detected, this indicates whether UE wakes up or UE does not wake up. Default is not wake up.

skipUplinkTxDynamic If set to true, the UE skips UL transmissions as described in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need R, in the ps-RNTI of an SpCell. It is absent otherwise. |

  - DCI format 3-0

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled cell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpaceToReleaseList are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId               OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                       OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                     OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                         OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                       OPTIONAL,   -- Need M

...

[[

searchSpacesToAddModList-v16xy      SEQUENCE(SIZE (1..10)) OF SearchSpace-v16xy                 OPTIONAL    -- Cond SpCellOnly

]]

}

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

PDCCH-Config field descriptions

controlResourceSetToAddModList List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network configures at most 3 CORESETs per BWP per cell (including UE-specific and common CORESETs). In case network reconfigures control resource set with the same ControlResourceSetId as used for commonControlResourceSet configured via PDCCH-ConfigCommon, the configuration from PDCCH-Config always takes precedence and should not be updated by the UE based on servingCellConfigCommon.

downlinkPreemption Configuration of downlink preemption indications to be monitored in this cell (see TS 38.213 [13], clause 11.2).

searchSpacesToAddModList List of UE specifically configured Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces).

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH.

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH.

tpc-SRS Enable and configure reception of group TPC commands for SRS.

| Conditional Presence | Explanation |
| --- | --- |
| SpCellOnly | The field is optionally present, Need R, in the ps-RNTI of an SpCell. It is absent otherwise. |

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need R

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...

}

}                                                                                                   OPTIONAL    -- Cond Setup

}

SearchSpace-v16xy ::=                         SEQUENCE {

searchSpaceType-v16xy                   CHOICE {

common-v16xy                            SEQUENCE {

dci-Format3-0-v16xy                     SEQUENCE {

...

}                                                                                           OPTIONAL   -- Need R

}                                                                                                   OPTIONAL    -- Cond Setup

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP