3GPP TSG-RAN WG2 Meeting #108	        				R2-1914607

Reno, USA, 18th - 22th November 2019

Agenda Item:	7.3.2.1.1

Source:	MediaTek Inc., Huawei, HiSilicon,

Title:	Support the change between normal PDCP entity and DAPS PDCP entity

Document for:     Discussion and Decision

# 1 Introduction

In RAN2#107bis meeting, following agreements were made for the single PDCP entity supporting DAPS:

Agreements

Single PDCP entity supporting DAPS:

16	The single PDCP entity for DAPS is modelled to have separate security/ROHC functions in the specification.

17	At the UE side for DRB, the normal PDCP entity is changed to the single PDCP entity supporting DAPS upon reception of HO command; the single PDCP entity supporting DAPS is changed to normal PDCP entity upon release of the source cell.

18	The change between the normal PDCP entity and the single PDCP entity supporting DAPS need to be captured in both RRC and PDCP. FFS on how to capture.

In this contribution, we discuss how to capture the change between the normal PDCP entity and the single PDCP entity supporting DAPS in PDCP.

# 2 Discussion

2.1 ‘DAPS’ PDCP Entity

The normal PDCP entity is the legacy PDCP entity used in current PDCP specification, just as illustrated in Figure 1.  The Single PDCP entity supporting DAPS is the new PDCP entity introduced for DAPS HO, just as illustrated in Figure 2.

In order to simplify the terminology, the single PDCP entity supporting DAPS can be named as ‘DAPS PDCP entity’.

Proposal 1: Confirm to use the term ‘DAPS PDCP’ to name the single PDCP entity supporting DAPS.

Figure 1 Normal PDCP entity

Figure 2 Single PDCP entity supporting DAPS

2.2 PDCP Reconfiguration

Based on the agreements made in RAN2#107bis meeting, for DRBs, upon reception of handover command with DAPS, UE reconfigures the PDCP entity for DAPS instead of performing PDCP re-establishment.

In current PDCP spec, we have PDCP entity handling including PDCP entity establishment, PDCP entity re-establishment, PDCP entity release and PDCP entity suspend.  In order to minimize the impact to the existing procedure and text, the change between the normal and the DAPS PDCP entity can be realized by PDCP reconfiguration.

Proposal 2: Introduce a subsection ‘PDCP reconfiguration’ in PDCP entity handling to support the change between the normal PDCP entity and the DAPS PDCP entity.

PDCP reconfiguration should considers two cases:

- Change from the normal PDCP entity to the DAPS PDCP entity upon reception of HO command;

- Change from the DAPS PDCP entity to normal PDCP entity upon release of the source protocol.

Proposal 3: The ‘PDCP reconfiguration’ procedure handles the two cases:

- Change from the normal PDCP entity to the DAPS PDCP entity;

- Change from the DAPS PDCP entity to normal PDCP entity.

2.3 PDCP Reconfiguration: Normal->DAPS PDCP

Upon reception of the HO command, the RRC layer requests PDCP entity reconfiguration, i.e. the change from the normal PDCP entity to the DAPS PDCP entity. It was agreed that DAPS is configured per DRB as working assumption. It is expected certain indication for DAPS is provided to enable DAPS operation for the DRB in RRCReconfiguration message. If DAPS is enabled, UE shall perform PDCP reconfiguration from normal PDCP to DAPS PDCP.

Proposal 4: When upper layers request a PDCP reconfiguration and DAPS is enabled, UE shall performs PDCP reconfiguration from the normal PDCP to DAPS PDCP.

For the change from the normal PDCP to the DAPS PDCP, the main behavior is to establish the ciphering function, integrity protection function and ROHC profile associated to the target gNB. UE shall apply the security algorithms and keys provided by upper layers to the established ciphering function and integrity protection functions. Meanwhile, the existing ciphering function, integrity protection function and ROHC profile keep working as it is.

Proposal 5: For the change from the normal PDCP to DAPS PDCP, UE establishes a ciphering function, integrity protection function and ROHC protocol stack and applies the security algorithms and keys provided by upper layer.

In NR, PDCP reordering is mandatory and always performed. Different from NR, PDCP reordering is optional in LTE and is performed when split bearer or LWA is configured. For DAPS DRB, PDCP reordering should be enabled.

2.4 PDCP Reconfiguration: DAPS->Normal PDCP

Upon release of the source cell, the RRC layer requests PDCP entity reconfiguration, i.e. the change from the DAPS PDCP entity to normal PDCP entity. It is expected certain indication is provided to release the source protocol. If the source protocol stack is released, UE shall perform PDCP reconfiguration from DAPS PDCP to normal PDCP.

Proposal 6: When upper layers request a PDCP reconfiguration and the source protocol is released, UE shall performs PDCP reconfiguration from DAPS PDCP to normal PDCP.

For the change from DAPS PDCP to the normal PDCP, since the RLC entity associated to the source cell is released, the main behavior is to release the ciphering function, integrity protection function and ROHC profile associated to the source gNB.

Proposal 7: For the change from DAPS PDCP to the normal PDCP, UE releases the ciphering function, integrity protection function and ROHC protocol stack associated to the released RLC entity.

Furthermore, the PDCP behavior is very similar as PDCP data recovery due to release of the RLC entity. The receiving PDCP entity shall trigger a PDCP status report to enable the DL data transmission at the target cell. The triggering of PDCP status report is for RLC AM bearer.

Proposal 8: PDCP status report is triggered when upper layer requests a PDCP reconfiguration with source protocol release.

Since both the DL and UL operation keeps on-going during the PDCP reconfiguration procedure, the state variables control the transmission and reception operation should not be reset and the corresponding timers including t-Reordering and discardTimer keeps running.

Proposal 9: The state variables control the transmission and reception operation should not be reset and the timers including t-Reordering and discardTimer keeps running during PDCP reconfiguration procedure.

# 3 Conclusion

In this contribution, we discuss how to capture the change between the normal PDCP entity and the single PDCP entity supporting DAPS in PDCP.

Proposal 1: Confirm to use the term ‘DAPS PDCP’ to name the single PDCP entity supporting DAPS.

Proposal 2: Introduce a subsection ‘PDCP reconfiguration’ in PDCP entity handling to support the change between the normal PDCP entity and the DAPS PDCP entity.

Proposal 3: The ‘PDCP reconfiguration’ procedure handles the two cases:

- Change from the normal PDCP entity to the DAPS PDCP entity;

- Change from the DAPS PDCP entity to normal PDCP entity.

Proposal 4: When upper layers request a PDCP reconfiguration and DAPS is enabled, UE shall performs PDCP reconfiguration from the normal PDCP to DAPS PDCP.

Proposal 5: For the change from the normal PDCP to DAPS PDCP, UE establishes a ciphering function, integrity protection function and ROHC protocol stack and applies the security algorithms and keys provided by upper layer.

Proposal 6: When upper layers request a PDCP reconfiguration and the source protocol is released, UE shall performs PDCP reconfiguration from DAPS PDCP to normal PDCP.

Proposal 7: For the change from DAPS PDCP to the normal PDCP, UE releases the ciphering function, integrity protection function and ROHC protocol stack associated to the released RLC entity.

Proposal 8: PDCP status report is triggered when upper layer requests a PDCP reconfiguration with source protocol release.

Proposal 9: The state variables control the transmission and reception operation should not be reset and the timers including t-Reordering and discardTimer keeps running during PDCP reconfiguration procedure.