3GPP TSG-RAN WG2 #108	Tdoc R2-1914750

Reno, U.S., 18th – 22nd November 2019	resubmission of R2-1909367

Agenda Item:	6.7.2.2.1

Source:	Ericsson

Title:	On support of non-integer multiple of CG/SPS periodicities

Document for:	Discussion, Decision

# Introduction

In this paper, we discuss the support of non-integer multiple of CG/SPS periodicities.

# Discussion

In the previous meeting, the following is agreed

R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption.

This was further discussed in RAN2#107bis without any agreements.

The 5G system, integrated to a TSN network, will need to transfer industrial type of traffic. This traffic is generated many times from existing hardware, for example PLCs, where the periodicities cannot be adjusted. Such hardware typically takes years, if not decades, to be replaced. The periodicity of such hardware devices come from their frequency of operation (i.e., 100 or 120 Hz) that typically provides non-divisible periodicities for the 5G system.

Since the TSN and the CG/SPS periodicities do not match, the latter needs to be adjusted every now and then to reduce/minimize the mis-aligned latency (mis-aligned latency is denoted as the time that the TSN traffic waits for the configured grant or the semi-persistent scheduling to become available).

Suppose the periodicity of the TSN stream is P and the closest CG/SPS periodicity to the TSN periodicity is P’. The difference between these two periodicities is Delta. The issue is that the mis-alignment delay is accumulated each time a new TSN packet arrives, that is, after N TSN packets, the mis-alignment latency between CG/SPS resource and packet arrival is N*Delta. To reduce the mis-alignment delay, the CG/SPS periodicity needs to be re-adjusted once the absolute value of N*Delta becomes large. To further reduce and even minimize the mis-alignment delay, the adjustment is needed when the value of N*Delta is larger than the smallest resource allocation unit in time (i.e., 2 OFDM symbols).

To account for accumulated differences due to periodicity mis-match, CG/SPS resource may need to be adjusted after some number of packet arrivals.

In the previous meeting, it is agreed that short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. The aim is to reduce the initial periodicity mis-match Delta, which will not resolve the issue itself but can reduce the frequency of the need for adjustment. If the need for adjustment is less frequently by a finer resource allocation, the Rel-15 adjustment by DCI/RRC (for configured grant type 1 only) seems to be enough.

RAN2 to confirm that the existing reconfiguration mechanisms in Rel-15 by DCI and RRC (for configured grant type 1 only) can also be used to mitigate the periodicity misalignment between TSN periodicity and CG/SPS periodicity.

As captured in TR 38.825 Section 6.5.2, the severity of this issue depends on the traffic's latency requirement and the frequency of misalignment occurrences. For example, in a smart grid application, the traffic period can be 0.833ms (corresponding to 1200 Hz) and the latency guarantee is 0.833ms. For the deployment of 15 KHz sub-carrier spacing, the closest SPS/CG periodicity is 1 ms and suppose gNB allocate a transmission of 0.5 ms. The Delta (i.e., periodicity mis-match) is 0.167 ms and for every third packet (i.e., every 2.5 ms), the accumulated offset is 0.5 ms and this calls for a need for adjustment either through DCI or RRC. This frequent reconfiguration of every 2.5 ms might be excessive.

One solution to solve this issue is for gNB to over-provision resources.

For supporting this type of traffic (periodicity of 0.833 ms and latency target of 0.833 ms) over the 5G, one example of using configured grant is given. The RAN delivery target is set to 0.5 ms. The configured grant resources are allocated every 0.5ms with a duration of 0.5ms (using a 7-symbol long mini-slot). That makes resources available at time 0, 0.5,1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5 milliseconds and so on.

Table 1 Configured Grants to support the TSN traffic reducing the mis-alignment latency.

|  | 1st | 2nd | 3rd | 4th | 5th | 6th | 7th | 8th |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arrival time (ms) | 0 | 0.833 | 1.667 | 2.5 | 3.333 | 4.167 | 5 | 5.833 |
| RAN delivery target (ms) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Start of the CG resource with the least alignment latency (ms) | 0 | 1 | 2 | 2.5 | 3.5 | 4.5 | 5 | 6 |

Note that, for larger TSN periodicities such as 16.67 ms, although a reconfiguration might be needed every third packet, but the frequency of reconfiguration in absolute time is still acceptable (in this case, it is 50 ms).

Thus, we observe that

In the case of a very short TSN periodicity, gNB need to over-provision resources to solve the misalignment issues.

The last row of the above table shows which CG resource is used for transmission. Not all available configured grants are used, e.g., the CG resource at 0.5 millisecond and 1.5 milliseconds are not used. In this example, only the 1st, 3rd, and 5th configured grants will be used, while the 2nd and the 4th are not utilized since there is no TSN traffic waiting to be transmitted. This pattern of configured grants continues over time to stay on par with the TSN pattern requirements.

As illustrated above, from the knowledge on the TSN traffic periodicity and the configured-grant/SPS periodicity, gNB can find out those configured-grant/SPS resource allocation timings that will not be used by TSN traffic. Since no TSN traffic is expected from these allocated resources, it can be beneficial for system capacity point of view for gNB to schedule transmissions on those configured-grant/SPS resources for traffic from other UEs.

In order to make sure no other traffic from the same UE would be mapped to the configured-grant resources, an LCH restriction can be introduced so that only the LCH that carries this periodic TSN stream can be mapped to the configured grant as discussed in our companion paper [1] and agreed in the RAN2#107 meeting. Of course, another approach is to not use configure grants for these cases at all. With dynamic scheduling we will get the best possible alignment and no resource waste.

Additionally, an indication of an active/in-active pattern of configured-grant/SPS resource allocation by network configuration (i.e. allow the unused configured grant/DL SPS resources to be assigned to other UEs using dynamic grants or dynamic assignments) can also be considered. This is similar to the bit-map solution as specified in the LTE feLAA.

Like LTE feLAA, a bitmap to indicate the allowed time resource for CG/SPS can be considered to address resource consumption.

From standardization efforts point of view, we should aim to achieve maximum synergies among different work items and re-use the techniques as much as possible. This solution is to address an optimization aspect in NR IIoT work item, while NR-U is working on similar problems related with configured grant time domain resource allocation from the essential LBT mechanisms. Thus, we propose to further consider this and wait for progress in NR-U work item.

RAN2 to further consider solutions to address resource consumption in the support for TSC message periodicities with non-integer multiple of NR supported CG/SPS periodicities.

# Conclusion

The following observations have been made:

Observation 1	To account for accumulated differences due to periodicity mis-match, CG/SPS resource may need to be adjusted after some number of packet arrivals.

Observation 2	In the case of a very short TSN periodicity, gNB need to over-provision resources to solve the misalignment issues.

Observation 3	Like LTE feLAA, a bitmap to indicate the allowed time resource for CG/SPS can be considered to address resource consumption.

Based on the discussion above, we propose the following:

Proposal 1	RAN2 to confirm that the existing reconfiguration mechanisms in Rel-15 by DCI and RRC (for configured grant type 1 only) can also be used to mitigate the periodicity misalignment between TSN periodicity and CG/SPS periodicity.

Proposal 2	RAN2 to further consider solutions to address resource consumption in the support for TSC message periodicities with non-integer multiple of NR supported CG/SPS periodicities.

# References

R2-1909374, Reliability aspects in LCP restriction enhancement, Ericsson, RAN2#107