3GPP TSG-RAN WG2 #108	Tdoc R2-1914761

Reno, U.S., 18th – 22nd November 2019	(Update of R2-1912559)

Agenda Item:	6.7.4.1

Source:	Ericsson

Title:	Open issues related to MAC CE for PDCP Duplication

Document for:	Discussion, Decision

# 1	Introduction

A new MAC CE format has been agreed to control PDCP duplication in Release 16. This new MAC CE format would provide an extra flexibility which allows individually controlling the RLC entities which are involved in the duplication operation. Yet, when this flexibility is not needed, Rel-15 MAC CE could still be used to control Rel-16 PDCP data duplication.

This contribution looks at the following 3 aspects:

- Rel-16 MAC CE configuration and operation details.

- Rel-15 MAC CE to control PDCP duplication features.

- Rel-16 MAC CE format.

# 2 Discussion

Apart from the open issues discussed in the below sections it is assumed that UE based solutions for duplication control are not pursued at this point in time due to prioritisation in RAN plenary. If additional changes are to be defined at a later stage, those additions should only be considered as additions on top of a baseline (based on NW control) when the current open issues have been resolved.

## 2.1 Rel-16 MAC CE details to control Rel-16 PDCP duplication

A new MAC CE format to control the RLC entities to which the PDCP delivers duplicates has been agreed. The MAC CE may be able to indicate the RLC entities involved in duplication. If all configured RLC entities can be involved in the duplication process independently, the MAC CE should be able to control each of the configured RLC entities individually.

### Activation and deactivation of duplication

PDCP Duplication can only be considered activated if there is a minimum of two activated RLC entities. Thus, at least a number of 2 RLC entities should be activated via RRC or MAC CE to activate PDCP duplication. Otherwise, duplication should be considered deactivated. In other words, PDCP duplication is de-activated when the Rel-16 MAC CE indicates 1 or 0 activated RLC entities.

PDCP Duplication is considered activated with Rel-16 MAC CE when PDCP duplication towards at least 2 RLC entities is activated and otherwise it is considered deactivated.

PDCP can deliver to RLC PDCP Data PDUs or PDCP Control PDUs. We consider that duplication always refers to PDCP Data PDUs and not to PDCP Control PDUs (following the Release 15 operation). Activation of an RLC entity therefore means that the PDCP should duplicate PDCP Data PDUs and deliver them to the activated RLC entities (one duplicated packet to each RLC entity). Thus, activation or deactivation of an RLC entity does not impact or change the RLC entity to which PDCP Control PDUs are delivered (i.e. the primaryPath).

Similar to the discussion above, the deactivation of an RLC entity should be interpreted as that the PDCP entity should stop delivering PDCP Data PDUs to that RLC entity.

Rel-16 MAC CE can activate and deactivate all configured RLC entities individually.

Activating/(de-activating) PDCP duplication towards an RLC entity means that the PDCP entity shall (not) deliver duplicated PDCP Data PDUs to that RLC entity.

When PDCP duplication is deactivated altogether, the UE applies the configured (legacy) DC/CA behaviour. Note that LCH cell restrictions w.r.t CA duplication may require additions to this baseline.

## 2.2 Rel-15 MAC CE to control Rel-16 PDCP duplication

In Release 16, it has been agreed that up to 4 RLC entities can be configured. RLC entities can be activated and deactivated via a new Rel-16 MAC CE format introduced for this purpose. However, when the network does not need the flexibility to individually control activation and deactivation of RLC entities, the Release 15 MAC CE could be used.

### Activation of duplication

Activation of duplication can be achieved by the Release 15 MAC CE. All configured RLC entities are then activated by using the Rel-15 MAC CE.

### Deactivation of duplication

Rel-15 MAC CE can be used to deactivate Rel-16 PDCP data duplication as used in Release 15. When a bearer is indicated to be deactivated, then the PDCP Duplication function is deactivated for that bearer i.e. PDCP stops delivering duplicates.

Rel-15 MAC CE can also activate and deactivate Rel-16 PDCP duplication.

Upon reception of a Rel-15 MAC CE activation/deactivation, all configured RLC entities are activated/deactivated.

## 2.3 MAC CE format

In general, the main reasons to choose one option or another option are the use cases to address, the flexibility within those use cases, and size of the MAC CE.

With regards use cases, we think that Release 16 PDCP Duplication will be primarily targeted to URLLC services with extreme latency and reliability requirements. This narrows down the number of services which could benefit from these enhancements. This also implies that the number of RBs which may be configured with these features will be very limited. In Release 15, RAN2 considered that up to 8 RBs may be configured with Rel-15 PDCP duplication. We think that it is not realistic to consider that the same number of bearers would be configured with such extreme requirements. It is more realistic to consider that 1 or 2 DRBs out of these 8 RBs may have such requirements.

With regards the size of the MAC CE, a MAC CE format indicating 1 DRB, see Figure 1, consists of 2 bytes; while a MAC CE format indicating all DRBs (up to 8 DRBs) consists of 5 bytes and 3 bytes for the solutions outlined in Figure 2 and Figure 3 respectively.

Figure 1	Rel-16 MAC CE to indicate 1 DRB and RLC activation status for the RB

Figure 2	Rel-16 MAC CE to implicitly signal all the DRBs and the RLC status for all the RBs.

Figure 3	Rel-16 MAC CE to indicate activation status of all RBs and the RLC status for the activated RBs

The main advantage of the MAC CE format shown in Figure 3 is the lower overhead when multiple RBs (more than 2 RBs) need to be updated simultaneously. Yet, we do not think that there will be more than 1 or 2 bearers configured with these features and it is not obvious either that all configured DRBs may need a dynamic change right at the same time.

Therefore, we think that the MAC CE should indicate only 1 DRB and its corresponding activation status for each RLC entity (e.g. Figure 1).

The MAC CE indicates only 1 DRB and indicates towards which of the four potential associated RLC entities of the DRB PDCP duplication is activated.

The PDCP duplication activation status towards the RLC entities is indicated by 3 or 4 bits (depending on whether primary path can be deactivated for duplication).

The MAC CE bits to activate/deactivate PDCP duplication for the 3 or 4 RLC entities consider the following order: RLC entities associated with DRBs for which PDCP duplication is configured in ascending order of LCIDs, first for MCG, then for SCG.

Example to clarify proposal 8: 4 RLC entities, 3 RLC (LCID = 3, 5, 6) in MCG and 1 RLC in SCG; each bit in the MAC CE would correspond as follows RLC_0 => LCID 3 (MCG),  RLC_1  LCID = 5 (MCG), RLC_2  LCID = 6 (MCG), RLC_3 = LCID from SCG

RAN2 to agree on the MAC CE format shown in Figure 1 (taking into account conclusion w.r.t primary path).

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	PDCP Duplication is considered activated with Rel-16 MAC CE when PDCP duplication towards at least 2 RLC entities is activated and otherwise it is considered deactivated.

Proposal 2	Rel-16 MAC CE can activate and deactivate all configured RLC entities individually.

Proposal 3	Activating/(de-activating) PDCP duplication towards an RLC entity means that the PDCP entity shall (not) deliver duplicated PDCP Data PDUs to that RLC entity.

Proposal 4	Rel-15 MAC CE can also activate and deactivate Rel-16 PDCP duplication.

Proposal 5	Upon reception of a Rel-15 MAC CE activation/deactivation, all configured RLC entities are activated/deactivated.

Proposal 6	The MAC CE indicates only 1 DRB and indicates towards which of the four potential associated RLC entities of the DRB PDCP duplication is activated.

Proposal 7	The PDCP duplication activation status towards the RLC entities is indicated by 3 or 4 bits (depending on whether primary path can be deactivated for duplication).

Proposal 8	The MAC CE bits to activate/deactivate PDCP duplication for the 3 or 4 RLC entities consider the following order: RLC entities associated with DRBs for which PDCP duplication is configured in ascending order of LCIDs, first for MCG, then for SCG.

Proposal 9	RAN2 to agree on the MAC CE format shown in Figure 1 (taking into account conclusion w.r.t primary path).

# References

R2-1914760, Configuration and RRC aspects of PDCP Duplication, Ericsson, RAN2#108