3GPP TSG-RAN WG2 #108																																																											R2-1914841

Reno, USA, November 18th – 22nd, 2019

Agenda item:					6.9.1

Source:	Intel Corporation

Title:		UE Feature list for Rel-16 Mobility Enhancement

Document for:	 				Discussion and decision

# Introduction

This is to capture UE Feature list for Rel-16 Mobility Enhancement from RAN2 perspective.

# UE Feature list for Rel-16 NR Mobility Enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 NR Mobility Enhancement | x-0 | DAPS Handover | - sync/async DAPS HO ; - support for multiple TAG (i.e. different TAG in source and target cells); - ability to simultaneously transmit with both source and target PCells based on RF chain capability (i.e. whether dual UL is supported during the DAPS HO or not); - DAPS Handover involving different SCS (as agreed in RAN4); |  | Yes | N/A | UE cannot support DAPS Hanover | 1)-4)per band combination | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling(1-4) |
|  | x-1 | CHO | - Handling of CHO configuration, including execution condition and candidate cell configuration; - Maximum candidate cells the UE can support; - CHO based failure handling; - Multiple trigger events for one execution condition |  | Yes | N/A | UE cannot support CHO | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1-4) |
|  | x-2 | Condition based NR PSCell addition/change | - Handling of condition based NR PSCell configuration, including execution condition and candidate cell configuration; - Maximum candidate PSCells the UE can support; - CHO based failure handling; - Support of EN-DC; - Support of ng-EN-DC; - Support of NR DC; | X-1 | Yes | N/A | UE cannot support condition based NR PSCell addition/chagne | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1-6) |
|  | x-3 | T312 based PSCell fast failure recovery | - T312 based PCell fast failure recovery |  | Yes | N/A | UE does not support T312 based PSCell fast failure recovery | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |
|  | X-4 | T312 based PSCell fast failure recovery | - T312 based PSCell fast failure recovery |  | Yes | N/A | UE does not support T312 based PSCell fast failure recovery | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |

# 3 References

[1] RP-192277 NR mobility enhancements, Intel Corporation

[2] TS38.331