3GPP RAN WG2 Meeting #108	R2-191xxxx
Reno, U.S.A., Nov. 18th-22nd, 2019	Re-submission of R2-1912892

Agenda Item:	6.2.3.3

Source:	InterDigital

Title:	RLM and RLF for NR-U

Document for:	Discussion, Decision

# Introduction

During the Study Item of NR-U, RLM topic was discussed and an agreement has been made that potential modifications to RLM/RRM procedures due to reduced transmission opportunities for DL signals and channels due to LBT failure should be identified and studied.

In TR 38.889, the following was captured regarding the changes of RLM procedures due to LBT and the support of multiple beam operation in NR-U:

For RRM, RLM, and mobility procedures, NR licensed specification in Rel-15 are considered as a baseline for NR-U. However, changes to these due to new physical layer design and LBT for the unlicensed operation can be introduced. These will support both synchronous and, except for LAA case, asynchronous deployments.

The RRM and RLM framework for NR-U will also support multiple beam operation.

In RAN1 #95  the following agreements were made:

- For RLM, the following recommendations are considered beneficial for further design in the WI:

  - Identifying a set of RLM-RS, e.g., DRS, SS/PBCH blocks, CSI-RS

    - Transmission of the RS in a COT may be subject to LBT

  - Identifying which set(s) of RLM-RS are used for in-sync and out-of-sync evaluations

    - For example, determining which RLM-RS within or outside the DMTC for RLM can be utilized for in-sync and out-of-sync evaluations

  - Potential definition of a metric, e.g., Rel-15 out-of-sync indication or new metric, to accurately identify instances of unsuccessful detection of RLM-RS. Whether/how to report such a metric to higher layers is to be further studied.

This contribution discusses implications to the radio link monitoring (RLM) and the radio link failure (RLF) framework for NR when operating in the unlicensed spectrum.

# Impact of LBT on RLM in NR-Unlicensed

In NR, the downlink radio link quality of the primary cell is monitored by a UE for indicating out-of-sync/in-sync status to higher layers. For this purpose, the UE is configured by higher layers with a set of resources comprising either SSB or CSI-RS.  The same modelling as NR was agreed to be used as baseline for radio link monitoring in NR-U.

In legacy RLM-RS operation, L1 measurements are performed to assess the quality of the radio link, indicating either in-sync (IS) or out of sync (OOS) to higher layers. These measurements rely on predictable transmission of the RLM-RS signals, which cannot be guaranteed in the unlicensed spectrum due to possible network LBT failure. Based purely on measurements, the UE might not be able to distinguish between poor link quality or network LBT failure. As such, both may lead to an OOS condition being indicated. This can result in RLF being triggered during good channel conditions, causing unnecessary interruption, latency, and signalling overhead.

Upon detecting physical layer problems for the SpCell (i.e. upon receiving N310 consecutive out-of-sync indications from lower layers), the timer T310 is started. Under high channel load, the NW may not be able to acquire the channel for the whole duration of T310, resulting in the UE declaring RLF and initiating re-establishment. Moreover, upon declaring RLF, the UE may re-select the same cell if the cause was not actually poor channel quality and if the channel becomes available, resulting in an unnecessary service interruption.

Observation 1: 	R15 RLF procedures may result in spurious triggering of RLF in unlicensed spectrum due to the network failing to acquire the channel and providing RLM-RS under high channel load.

Therefore, when the UE has determined that an RS is missed due to LBT, an OOS indication may not be suitable. However, completely ignoring cases where the RLM_RS are absent for extended periods of time, may lead the UE will stay in a prohibitively busy channel without triggering any recovery action.

Observation 2: If the RLM_RS are absent for extended periods of time and ignored by the UE, the UE may stay in a busy channel without triggering any recovery action.

Proposal 1:  	NR-U supports a recovery action when lower layers indicate failure to evaluate link quality due to missing RLM-RS samples. Details of recovery action (e.g. RLF, switching to default BWP) are FFS.

The RLM mechanism can be enhanced for operation in unlicensed spectrum through several possible solutions. A desirable feature would be the ability for the UE to distinguish a missed RLM-RS from poor channel conditions, possibly through clear channel assessment (CCA) during RLM (e.g. RSSI measurement, channel occupancy…).  Another way to evaluate the conditions for this indication may be based on a new metric. This metric may be based on a specific BLER threshold i.e. considerably lower than OOS BLER threshold.

RAN1 has agreed a UE may assume the presence of a signal, such as the DMRS and/or GC-PDCCH, to detect transmission bursts by the serving gNB and enable efficient PDCCH monitoring. The detection of a DL transmission burst may also be used by the UE to determine whether an RLM-RS in the active DL BWP has been transmitted by the gNB.

RAN1 also agreed that the payload of a GC-PDCCH transmission can contain information regarding COT structure that can be used by the UE for power saving. The COT structure indication can also be used by the UE to determine whether an RLM-RS has been transmitted.

Proposal 2:  	The UE can determine whether an RLM-RS is missing by signal-based detection of a DL transmission burst (e.g. DMRS and/or GC-PDCCH) or COT structure indication.

While it may not be suitable to indicate OOS when the UE has identified missed RLM-RSs, it is still beneficial that an indication be sent to higher layers to enable recovery actions when the channel has been busy for an extended period.

At each evaluation period, the physical layer in the UE may indicate to higher layers either OOS, IS or a third type of indication when the RLM-RS are missing e.g. a non-sync indication. This indication could be used by the UE when it has not detected a DL burst in its active DL BWP as described in proposal 2.

Proposal 3:  	NR-U supports an indication of missing RLM-RS (Non-Sync indication), in addition to IS and OOS.

This indication may be treated differently than the OOS and may be for example associated with counters and timers to declare RLF that are less aggressive than, and independent from, the ones of OOS.

# Conclusion

In this contribution the following observations were made on RLM-RLF for NR-U:

Observation 1: The UE may spuriously trigger RLF in the unlicensed spectrum due to the network failing to acquire the channel and provide RLM-RS under high channel load.

Observation 2: If the RLM_RS are absent for extended periods of time and ignored by the UE, the UE may stay in a busy channel without triggering any recovery action.

And the following proposals were made:

Proposal 1:  	NR-U supports a recovery action when lower layers indicate failure to evaluate link quality due to missing RLM-RS samples. Details of recovery action (e.g. RLF, switching to default BWP) are FFS.

Proposal 2:  	The UE can determine whether an RLM-RS is missing by signal-based detection of a DL transmission burst (e.g. DMRS and/or GC-PDCCH) or COT structure indication.

Proposal 3:  	NR-U supports an indication of missing RLM-RS (Non-Sync indication), in addition to IS and OOS.

# Annex

In RAN2#105bis [3], impacts of LBT on RLM procedure were discussed as part of the connected mode measurements email discussion [105#48] and the following was agreed:

R2 assumes that missing measurements due to LBT failures do not impact the R2 specification of L3 filtering and the subsequent steps.

Handling of delayed SSB transmissions due to LBT does not impact L3

FFS: A new RLF trigger mechanism for missing RLM-RS may be defined at upper layers but RAN2 should wait for RAN1 conclusion on this issue

RSSI and Channel Occupancy configuration and reporting, in particular measurements over an interval (at least for CO) and periodical reporting, are used as a baseline for NR-U

Measurement and reporting of WLAN nodes are not supported in NR-U

# References

TR 38.889, “Study on NR-based Access to Unlicensed Spectrum”.

R2-1905501 – RAN2 105bis Chairman Notes – RAN2 Chairman.