3GPP TSG-RAN WG2 Meeting #108	R2-1914992

Reno, USA, November 18th – 22nd, 2019	                                              Resubmission of R2-1913514

Title:	Draft LS on the Timing of AS Key Derivation in Conditional Handover

Response to:	-

Release:	Release 16

Work Item:	NR_Mob_enh-Core, LTE_feMob-Core

Source:	Futurewei (to be TSG RAN WG2)

To:	TSG SA WG3

Cc:	TSG RAN WG3

Contact Person:

#### Name:	Hao Bi

E-mail Address:	Hao dot Bi at Futurewei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In conventional handover, receiving nextHopChainingCount in RRCReconfiguration/RRCConnectionReconfiguration message will trigger UE to update KgNB/KeNB key based on the current KgNB/KeNB key or the NH, depending on if the received nextHopChainingCount is different from the nextHopChainingCount associated with the currently active KgNB/KeNB.

Conditional handover may not occur when UE receives RRCReconfiguration/RRCConnectionReconfiguration message with nextHopChainingCount, if the configured condition is not met on the candidate cell. As the source cell has only one nextHopChainingCount valid for handover, the same nextHopChainingCount can be received multiple times when multiple candidate cells are configured for conditional handover, or when there is modification of the target cell configuration, or when a conventional handover is triggered before previous configured conditional handover is executed.

Hence, the receiving of nextHopChainingCount in the RRCReconfiguration/RRCConnectionReconfiguration message for conditional handover should not trigger the derivation and update of security key in conditional handover. Instead, the derivation and update of security key should be done when conditional handover is being executed.

2. Actions:

To SA3 group.

ACTION: 	RAN2 respectfully asks SA3 to take into account the above information on conditional handover.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109		February 24 – 28, 2020	Athens, Greece

3GPP RAN2#109bis		April 20 – 24, 2020		TBD, Japan