3GPP TSG-RAN WG2 Meeting #108	R2-1915252

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1364 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change on transfer-type of handoverCommandMessage for NR SA handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently, handoverCommandMessage which is included in HandoverCommand inter-node message contains the OCTET STRING of RRCReconfiguration as below. handoverCommandMessage Contains the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB. However, entire DL-DCCH-Message including the RRCConnectionReconfiguration message used to perform handover is contained for handoverCommandMessage in LTE case. HandoverCommand-r8-IEs ::= SEQUENCE { handoverCommandMessage OCTET STRING (CONTAINING DL-DCCH-Message), nonCriticalExtension SEQUENCE {} OPTIONAL } It is not helpful for the network because the transparent transfer of handover message from source node is not possible in this case i.e. NR SA handover. Source node should make DL-DCCH-Message header and add it to the RRCReconfiguration message which is received from the target node, and then send this DL-DCCH-Message to UE. In addition, RAN2 already approved the below changes for MobilityFromNRCommand in the latest phase since Rel-15 specification was freezing. MobilityFromNRCommand-IEs ::= SEQUENCE { targetRAT-Type ENUMERATED { eutra, spare3, spare2, spare1, ...}, targetRAT-MessageContainer OCTET STRING, nas-SecurityParamFromNR OCTET STRING OPTIONAL, -- Cond HO-ToEPC lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } NOTE 1: The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below: targetRAT-Type; Standard to apply; targetRAT-MessageContainer. eutra; TS 36.331 [10] (clause 5.4.2); DL-DCCH-Message including the RRCConnectionReconfiguration. |
| Summary of change: |  | This CR includes the following changes: - Change the containing message of handoverCommandMessage from RRCReconfiguration to DL-DCCH-Message. Impact analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: Message type change for handoverCommandMessage Inter-operability: No interoperability issue. CR does not affect UE i.e. merely changes network (inter-node signalling) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transparent tranfer of handoverCommandMessage from source node is not possible i.e. source node should add the header of DL-DCCH-Message after receiving handoverCommandMessage from the target node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### –11.2.2	Message definitions

#### –	HandoverCommand

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN.

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=                 SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

handoverCommand                     HandoverCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverCommand-IEs ::=             SEQUENCE {

handoverCommandMessage              OCTET STRING (CONTAINING DL-DCCH-Message),

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the entire DL-DCCH-Message including the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB.

END OF FIRST CHANGE