3GPP TSG-RAN WG2 #108	 TDoc R2-1915275

# Reno, USA, 18th – 22th November 2019

Agenda Item:	6.4.6

Source:	Ericsson

Title:	Discussion on NR SL QoS management

Document for:	Discussion, Decision

# Introduction

In this paper, we discuss the issues related to NR SL QoS management taking into account the agreed NR SL QoS framework.

# Discussion

## Congestion control

In LTE SL, the level of congestion is measured by channel busy radio (CBR). Each LTE SL packet is associated with a ProSe per packet priority (PPPP) value and each PPPP is associated with an upper CBR threshold and a lower CBR threshold.

When a packet of priority PPPP arrives, UE needs to select a carrier for transmission. For initial selection, one carrier is regarded as a candidate if the CBR of the carrier is below threshCBR-FreqReselection associated with the priority. In case of multiple candidate carriers, UE selects the one with the lowest CBR.

Carrier reselection will be triggered if CBR of the current carrier exceeds the threshCBR-FreqReselection associated with the priority. UE may reselect the carrier from other candidate carriers. If there is no other candidate carrier available, UE will start dropping the packets of associated priority until the CBR of the current carrier is below the corresponding threshCBR-FreqKeeping or other candidate carriers appear.

In LTE SL, congestion control balances the load among multiple carriers using CBR thresholds associated with different PPPPs, i.e. UE may transmit the packet of certain PPPP only if the CBR of the current carrier is below the corresponding threshold.

It has been agreed in RAN1#97 meeting that LTE V2X sidelink congestion control is the starting point of defining NR sidelink congestion control. On the other hand, in the latest NR SL QoS framework design, PC5 5QI (PQI) value, instead of PPPP/PPPR, is used to represent QoS requirements for a SL QoS flow. Similar as in NR Uu, PQI is an index which maps to a set of QoS parameters such as priority, latency, packet error rate etc. When a SL QoS flow is mapped to a SLRB at access stratum, the priority parameter of a flow is mapped to the priority of a SLRB/LCH. At MAC layer, UE only considers LCH priority and starvation condition to decide which LCH data should be transmitted first.

RAN1 #97 Agreements: LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control. TS 23.287: 5.4.1 QoS handling for V2X communication over PC5 reference point For NR based PC5, a QoS model similar to that defined in TS 23.501 [6] for Uu reference point is used, i.e. based on 5QIs, with additional parameter of Range. For the V2X communication over PC5 reference point, a special set of standardized PC5 5QIs (PQI) are defined in clause 5.4.3. The UE may be configured with a set of default PQIs to use for the V2X services, as defined in clause 5.1.2.1. RAN2 105 Agreements: 2: From RAN2 perspective, PQI defined by SA2 for NR SL is feasible. Final decision on whether/how other QoS parameters are defined in addition to PQI is up to SA2. RAN2 106 Agreements on LCH priority: 1: For unicast for IC connected UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by dedicated RRC). 2: For unicast for IC idle/inactive UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by SIB). 3: For unicast for OOC UE, logical channel priority level is configured by NW. Mapping between PQI/PFI, LCH and SLRB is also configured by NW (e.g. by preconfiguration).

LTE SL congestion control is taken as the baseline for NR SL. However, NR SL adopts different QoS framework where LCH is the finest granularity to distinguish different QoS requirement at access stratum.

In LTE, for each carrier the mapping between CBR thresholds and PPPP values is preconfigured or configured by eNB via SIB/RRC signalling, i.e. SL-V2X-FreqSelectoinConfigList. Then, the same PPPP to CBR mapping will be adopted by all SL LCHs. In NR, a different principle can be adopted. For a certain SL LCH, the CBR threshold(s) used for congestion control is configured by the NW together with the SL LCH configuration signaling in a per SL LCH manner (instead of providing a priority to CBR thresholds mapping which will be applied to all LCHs in the UE as in the legacy LTE system). The CBR threshold information can be delivered via dedicated RRC message when RRC_CONNECTED UE requests SLRB configuration from NW, or via SIB message to UEs in RRC_IDLE/INACTIVE mode.

Besides, we believe it is beneficial to configure CBR threshold in a per SL LCH manner from QoS management point of view. In LTE SL, packet priority value PPPP is used for two purposes: 1. For congestion control, when congestion level is high, packets of lower priority should be dropped first (i.e., meaning the service is less important like non-safety related); 2. During LCP procedure, packets of higher priority should be transmitted first (i.e., meaning the service has low latency requirement). However, a service which is more important than others (e.g., safety related services) does not necessarily require low latency. Therefore, by configuring CBR threshold in a per SL LCH manner decouples the relationship between LCH priority and the service importance. SL LCHs of the same priority value can be configured with same or different CBR thresholds.

The CBR threshold(s) used for congestion control should be preconfigured or configured by gNB via SIB/RRC signalling, together with the SL LCH configuration signaling in a per SL LCH manner.

SL LCHs of the same priority value can be configured with same or different CBR thresholds.

## Starvation avoidance

RAN2 106 Agreements: 4: Uu like starvation avoidance mechanism is applied to LCP.

It is already agreed in RAN2 106 agreements to support Uu like starvation avoidance mechanism in NR SL. In NR Uu, given an uplink grant, UE has to decide to transmit data from which logical channel first. Generally, a LCH associated with higher priority value should be transmitted first. On the other hand, in many cases, grant size allocated by the base station within one instance of the LCP procedure is limited, and the available uplink grant cannot hold the data from all logical channels. Then low priority data has to wait for the next transmission opportunity. In the next instance of the LCP procedure, the high priority logical channel is still served first. Hence the low priority logical channel may not be serviced for a long time, which causes starvation.

Starvation avoidance mechanism for NR Uu is described in TS38.321, a variable Bj is maintained for each LCH j and initially set to zero. Bj is incremented by the product prioritisedBitRate (PBR) × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented.

When a new transmission is performed, only LCHs with Bj > 0 are allocated resources in a decreasing priority order, and decrement Bj by the total size of MAC SDUs served to LCH j (Bj can be negative after this step). If any resource remains, all the LCHs are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first.

In NR sidelink, since some services have stringent data rate requirement, guaranteed bit rate (GBR) flow is supported. When there are multiple GBR SL flows mapped to LCHs of different priorities, LCH of higher priority may always take the resource while data rate of lower priority LCH cannot be fulfilled. Therefore, starvation avoidance mechanism similar as in NR Uu should be supported.

Starvation avoidance mechanism in NR Uu can be taken as the baseline for NR SL starvation avoidance.

In NR SL, starvation avoidance mechanism will coexist with congestion control mechanism.

On the other hand, following the legacy procedure, the UE will continue updating the starvation avoidance related variables when SL LCH transmission suspends due to congestion control, i.e. measured CBR is above the CBR threshold of a certain SL LCH. As a consequence, when the LCH re-start transmitting due to reduced congestion, it is regarded as “super hungry” with large starvation state variable (i.e. Bj). This might impact other non-starved LCH transmission of higher priority.

In case of congestion, the starvation timer (i.e. T) will stop counting and the starvation state variable (i.e. Bj) is reset to be zero and stop updating.

## PC5 Interface availability

In the NR V2X WID, the discussion on interface and RAT selection is limited to PC5 availability/unavailability for unicast.

RP-190766 WID on 5G V2X with NR sidelink - Sidelink L2/L3 protocols and signalling - Support of sidelink transmission and reception in RRC, MAC, RLC, PDCP, and SDAP [RAN2] - AS level link management for unicast [RAN2, RAN1] - Define the criteria of PC5 availability/unavailability for unicast based on this functionality.

In our view, and according to the WID, it is needed to at least specify the criteria for different UEs to determine the interface availability/unavailability. Such criteria ensure certain radio conditions before the UE starts accessing the interface. Thus, suitable criteria increase the ratio of successful access and transmission. Secondly, if such criteria are up to UE implementation, it might be the case that under the same radio condition UEs using less stringent criteria will start the transmission while other UEs with more stringent criteria will not.

Specifying criteria to determine PC5 interface availability/unavailability ensure a unified channel access framework across different UEs.

Of course, the criteria to determine interface availability/unavailability can vary from fundamental to very stringent. It may be wise to take fundamental criteria as a baseline. For instance, the PC5 interface availability/unavailability can be determined according to if a SL transmitting pool is (pre)configured on the frequency(ies) in which a V2X service shall be transmitted, and if the measured CBR of the transmitting pool is above/below a threshold value.

In our understanding, the determination of the PC5 interface availability/unavailability can be per-service and may lead to the corresponding SL QoS flow/SLRB establishment and release.

However, the cross-layer interaction, i.e. how the access stratum signals to upper layers, can be left to UE implementation.

The criterion to determine PC5 interface availability/unavailability shall be specified as per work item description.

PC5 interface availability/unavailability is determined for each service.

The PC5 interface is available if a SL transmitting pool is (pre)configured and if the measured CBR of the transmitting pool is above a threshold (the threshold can be different for different QoS requirements). Otherwise the PC5 interface is not available.

The determination of the PC5 interface availability/unavailability may trigger a corresponding SL QoS flow/ SLRB establishment/release.

It is up to UE implementation how/when the access stratum signals the interface availability/unavailability to upper layers.

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE SL, congestion control balances the load among multiple carriers using CBR thresholds associated with different PPPPs, i.e. UE may transmit the packet of certain PPPP only if the CBR of the current carrier is below the corresponding threshold.

Observation 2	LTE SL congestion control is taken as the baseline for NR SL. However, NR SL adopts different QoS framework where LCH is the finest granularity to distinguish different QoS requirement at access stratum.

Observation 3	In NR SL, starvation avoidance mechanism will coexist with congestion control mechanism.

Observation 4	Specifying criteria to determine PC5 interface availability/unavailability ensure a unified channel access framework across different UEs.

Based on the discussion in section 2 we propose the following:

Proposal 1	The CBR threshold(s) used for congestion control should be preconfigured or configured by gNB via SIB/RRC signalling, together with the SL LCH configuration signaling in a per SL LCH manner.

Proposal 2	SL LCHs of the same priority value can be configured with same or different CBR thresholds.

Proposal 3	Starvation avoidance mechanism in NR Uu can be taken as the baseline for NR SL starvation avoidance.

Proposal 4	In case of congestion, the starvation timer (i.e. T) will stop counting and the starvation state variable (i.e. Bj) is reset to be zero and stop updating.

Proposal 5	The criterion to determine PC5 interface availability/unavailability shall be specified as per work item description.

Proposal 6	PC5 interface availability/unavailability is determined for each service.

Proposal 7	The PC5 interface is available if a SL transmitting pool is (pre)configured and if the measured CBR of the transmitting pool is above a threshold (the threshold can be different for different QoS requirements). Otherwise the PC5 interface is not available.

Proposal 8	The determination of the PC5 interface availability/unavailability may trigger a corresponding SL QoS flow/ SLRB establishment/release.

Proposal 9	It is up to UE implementation how/when the access stratum signals the interface availability/unavailability to upper layers.

# References