3GPP TSG-RAN WG2 #108	Tdoc R2-1915398

Reno, USA, 18th – 22nd November 2019

Agenda Item:	7.1.8

Source:	Ericsson

Title:	UE capabilities for enhancements for non-BL UEs

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 Work Item (WI) on “Additional MTC enhancements for LTE” [1] has the following as one of its objectives for BL/CE UEs:

Extreme coverage for non-BL UEs:

- Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4]

  - Enhancements to idle mode mobility

  - UE demodulation performance requirements for 2 RX antennas and full duplex FDD

  - Dual layer DL reception

  - Feedback based on CSI-RS

  - ETWS/CMAS in connected mode

These improvements have been discussed both in RAN1 and RAN2, and RAN1 has provided a RAN1 UE feature list [2] for the features that initially are considered to be included in 3GPP Release 16. At least the highlighted improvements will be supported in Rel-16.

In this contribution we provide a text proposal for supporting UE capabilities for the features related to non-BL UE enhancements of this Work Item.

# 2	Discussion

As shown in the RAN1 UE feature list, the UE capabilities to be supported that are related to non-BL UE enhancement objective are CSI-RS-based feedback for non-BL UE for CE Mode A and ETWS/CMAS indication in connected mode for non-BL UE for CE Mode A and CE Mode B. Note that these features only apply to non-BL UEs in CE. Below can be found our text proposal for implementation of these capabilities to TS 36.306.

RAN2 adopts the text proposal to TS 36.306 below regarding UE capabilities related to non-BL UE enhancements based on RAN1 UE feature list.

# 3	Text proposal

TS 36.306:

First change

#### 4.3.4.x	ce-ETWS-CMAS-reception-CE-ModeA-r16

This field indicates whether the UE supports reception of ETWS and/or CMAS indication while in RRC_CONNECTED in CE Mode A, as specified in TS 36.XXX [XX]. A UE indicating support of ce-ETWS-CMAS-reception-CE-ModeA-r16 shall also indicate support of ce-ModeA-r13. This capability can be supported only by a non-BL UE.

#### 4.3.4.y	ce-ETWS-CMAS-reception-CE-ModeB-r16

This field indicates whether the UE supports reception of ETWS and/or CMAS indication while in RRC_CONNECTED in CE Mode B, as specified in TS 36.XXX [XX]. A UE indicating support of ce-ETWS-CMAS-reception-CE-ModeB-r16 shall also indicate support of ce-ModeB-r13. This capability can be supported only by a non-BL UE.

Next change

#### 4.3.4.z	csi-RS-Feedback-CE-ModeA-r16

This field indicates whether the UE supports CSI-RS based feedback in CE Mode A, as specified in TS 36.XXX [XX]. A UE indicating support of csi-RS-Feedback-CE-ModeA-r16 shall also indicate support of ce-ModeA-r13. This capability can be supported only by a non-BL UE.

End of changes

# 4	Conclusion

In this contribution we propose the following:

Proposal 1	RAN2 adopts the text proposal to TS 36.306 below regarding UE capabilities related to non-BL UE enhancements based on RAN1 UE feature list.

# References

RP-191356, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN#84, Newport Beach, USA, June 2019.

R1-1911574, “Summary of RRC parameters for L1 configuration for Rel-16 LTE-MTC”, Ericsson, RAN1#98bis, Chongqing, China, October 2019.