3GPP TSG-RAN WG2 #108	Tdoc R2-1915436

Reno, USA, 18th November – 22nd November 2019

Agenda Item:	6.12.2

Source:	Ericsson

Title:	Open issues of CEF report

Document for:	Discussion, Decision

# Introduction

After the RAN2#107bis meeting, it was agreed to have a running CR related email discussion on MDT.

[107#xx][NR/MDT] running 331 CR for introducing MDT (Huawei)

Capture agreements from this meeting

Including stage3 details event triggered MDT

Intended outcome: endorsed running CR

Deadline: Thursday 07/11/2019

The running CR also captures the progress made on the CEF report. However, there are still some open issues associated to the CEF report and we are addressing the same in this contribution.

# Discussion

As the outcome of the SI phase, the approved TR, 37.816 captured the wanted solution for the CEF report. The following aspects were covered as part of CEF report.

- The information included in LTE RLF and CEF reporting is taken as baseline for 5G NR.

- The use cases of NR accessibility measurements includes:

  - The UE fails to send RRCSetupRequest, i.e. when timer T300 expires

  - The UE fails to send RRCResumeRequest/RRCResumeRequest1, i.e. when timer T319 expires

- To address Coverage Optimisation use case, RLF report and accessibility measurements are enhanced with:

  - NR RLF Report can indicate the information that enable to differentiate DL and UL availability after RLF occurrence. NR RLF report is extended with a flag, where at least “no suitable cell is found” is indicated. There is no additional UE measurement requirement for this. When to set the flag can be discussed in WI phase.

  - NR CEF Report is enhanced with further information elements expressing the number of failed connection setup attempts after RLF at least including the number and available location information. NR CEF Report is extended with “Number of connection failures per cell” field. The UE counts the number of CEFs that it has experienced within the last 48 hours.

  - RACH failure information, if available, shall be included in both RLF report and CEF report. Attempted SSB index can be indicated as part of RACH failure information.

  - Include SSB related information consisting of SSB index and number of preambles sent for each tried SSB in the RACH information report.

  - SS Block index, CSI-RS index for both of serving and neighbouring cells could be included in the NR RLF report.

  - The mechanism of RRC resume failure reporting takes CEF reporting as baseline.

  - The information included in the NR RRC resume failure reporting takes the information included in the 5G NR CEF reporting as baseline.

  - CSI-RS index and the corresponding number of preambles sent for each tried beam carrying CSI-RS index can be included in the NR RLF report, if it is RACH procedure failure leading to the RLF.

  - SSB index of the downlink beams of both serving cell and neighbour cells, and SUL/NUL carrier information can be included in the NR CEF reporting.

- for WLAN/Bluetooth measurement collection: procedure for LTE WLAN/Bluetooth measurement collection is taken as baseline. The procedure baseline in TS 36.331 includes:

  - Supporting WLAN/Bluetooth measurement collection for logged MDT, immediate MDT, RLF report and accessibility measurements.

  - Supporting WLAN and Bluetooth name list configuration in order to only collect the measurements of operator deployed WLAN and Bluetooth.

  - Supporting reporting WLAN ID, RSSI, RTT for WLAN measurements and reporting Bluetooth address, RSSI for Bluetooth measurement.

The highlighted text discusses the RACH failure information. If the UE declares CEF due to being unable to access the cell within a certain duration, it is an indication that the UE had some RACH issues. In the meanwhile, the UE anyway collects the RACH information associated to the RACH report when the performs connection establishment attempt. Upon failure, the UE can store the current contents of the RACH report which is of similar format as that of the normal RACH report. This includes amongst other things, the RACH accesses in the chronological order, whether the SSB was above the threshold or not etc.

While performing the random access for the connection establishment or connection resume, the UE anyway stores the random access related measurements as part of the RACH report so that it can send such a RACH report if the connection establishment or resume is successful.

Harmonize the contents of RACH failure information to be included in the CEF report with the actual contents of the RACH report.

In the running CR the location information is missing which needs to be included as well.

Location information (both detailed location and WLAN-Bluetooth based measurement) is included in the CEF report, if available.

# Conclusion

In this contribution, the following observations were made:

Observation 1	While performing the random access for the connection establishment or connection resume, the UE anyway stores the random access related measurements as part of the RACH report so that it can send such a RACH report if the connection establishment or resume is successful.

In this contribution, the following proposals were captured:

Proposal 1	Harmonize the contents of RACH failure information to be included in the CEF report with the actual contents of the RACH report.

Proposal 2	Location information (both detailed location and WLAN-Bluetooth based measurement) is included in the CEF report, if available.

# References