3GPP TSG-RAN WG2 Meeting #108	R2-1915645

Reno, USA, November 18th – 22nd, 2019

Title:	Draft LS on S-KgNB/KSN Derivation in Conditional PSCell Addition/Change

Response to:	-

Release:	Release 16

Work Item:	NR_Mob_enh-Core, LTE_feMob-Core

Source:	Futurewei (to be TSG RAN WG2)

To:	TSG SA WG3

Cc:	TSG RAN WG3

Contact Person:

#### Name:	Hao Bi

E-mail Address:	Hao dot Bi at Futurewei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In conventional PSCell addition/change, the secondary key (S-KgNB or S-KeNB) is derived or updated at UE upon the reception of sk-Counter in RRCReconfiguration/RRCConnectionReconfiguration message as specified in TS 38.331.

However, with conditional PSCell addition/change, UE may not perform PSCell addition/change after receiving RRCReconfiguration/RRCConnectionReconfiguration message with sk-Counter. The conditional PSCell addition/change may occur in the future when the configured condition is met. And a conditional PSCell addition/change may never happen if conditional PSCell addition/change to another candidate PSCell or a conventional PSCell addition/change is executed in advance.

Futhermore, UE may receive multiple RRCReconfiguration/RRCConnectionReconfiguration messages with sk-Counter for different candidate PSCells before a conditional or conventional PSCell addition/change is executed.

2. Actions:

To SA3 group.

ACTION: 	RAN2 respectfully asks SA3 to take into account the above information on Conditional PSCell Addition/Change.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109		February 24 – 28, 2020	Athens, Greece

3GPP RAN2#109bis		April 20 – 24, 2020		TBD, Japan