3GPP TSG-RAN WG2#108	R2-1915652

Reno, NV, USA, 18th – 22nd November 2019

Agenda Item:	6.8.2.1

Source:	Ericsson

Title:	LPP, RRC and UE Rx-Tx Aspects of NR RAT dependent positioning

Document for:	Discussion, Decision

# Introduction

In RAN1#98, an LS was sent to RAN2 to inform about agreed reference signals, measurements and also some relation to positioning techniques for which the reference signals and associated measurements are intended to facilitate. The positioning techniques are indicated for information purpose as examples and do not intend to influence higher layer signalling to be developed by RAN2. The tables below from the LS provide mappings between reference signals and gNB measurements based on the latest status of the RAN1 discussion.

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | E-CID |

Furthermore, RAN1 has summarized the NR positioning parameter status in [1]. In this contribution, we discuss the signalling design related to the RAN1 parameters and NR positioning support in Rel. 16 in general.

# Discussion

Two fundamental discussions regarding the introduction of Rel. 16 NR positioning support in stage 3 RAN specification concerns

- Signalling design in LPP, especially whether the NR positioning support shall be handled per positioning method or via a general NR positioning measurements structure, but also capturing the NR DL-PRS details

- Signalling design in RRC to capture the Rel. 16 UL-SRS for positioning details

We will discuss these two concerns in the following two subsections and include text proposals in separate appendices.

## LPP Signalling Design

As for previous RATs, the ECID positioning method concerns measurements that are available due to other configured procedures and mechanisms, mainly for radio resource management. These measurements har configured via RRC and can either be reported via RRC or via LPP.

The existing ECID positioning method can be enhanced to include also support for NR measurements.

Enhance ECID Positioning subsection of the Positioning Method IEs section in LPP to introduce support for Rel-16 NR ECID positioning

The NR RRC measurement reporting structure for measurement results is rather different compared to the LTE RRC measurement reporting structure.

The LTE RRC and NR RRC measurement result reporting structures are significantly different

Therefore, it is not relevant to enhance existing information elements to support NR RRM measurements. Instead, we propose to introduce a new information element ECID-SignalMeasurementInformationNR in LPP to handle NR measurements made available due to NR RRC configuration.

Introduce a new information element to represent the NR ECID measurements

The NR RRC measurement scope also includes measurements per beam, associated to either SSB or CSI-RS. It also includes a more comprehensive cell global information. Furthermore, SFTD has been introduced to enable reporting of timing differences, which also is related to positioning. Moreover, UE Rx-Tx based on RRC-configured signals should also be reported as part of LPP NR ECID.

Introduce LPP NR ECID reporting of Rel-15 NR measurements, including measurements per beam associated to either SSB or CSI-RS, CGI and SFTD.

At RAN2#107bis, there was a comprehensive discussion about whether the Rel 16 NR positioning support shall be introduced by method or by a general positioning measurement structure. There were some concerns whether it was possible to introduce a general structure that is flexible enough to accommodate all agreements. A proposal in provided in the enclosed text proposal.

It is indeed possible to define a generic positioning measurements-based structure

The generic structure in the text proposal is compact and flexible

Given that the main concern that was raised about feasibility and that the proposed measurement based structure is compact and flexible, we have the following proposals.

Introduce a flexible NR positioning measurement structure to accommodate Rel. 16 NR positioning support.

Introduce the subsection structure in Section 6.5.8 of the text proposal in LPP.

The final representation of DL-PRS needs to await the final RAN1 agreements. However, some things can be discussed already. For example, the hierarchy of the frequency layer, DL-PRS and TRP assistance data could be arranged as follows:

FrequencyLayers (1..maxNoOfFreqLayers) All frequency layer parameters TRPs (1..maxNoOfTRPs) ResourceSets (1..maxNoOfResourceSetsPerTRP) FrequencyLayerIndex ResourceSetID Resources (1..maxNoOfResPerResSet) ResourceID DL PRS SequenceID Etc Static TRP Info (1..maxNoOfTRPs) TRP location TRP beam configuration Dynamic TRP Info (1..maxNoOfTRPs) TRP relative time differences

From the hierarchy proposal, we derive the following proposals

Introduce a frequency layer link per DL PRS resource set.

Introduce one list of TRPs, where the first TRP in the list is considered as the reference TRP in the context where this matters

The TRP index of the TRP list is enough reference to a specific TRP, since the TRPs are only well-defined with the assistance data. No long TRP ID attribute is needed, an index (1..no of listed TRPs) is enough.

Introduce separate lists with static and dynamic TRP information to enable a flexible handling of UE-based assistance data via broadcast, where it can be relevant to have different update rates for the static and dynamic parts

For compact measurement reporting it is relevant to enable a configuration where the timing measurements are aggregated per TRP. For clarity, it is important to define the aggregation methods, such as

- Strongest DL-PRS resource. Among all configured DL PRS resources of a TRP, the UE selects and reports the timing information associated to the DL PRS resource corresponding to the strongest DL-PRS RSRP

- DL PRS resource corresponding to the shortest Time of Flight. Among the configured resources of a TRP, the UE selects and reports the timing information associated to the DL PRS resource corresponding to the shortest ToF. This is the DL PRS resource that most likeliy corresponds to line of sight.

Introduce timing measurement aggregation per TRP based on either strongest DL PRS resource of the DL PRS resource corresponding to the shortest time of flight.

Assume that there are N1 resources defined for resource set 1, and N2 resources defined for resource set 2. When reporting relative time difference measurements between resources of two resource sets, two alternative representations have been discussed in RAN1:

- Report relative time differences between all possible pairs of resources from the two resource sets. This corresponds to N1*N2 measurements of RSTD

- Define a reference resource per TRP. Report relative time difference between the reference resource of each resource set, and report the relative time difference between resources of the same resource set. This corresponds to 1 + (N1-1) + (N2-1) = N1 + N2 – 1 measurements of RSTD, where the intra resource set RSTDs also could be represented with fewer bits than the RSTD between reference resources of the resource sets.

Clearly, the second alternative is more efficient.

Define a reference resource per TRP. The UE can use a different reference resource compared to what the location server has proposed.

Represent relative time measurements between reference resources between resource sets and between resources with the resource sets

## RRC Signalling Design

RAN1 has sent parameter list for UL SRS configuration and further recommended that:

- RAN1 assumes that the SRS for positioning configuration will be included in the RRC configuration of a UE and that it is up to RAN2 and RAN3 WG’s scope to analyze further the system level aspects along with any further work on the design of higher layer signaling.

From RAN2 perspective, we do not see any deviation or motivation to change this. If LPP and RRC happens to configure UL SRS simultaneously, UE may not be able to fulfil both the requirements.

Further, one can reuse the existing SRS-Config for Positioning purpose.

An example ASN.1 is provided below.

Based upon the RAN1 table the parameters that has clear impact as compared to legacy UL SRS configurations are for below two; others are still FFS:

a) transmissionComb

For positioning, the SRS comb size set is extended from {2,4} to {2,4,8}

b) resourceMapping startPosition, resourceMapping nrofSymbols

For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12}

SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources. The network triggers the transmission of the set of SRS-Resources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

- SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId    OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet      OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId          OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource            OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}     OPTIONAL,   -- Need S

... ,

srs-ResourceToAddModList-r16            SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource-v16xy      OPTIONAL   -- Need N

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                     OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList-v1530      SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)  OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                       OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                          OPTIONAL, -- Cond Setup

pathlossReferenceRS                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

}                                                                                                   OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                OPTIONAL, -- Need S

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                       OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                 OPTIONAL,   -- Need R

...

}

SRS-Resource-v16xy ::=                     SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }      OPTIONAL,   -- Need R

transmissionComb-r16                    CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

},

n8                                      SEQUENCE {

combOffset-n8                           INTEGER (0..7),

cyclicShift-n8                          INTEGER (0..5)

},

},

resourceMapping-r16                       SEQUENCE {

startPosition                           INTEGER (0..13),

nrofSymbols                             ENUMERATED {n1, n2, n4, n8, n12},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                 OPTIONAL,   -- Need R

...

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                               OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

Existing SRS-Config is taken as baseline for Positioning purpose.

## UE Rx-Tx

A high precision serving cell RTT measurement is very valuable. It can be used for ECID or combined with angle of arrival measurements to achieve accurate single cell positioning. This could be done without implementing a PRS and without any coordination among network nodes. We therefore propose that the UE Rx-Tx time difference measurement in the serving cell should be supported based on the TRS in the DL and the SRS in the uplink. To allow a very simple implementation with minimal network signaling we propose that the UE Rx-Tx time difference measurement in the serving cell is configured by RRC. No new signaling is needed to configure the TRS since the TRS is already today configured by RRC for communication purposes. The simulation results show comparable performance with PRS.

The UE Rx-Tx time difference measurement in the serving cell should be supported based on Rel-15 signals TRS in the DL and the SRS in the uplink.

## Correcting RTT Errors

According to section 7.1 on UE transmit timing in TS 38.133 the UE performs autonomous timing adjustments. The size of these adjustments can be up to 5.5*64*Tc  = 179ns = 54m/c in FR1 and up to 2.5*64*Tc = 81ns = 24m/c in FR2. If such timing adjustments are performed between the reception of a DL PRS utilized by the UE for UE Rx-Tx TD measurement and the transmission of an UL SRS utilized by the gNB for a gNB Rx-Tx TD measurement this will cause a serious error in the resulting RTT measurement as illustrated in below figure. This could easily be solved by the UE adjusting the reported UE Rx-Tx TD measurement accordingly but this would require that the UE knows which UL SRS occasion the gNB will use for it’s measurement or equivalently the timing of this UL SRS occasion. We therefore propose that the UE Rx-Tx TD measurement configuration includes the ID of the UL SRS to be used by the UE to define it’s TX time framing to be used for the UE Rx-Tx TD measurement and a timestamp giving the slot level position of the DL PRS used for the measurement.

The UE Rx-Tx TD measurement configuration includes the ID of the UL SRS to be used by the UE to define it’s TX time framing to be used for the UE Rx-Tx TD measurement and a timestamp giving the slot level position of the DL PRS used for the measurement.

Figure: If the UE performs autonomous timing adjustments between the reception of the reference signal used by the UE to estimate the rx frame timing for the UE Rx-Tx time difference measurement and the transmission of the reference signal used by the gNB to estimate the rx frame timing for the gNB Rx-Tx time difference measurement, then the timing adjustment will cause a serious error in the RTT estimate

Moreover, different types of timing adjustments can occur during the measurement time window which will introduce UE Rx-Tx measurement errors unless such adjustments during the measurement time window are either prohibited, compensated for or informed about.

RAN2 shall introduce some mechanism to mitigate effects from timing adjustments during measurement time windows for UE Rx-Tx measurements

# Conclusion

In the previous sections we made the following observations:

Observation 1	The LTE RRC and NR RRC measurement result reporting structures are significantly different

Observation 2	It is indeed possible to define a generic positioning measurements-based structure

Observation 3	The generic structure in the text proposal is compact and flexible

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhance ECID Positioning subsection of the Positioning Method IEs section in LPP to introduce support for Rel-16 NR ECID positioning

Proposal 2	Introduce a new information element to represent the NR ECID measurements

Proposal 3	Introduce LPP NR ECID reporting of Rel-15 NR measurements, including measurements per beam associated to either SSB or CSI-RS, CGI and SFTD.

Proposal 4	Introduce a flexible NR positioning measurement structure to accommodate Rel. 16 NR positioning support.

Proposal 5	Introduce the subsection structure in Section 6.5.8 of the text proposal in LPP.

Proposal 6	Introduce a frequency layer link per DL PRS resource set.

Proposal 7	Introduce one list of TRPs, where the first TRP in the list is considered as the reference TRP in the context where this matters

Proposal 8	The TRP index of the TRP list is enough reference to a specific TRP, since the TRPs are only well-defined with the assistance data. No long TRP ID attribute is needed, an index (1..no of listed TRPs) is enough.

Proposal 9	Introduce separate lists with static and dynamic TRP information to enable a flexible handling of UE-based assistance data via broadcast, where it can be relevant to have different update rates for the static and dynamic parts

Proposal 10	Introduce timing measurement aggregation per TRP based on either strongest DL PRS resource of the DL PRS resource corresponding to the shortest time of flight.

Proposal 11	Define a reference resource per TRP. The UE can use a different reference resource compared to what the location server has proposed.

Proposal 12	Represent relative time measurements between reference resources between resource sets and between resources with the resource sets

Proposal 13	Existing SRS-Config is taken as baseline for Positioning purpose.

Proposal 14	The UE Rx-Tx time difference measurement in the serving cell should be supported based on Rel-15 signals TRS in the DL and the SRS in the uplink.

Proposal 15	The UE Rx-Tx TD measurement configuration includes the ID of the UL SRS to be used by the UE to define it’s TX time framing to be used for the UE Rx-Tx TD measurement and a timestamp giving the slot level position of the DL PRS used for the measurement.

Proposal 16	RAN2 shall introduce some mechanism to mitigate effects from timing adjustments during measurement time windows for UE Rx-Tx measurements

# References

R1-1911564, Update to the list of higher layer parameters for NR Positioning (RAN1-98bis), Intel

3GPP TSG-RAN2 Meeting #108	R2-19xxxxx

# Reno, USA, 18th – 22nd November 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | DRAFT | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running LPP CR NR Positioning Support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducing support for Rel. 16 NR positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.305, 38.331, 38.455 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0)", December 2013.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[xx]	3GPP TS 38.305: "User Equipment (UE) positioning in NG-RAN".

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.305 [2] and TS 23.271 [3] apply. Other definitions are provided below.

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Location Server: a physical or logical entity (e.g., E-SMLC or SUPL SLP) that manages positioning for a target device by obtaining measurements and other location information from one or more positioning units and providing assistance data to positioning units to help determine this. A Location Server may also compute or verify the final location estimate.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

Reference Source: a physical entity or part of a physical entity that provides signals (e.g., RF, acoustic, infra-red) that can be measured (e.g., by a Target Device) in order to obtain the location of a Target Device.

Target Device: the device that is being positioned (e.g., UE or SUPL SET).

Transmission Point (TP): A set of geographically co-located transmit antennas for one cell, part of one cell or one PRS-only TP. Transmission Points can include base station (eNodeB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a PRS-only TP, etc. One cell can be formed by one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Observed Time Difference Of Arrival (OTDOA): The time interval that is observed by a target device between the reception of downlink signals from two different TPs. If a signal from TP 1 is received at the moment t1, and a signal from TP 2 is received at the moment t2, the OTDOA is t2 – t1.

PRS-only TP: A TP which only transmits PRS signals for PRS-based TBS positioning and is not associated with a cell.

Relative Time Difference (RTD): The relative time difference between two TRPs in terms of the well defined time reference start of a downlink radio frame.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ADR	Accumulated Delta-Range

A-GNSS	Assisted-GNSS

AP	Access Point

ARFCN	Absolute Radio Frequency Channel Number

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

BTS	Base Transceiver Station (GERAN)

CID	Cell-ID (positioning method)

CNAV	Civil Navigation

CRS	Cell-specific Reference Signals

DL-AOD	Downlink Angle Of Departure

DL-TDOA	Downlink Time Difference Of Arrival

ECEF	Earth-Centered, Earth-Fixed

ECGI	Evolved Cell Global Identifier

ECI	Earth-Centered-Inertial

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EOP	Earth Orientation Parameters

EPDU	External Protocol Data Unit

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

FKP	(German) Flächen-Korrektur-Parameter (area correction parameter)

FTA	Fine Time Assistance

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

HA GNSS	High-Accuracy GNSS (RTK, PPP)

ICD	Interface Control Document

IGS	International GNSS Service

IOD	Issue of Data

IS	Interface Specification

LLA	Latitude Longitude Altitude

LMF	Location Management Function

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LSB	Least Significant Bit

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MSAS	Multi-functional Satellite Augmentation System

MSB	Most Significant Bit

msd	mean solar day

MT-LR	Mobile Terminated Location Request

NAV	Navigation

NB-IoT	NarrowBand Internet of Things

NCGI	NR Cell Global Identifier

NICT	National Institute of Information and Communications Technology

NI-LR	Network Induced Location Request

NPRS	Narrowband Positioning Reference Signals

NR	NR Radio Access

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NTSC	National Time Service Center of Chinese Academy of Sciences

OSR	Observation Space Representation

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

PPP	Precise Point Positioning

PRB	Physical Resource Block

PRC	Pseudo-Range Correction

PRS	Positioning Reference Signals

posSIB	Positioning System Information Block

PZ-90	Parametry Zemli 1990 Goda – Parameters of the Earth Year 1990

QZS	Quasi Zenith Satellite

QZSS	Quasi-Zenith Satellite System

QZST	Quasi-Zenith System Time

RF	Radio Frequency

RRC	Range-Rate Correction

Radio Resource Control

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

RTT	Round Trip Time

RU	Russia

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SFN	System Frame Number

SLP	SUPL Location Platform

SSID	Service Set Identifier

SSR	State Space Representation

SUPL	Secure User Plane Location

SV	Space Vehicle

TB	Terrestrial Beacon

TBS	Terrestrial Beacon System

TLM	Telemetry

TOA	Time Of Arrival

TOD	Time Of Day

TOW	Time Of Week

TP	Transmission Point

UDRE	User Differential Range Error

ULP	User Plane Location Protocol

USNO	US Naval Observatory

UT1	Universal Time No.1

UTC	Coordinated Universal Time

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

# 4 Functionality of Protocol

## 4.1	General

### 4.1.1	LPP Configuration

LPP is used point-to-point between a location server (E-SMLC, LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources. Figure 4.1.1-1 shows the configuration as applied to the control- and user-plane location solutions for E-UTRAN (as defined in TS 36.305 [2] and TS 23.271 [3]).

NB-IoT is a non-backward compatible variant of E-UTRAN supporting a reduced set of functionalities. In this specification, procedures and messages specified for the UE equally apply to the UE in NB-IoT.

# Figure 4.1.1-1: LPP Configuration for Control- and User-Plane Positioning in E-UTRAN

### 4.1.2	LPP Sessions and Transactions

An LPP session is used between a Location Server and the target device in order to obtain location related measurements or a location estimate or to transfer assistance data. A single LPP session is used to support a single location request (e.g., for a single MT-LR, MO-LR or NI-LR). Multiple LPP sessions can be used between the same endpoints to support multiple different location requests (as required by TS 23.271 [3]). Each LPP session comprises one or more LPP transactions, with each LPP transaction performing a single operation (capability exchange, assistance data transfer, or location information transfer). In E-UTRAN and NR the LPP transactions are realized as LPP procedures. The instigator of an LPP session will always instigate the first LPP transaction, but subsequent transactions may be instigated by either end. LPP transactions within a session may occur serially or in parallel. LPP transactions are indicated at the LPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response).

Messages within a transaction are linked by a common transaction identifier.

### 4.1.3	LPP Position Methods

Internal LPP positioning methods and associated signalling content are defined in this specification.

This version of the specification defines OTDOA (based on LTE signals), A-GNSS, E-CID (for LTE and NR), Sensor, TBS, WLAN and Bluetooth positioning methods, as well as measurements to realize the NR methods DL-TDOA, DL-AOD and Multi-RTT.

## 6.3	Message Body IEs

#### –	RequestCapabilities

The RequestCapabilities message body in a LPP message is used by the location server to request the target device capability information for LPP and the supported individual positioning methods.

-- ASN1START

# RequestCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestCapabilities-r9		RequestCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

# epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

# ...,

# [[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

# wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

# ]] ,

# [[

# nr-posMeas-RequestCapabilities-r16	NR-PosMeas-RequestCapabilities-r16	OPTIONAL,	-- Need ON

# ]]

}

-- ASN1STOP

#### –	ProvideCapabilities

The ProvideCapabilities message body in a LPP message indicates the LPP capabilities of the target device to the location server.

-- ASN1START

# ProvideCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideCapabilities-r9		ProvideCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideCapabilities-r9-IEs ::= SEQUENCE {

commonIEsProvideCapabilities		CommonIEsProvideCapabilities		OPTIONAL,

a-gnss-ProvideCapabilities			A-GNSS-ProvideCapabilities			OPTIONAL,

otdoa-ProvideCapabilities			OTDOA-ProvideCapabilities			OPTIONAL,

ecid-ProvideCapabilities			ECID-ProvideCapabilities			OPTIONAL,

# epdu-ProvideCapabilities			EPDU-Sequence						OPTIONAL,

# ...,

# [[	sensor-ProvideCapabilities-r13	Sensor-ProvideCapabilities-r13		OPTIONAL,

# tbs-ProvideCapabilities-r13		TBS-ProvideCapabilities-r13			OPTIONAL,

# wlan-ProvideCapabilities-r13	WLAN-ProvideCapabilities-r13		OPTIONAL,

# bt-ProvideCapabilities-r13		BT-ProvideCapabilities-r13			OPTIONAL

# ]] ,

# [[

# nr-posMeas-ProvideCapabilities-r16	NR-PosMeas-ProvideCapabilities-r16	OPTIONAL,

# ]]

}

-- ASN1STOP

#### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

# RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

# [[	sensor-RequestAssistanceData-r14

# Sensor-RequestAssistanceData-r14	OPTIONAL,

# tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

# wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

# ]] ,

# [[

nr-posMeas-RequestAssistanceData-r16	NR-posMeas-RequestAssistanceData-r16	OPTIONAL,

# ]]

}

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

# ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

# [[

# sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

# tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

# wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

# ]] ,

# [[

nr-posMeas-ProvideAssistanceData-r16	NR-posMeas-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

# ]]

}

-- ASN1STOP

ProvideAssistanceData field descriptions

commonIEsProvideAssistanceData This IE is provided for future extensibility and should not be included in this version of the protocol.

#### –	RequestLocationInformation

The RequestLocationInformation message body in a LPP message is used by the location server to request positioning measurements or a position estimate from the target device.

-- ASN1START

# RequestLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestLocationInformation-r9	RequestLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

# epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

# ...,

# [[

# sensor-RequestLocationInformation-r13

# Sensor-RequestLocationInformation-r13

# OPTIONAL,	-- Need ON

# tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

# wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

# bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

# ]] ,

# [[

nr-posMeas-RequestLocationInformation-r16

NR-PosMeas-RequestLocationInformation-r16	OPTIONAL, -- Need ON

# ]]

}

-- ASN1STOP

RequestLocationInformation field descriptions

commonIEsRequestLocationInformation This field specifies the location information type requested by the location server and optionally other configuration information associated with the requested location information. This field should always be included in this version of the protocol.

#### –	ProvideLocationInformation

The ProvideLocationInformation message body in a LPP message is used by the target device to provide positioning measurements or position estimates to the location server.

-- ASN1START

# ProvideLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideLocationInformation-r9	ProvideLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

# epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

# ...,

# [[

# sensor-ProvideLocationInformation-r13

# Sensor-ProvideLocationInformation-r13

# OPTIONAL,

# tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

# wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

# bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

# ]] ,

# [[

nr-posMeas-ProvideLocationInformation-r16	NR-posMeas-ProvideLocationInformation-r16	OPTIONAL,

# ]]

}

-- ASN1STOP

#### –	Abort

The Abort message body in a LPP message carries a request to abort an ongoing LPP procedure.

-- ASN1START

# Abort ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

abort-r9		Abort-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# Abort-r9-IEs ::= SEQUENCE {

commonIEsAbort		CommonIEsAbort			OPTIONAL,	-- Need ON

...,

epdu-Abort			EPDU-Sequence			OPTIONAL	-- Need ON

}

-- ASN1STOP

#### –	Error

The Error message body in a LPP message carries information concerning a LPP message that was received with errors.

-- ASN1START

# Error ::= CHOICE {

error-r9					Error-r9-IEs,

criticalExtensionsFuture	SEQUENCE {}

}

# Error-r9-IEs ::= SEQUENCE {

commonIEsError		CommonIEsError			OPTIONAL,	-- Need ON

...,

epdu-Error			EPDU-Sequence			OPTIONAL	-- Need ON

}

-- ASN1STOP

***** End of change *****

***** Start of change *****

### 6.5.3	Enhanced Cell ID Positioning

#### 6.5.3.1	E-CID Location Information

#### –	ECID-ProvideLocationInformation

The IE ECID-ProvideLocationInformation is used by the target device to provide E-CID location measurements to the location server. It may also be used to provide ECID positioning specific error reason.

-- ASN1START

# ECID-ProvideLocationInformation ::= SEQUENCE {

ecid-SignalMeasurementInformation	ECID-SignalMeasurementInformation		OPTIONAL,

ecid-Error							ECID-Error								OPTIONAL,

... ,

[[ ecid-SignalMeasurementInformationNR-r16	ECID-SignalMeasurementInformationNR-r16		OPTIONAL

]]

}

-- ASN1STOP

#### 6.5.3.2	E-CID Location Information Elements

#### –	ECID-SignalMeasurementInformation

The IE ECID-SignalMeasurementInformation is used by the target device to provide various UE-measurements to the location server.

-- ASN1START

# ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

# MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

# MeasuredResultsElement ::= SEQUENCE {

physCellId						INTEGER (0..503),

cellGlobalId					CellGlobalIdEUTRA-AndUTRA	OPTIONAL,

arfcnEUTRA						ARFCN-ValueEUTRA,

systemFrameNumber				BIT STRING (SIZE (10))		OPTIONAL,

rsrp-Result						INTEGER (0..97)				OPTIONAL,

rsrq-Result						INTEGER (0..34)				OPTIONAL,

ue-RxTxTimeDiff					INTEGER (0..4095)			OPTIONAL,

...,

[[	arfcnEUTRA-v9a0				ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond EARFCN-max

]],

[[	nrsrp-Result-r14			INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14			INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14		CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14				BIT STRING (SIZE (10))		OPTIONAL

]],

[[

rsrp-Result-v1470			INTEGER (-17..-1)			OPTIONAL,

rsrq-Result-v1470			INTEGER (-30..46)			OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding arfcnEUTRA (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| NB-IoT | The field is mandatory present if the measured cell is a NB-IoT cell. Otherwise it is not present. |

ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell (if the primary cell is a E-UTRA or NB-IoT cell), when the target device reports measurements for both primary cell (E-UTRA or NB-IoT) and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell (E-UTRA or NB-IoT) only, in which case the measurements for the primary cell (E-UTRA or NB-IoT) is reported in the measuredResultsList. This field shall be omitted when the primary cell is not a E-UTRA or NB-IoT cell.

measuredResultsList This list contains the E-CID measurements for up to 32 E-UTRA or NB-IoT cells.

physCellId This field specifies the physical cell identity of the measured cell.

cellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the ECGI of the measured cell at the time of measurement.

arfcnEUTRA This field specifies the ARFCN of the measured E-UTRA carrier frequency, as defined in TS 36.331 [12]. In case the target device includes arfcnEUTRA-v9a0, the target device shall set the corresponding arfcnEUTRA (i.e. without suffix) to maxEARFCN.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

rsrp-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In case the target device includes rsrp-Result-v1470, the target device shall set the corresponding rsrp-Result (i.e. without suffix) to value 0.

rsrq-Result This field specifies the reference signal received quality (RSRQ) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In case the target device includes rsrq-Result-v1470, the target device shall set the corresponding rsrq-Result (i.e. without suffix) to value 0 or 34.

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 36.214 [17]. It is provided only for measurements on the UE's primary cell. Measurement report mapping is according to TS 36.133 [18].

nrsrp-Result This field specifies the narrowband reference signal received power (NRSRP) measurement, as defined in TS 36.214 [17]. Measurement report mapping is according to TS 36.133 [18].

nrsrq-Result This field specifies the narrowband reference signal received quality (NRSRQ) measurement, as defined in TS 36.214 [17]. Measurement report mapping to the value defined in TS 36.133 [18]. Values 0..29 map to values NRSRQ_-30..NRSRQ_-1. Values 30..62 map to NRSRQ_01..NRSRQ_33. Values 63..74 map to NRSRQ_35..NRSRQ_46. The UE does not report NRSRQ_00 nor NRSRQ_34.

carrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to ARFCN given by arfcnEUTRA as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper-SFN of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the hyper-SFN of the cell at the time of measurement.

ECID-SignalMeasurementInformation field descriptions

primaryCellNRMeasuredResults This field contains measurements for the NR primary cell, when the target device reports measurements for both primary cell and NR neighbour cells. This field shall be omitted when the target device reports measurements for the NR primary cell only, in which case the measurements for the NR primary cell is reported in the measuredResultsNRList. This field shall be omitted when the primary cell is not an NR cell.

measuredResultsList This list contains the E-CID measurements for up to 32 NR cells.

physCellIdNR This field specifies the physical cell identity of the measured cell.

measResultNR Measured results of an NR cell.

noSIB1 Contains ssb-SubcarrierOffset and pdcch-ConfigSIB1 fields acquired by the UE from MIB of the cell for which report CGI procedure was requested by the network in case SIB1 was not broadcast by the cell.

plmn-IdentityInfoList The plmn-IdentityInfoList contains a list of one or more PLMN Identities and additional information associated with those PLMNs. The total number of PLMNs in the PLMN-IdentityInfoList does not exceed 12. The parameter is defined in TS 36.331 [12] as PLMN-IdentityInfoList. The first/leftmost bit of the first octet contains the most significant bit.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. The parameter is defined in TS 36.331 [12] as MultiFrequencyBandListNR. The first/leftmost bit of the first octet contains the most significant bit.

ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13). The parameter is defined in TS 36.331 [12] as PDCCH-ConfigSIB1. The first/leftmost bit of the first octet contains the most significant bit.

pdcch-ConfigSIB1 Determines a common ControlResourceSet (CORESET), a common search space and necessary PDCCH parameters. If the field ssb-SubcarrierOffset indicates that SIB1 is absent, the field pdcch-ConfigSIB1 indicates the frequency positions where the UE may find SS/PBCH block with SIB1 or the frequency range where the network does not provide SS/PBCH block with SIB1 (see TS 38.213 [13], clause 13).

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 36.214 [17]. It is provided only for measurements on the UE's primary cell. Measurement report mapping is according to TS 36.133 [18].

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

#### –	ECID-SignalMeasurementInformationNR

The IE ECID-SignalMeasurementInformationNR is used by the target device to provide various UE NR measurements to the location server.

-- ASN1START

# ECID-SignalMeasurementInformationNR-r16 ::= SEQUENCE {

primaryCellNRMeasuredResults-r16	MeasuredResultsNRElement-r16			OPTIONAL,

measuredResultsNRList-r16			MeasuredResultsNRList-r16,

measResultCellListSFTD-NR-r16		MeasResultCellListSFTD-NR-r16			OPTIONAL

...

}

# MeasuredResultsNRList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsNRElement-r16

# MeasuredResultsNRElement-r16 ::= SEQUENCE {

physCellIdNR-r16				INTEGER (0..1007),

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

cgi-InfoNR-r16					NCGI-r15					OPTIONAL

ue-RxTxTimeDiff-r16				INTEGER (0..4095)			OPTIONAL,

}

MeasQuantityResults-r16 ::= SEQUENCE {

rsrp-r16	INTEGER (0..127)		OPTIONAL,

rsrq-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

CGI-InfoNR-r16 ::=	SEQUENCE {

plmn-IdentityInfoList-r16			OCTET STRING				OPTIONAL,

frequencyBandList-r16				OCTET STRING				OPTIONAL,

noSIB1								SEQUENCE {

ssb-SubcarrierOffset-r16		INTEGER (0..15),

pdcch-ConfigSIB1-r16			OCTET STRING

}																OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

MeasResultNR field descriptions

cellResults Cell level measurement results.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

[…]

### 6.5.8	NR Positioning Measurements

This subclause defines the information elements for downlink NR positioning measurements based on DL-PRS signals (TS 38.305 [xx]).

#### 6.5.8.1	NR Positioning Measurements Assistance Data

#### –	NR-PosMeas-ProvideAssistanceData

The IE NR-PosMeas-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based downlink NR positioning measurements and NR positioning measurements specific error reason.

-- ASN1START

# NR-PosMeas-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-PosFreqLayerInfo-r16		NR-PosFreqLayerInfo-r16,

nr-TRS-PrsInfo-r16			NR-TRS-PRS-Info-r16		OPTIONAL,	-- Need ON

nr-TRS-staticInfo-r16		NR-TRS-StaticInfo-r16	OPTIONAL,	-- Cond UEB

nr-TRS-dynamicInfo-r16		NR-TRS-DynamicInfo-r16	OPTIONAL,	-- Cond UEB

nr-posMeas-Error-r16		NR-PosMeas-Error-r16	OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | This field is mandatory present if the target device is configured for UE-based NR positioning. Note 1. |

NOTE 1: 	UE-based NR positioning is supported for DL-only positioning methods in this release of the specification.

NR-PosMeas-ProvideAssistanceData field descriptions

nr-PosFreqLayerInfo This field specifies the NR positioning frequency layers.

nr-TRS-PrsInfo This field specifies the DL-PRS information for a list of TRPs.

nr-TRS-staticInfo This field specifies static TRP information to enable UE-based positioning. This TRP list is of the same length as the list nr-TRP-PrsInfo, and two corresponding elements in the lists are associated to the same TRP.

nr-TRS-dynamicInfo This field specifies dynamic TRP information to enable UE-based positioning. This TRP list is of the same length as the list nr-TRP-PrsInfo, and two corresponding elements in the lists are associated to the same TRP.

nr-posMeas-Error This field specifies errors specific for NR positioning measurements.

#### 6.5.8.2	NR Positioning Measurements Assistance Data Elements

#### –	NR-PosFreqLayerInfo

The IE NR-PosFreqLayerInfo is used to provide configuration information for NR positioning frequency layers.

-- ASN1START

NR-PosFreqLayerInfo-r16 ::= SEQUENCE {SIZE (1..maxPosFreqLayers-r16)) OF

NR-PosFreqLayerElement-r16

NR-PosFreqLayerElement-r16 ::= SEQUENCE {

dL-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120},

dL-PRS-PointA-r16					ARFCN-ValueNR-r15,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

NR-PosFreqLayerInfo field descriptions

#### –	NR-TRS-PRS-Info

The IE NR-PRS-TRS-Info is used to provide DL-PRS configuration information for NR TRPs.

NOTE: 	This IE provides DL-PRS configuration information for a list of TRPs. The first TRP in the list can be seen by the target device as a proposed reference TRP in configurations where a reference TRP has a meaning. The index of an entry in IE NR-TRP-PRS-Info will be used to reference to individual elements, where index value 1 corresponds to a proposed reference TRP.

-- ASN1START

NR-TRP-PRS-Info-r16 ::= SEQUENCE {SIZE (1..nrMaxTRP-r16)) OF

NR-TRP-PRS-InfoElement-r16

NR-TRP-PRS-InfoElement-r16 ::= SEQUENCE {

nr-PRS-ResourceSets-r16			NR-PRS-ResourceSets-r16,

nrPhysCellId-r16				INTEGER (0..1007)		OPTIONAL,

nr-expectedRSTD-r16				INTEGER (FFS)			OPTIONAL,	-- Cond TOAmeas

nr-expectedRSTD-Uncertainty-r16	INTEGER (FFS)			OPTIONAL,	-- Cond TOAmeas

...

}

NR-PRS-ResourceSets-r16 ::= SEQUENCE {SIZE (1..nrMaxPrsResourceSets-r16)) OF

NR-PRS-ResourceSet-Info-r16

nrMaxTRP 					INTEGER ::= FFS   -- Max nummber of TRPs

nrMaxPrsResourceSets-r16	INTEGER ::= FFS   -- Max nummber of PRS resource Sets per TRP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| TOAmeas | The field, except for the first element in the list, is mandatory present when measurements based on TOA measurements are configured; otherwise it is not present (Need ON). |

NR-DL-TRS-PRS field descriptions

nr-PRS-ResourceSets This field specifies the information about the DL-PRS resource sets.

#### –	NR-PRS-ResourceSet-Info

The IE NR-PRS-ResourceSet-Info provides configuration of a DL-PRS resource set.

NOTE: 	TBD

-- ASN1START

NR-PRS-ResourceSet-Info-r16 ::= SEQUENCE {

nr-prsResourceSetID-r16				INTEGER (0..FFS),

nr-PosFreqLayerIndex				INTEGER (1.. maxPosFreqLayers-r16),

nr-PRS-Resources-r16				NR-PRS-Resources-r16

nr-prsResourceBandwidth-r16			INTEGER (0..62), 		-- FFS placement

nr-prsStartPRB-r16					INTEGER(0..2176), 		-- FFS placement

nr-prsSFN0-Offset-r16				INTEGER (1..FFS),			-- FFS us, FFS placement

ss-PBCH-BlockPower-r16				INTEGER (-60..50),  		-- FFS placement

nr-prsPeriodicity-r16				ENUMERATED {s4, s5, s8, s10, s16, s20, s32, s40, s64,

s80, s160, s320, s640, s1280, s2560, s5120,

s10240, s20480},

nr-prsResourceRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n6, n8, n16, n32},

nr-prsResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32},

nr-prsResourceSetSlotOffset-r16		CHOICE {

n4-r16							INTEGER (0..3),

n5-r16							INTEGER (0..4),

n8-r16							INTEGER (0..7),

n10-r16							INTEGER (0..9),

n16-r16							INTEGER (0..15),

n20-r16							INTEGER (0..19),

n32-r16							INTEGER (0..31),

n40-r16							INTEGER (0..39),

n64-r16							INTEGER (0..63),

n80-r16							INTEGER (0..79),

n160-r16						INTEGER (0..159),

n320-r16						INTEGER (0..319),

n640-r16						INTEGER (0..639),

n1280-r16						INTEGER (0..1279),

n2560-r16						INTEGER (0..2559),

n5120-r16						INTEGER (0..5119),

n10240-r16						INTEGER (0..10239),

n20480-r16						INTEGER (0..20479),

},

nr-prsNumSymbols-r16				ENUMERATED {n2, n4, n6},	-- FFS on {1, 3, 8, 12}

nr-prs-MutingInfo-r16			CHOICE {

po2-r16							BIT STRING (SIZE(2)),

po4-r16							BIT STRING (SIZE(4)),

po8-r16							BIT STRING (SIZE(8)),

po16-r16						BIT STRING (SIZE(16)),

po32-r16						BIT STRING (SIZE(32)),

...

}															OPTIONAL

nr-prsResourcePower-r16				INTEGER (0..FFS),

nr-prsCombSizeN-r16					ENUMERATED {n2, n4, n6},

...

}

NR-prsResources-r16 ::= SEQUENCE (SIZE (1..nrMaxPrsResources-r16)) OF NR-PRS-Resource-Info-r16

nrMaxPrsResources-r16		INTEGER ::= FFS   -- Max nummber of PRS resources per PRS resource set

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

NR-DL-PRS-Info field descriptions

nr-prsResourceBandwidth This field specifies the DL-PRS resource bandwidth in number of resource blocks. Integer value n defines the resource bandwidth (24 + 4n) resource blocks

nr-prsStartPRB This field specifies the DL-PRS resource bandwidth in number of resource blocks. Integer value n defines the resource bandwidth (24 + 4n) resource blocks

#### –	NR-PRS-Resource-Info

The IE NR-PRS-Resource-Info provides configuration of a DL-PRS resource set.

NOTE:

-- ASN1START

NR-PRS-Resource-Info-r16 ::= SEQUENCE {

nr-prsResourceID-r16			INTEGER (0..FFS),

nr-prsSequenceID-r16			INTEGER {0.. 4095},	--FFS larger value

nr-prsReOffset-r16				INTEGER (0..FFS),	--FFS on the value range

nr-prsResourceSlotOffset-r16		CHOICE {

n2-r16							INTEGER (0..1),

n4-r16							INTEGER (0..3),

n6-r16							INTEGER (0..5)

},

nr-prsResourceSymbolOffset-r16		INTEGER (0.. nrMaxPrsResoourceOffset-r16),

nr-prsQCL-Info-r16					ENUMERATED {typeC, typeD},

...

}

nrMaxPrsResoourceOffset-r16		INTEGER ::= FFS   -- Max PRS resource offset

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

NR-DL-PRS-Info field descriptions

nr-prsResourceBandwidth This field specifies the DL-PRS resource bandwidth in terms on number of PRBs

#### –	NR-TRP-StaticInfo

The IE NR-TRP-StaticInfo is used to provide static TRP configuration information to the target device to support UE-based positioning. This TRP list, if present, is of the same length as the TRP list nr-TRP-PrsInfo, and the elements are ordered in the same way, which means that the information from an element k in this list and an element k in the list nr-TRP-PrsInfo corresponds to the same TRP. In a similar way are the DL PRS resource sets and resources listed in the same order in nr-prsBeamInfo and nr-prsInfo., so corresponding elements in the DL PRS resource set and resources lists are associated to the same resource set and resource, respectively.

-- ASN1START

NR-TRP-StaticInfo-r16 ::= SEQUENCE {SIZE (1.. nrMaxTRP-r16)) OF

NR-TRP-StaticInfoElement-r16

NR-TRP-StaticInfoElement-r16 ::= SEQUENCE {

nr-TRP-locationInfo-r16 	NR-TRP-LocationInfo-r16,

nr-prsBeamInfo-r16			NR-PRS-BeamInfo-r16			OPTIONAL,	-- Cond AoDsupport

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

NR-TRS-StaticInfo field descriptions

nr-TRP-locationInfo This field specifies the location information associated to a TRP.

#### –	NR-TRP-DynamicInfo

The IE NR-TRP-DynamicInfo is used to provide dynamic TRP configuration information to the target device to support UE-based positioning. This TRP list, if present, is of the same length as the TRP list nr-TRP-PrsInfo, and the elements are ordered in the same way, which means that the information from an element k in this list and an element k in the list nr-TRP-PrsInfo corresponds to the same TRP.

-- ASN1START

NR-TRP-DynamicInfo-r16 ::= SEQUENCE {SIZE (1.. nrMaxTRP-r16)) OF

NR-TRP-DynamicInfoElement-r16

NR-TRP-DynamicInfoElement-r16 ::= SEQUENCE {

nr-rtdInfo-r16			NR-RTD-Info-r16		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| TOAmeas | The field, except for the first element in the list, is mandatory present when measurements based on TOA measurements are configured; otherwise it is not present (Need ON). |

NR-DL-TRS-PRS field descriptions

nr-rtdInfo This field specifies the relative time difference between pairs of TRPs.

#### 6.5.8.3	NR-PosMeas Assistance Data Request

#### –	NR-PosMeas-RequestAssistanceData

The IE NR-PosMeas-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# NR-PosMeas-RequestAssistanceData-r16 ::= SEQUENCE {

nrPhysCellId-r16		INTEGER (0..1007)											OPTIONAL,

adType-r16				BIT STRING {dl-prs   (0),

trp-loc  (1),

trp-rtd  (2),

trp-beam (3) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-PosMeas-RequestAssistanceData field descriptions

nrPhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device.

adType This field specifies the assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. Bit 0 indicates that NR DL-PRS assistance data is requested. Bit 1 indicates TRP location information to support UE-based positioning is requested. Note 1 Bit 2 indicates TRP relative time difference information to support UE-based positioning is requested. Note 1 Bit 3 indicates TRP beam information to support UE-based positioning is requested. Note 1

#### 6.5.8.4	NR Positioning Measurements Location Information

#### –	NR-PosMeas-ProvideLocationInformation

The IE NR-PosMeas-ProvideLocationInformation is used by the target device to provide NR positioning measurements or location estimate to the location server. It may also be used to provide NR positioning measurements specific error reason.

-- ASN1START

# NR-PosMeas-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-posMeas-SignalMeasurementInformation-r16 NR-SignalMeasurementInformation-r16	OPTIONAL,

nr-posMeasError-r16							NR-PosMeas-Error-r16				OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.8.5	NR Positioning Measurements Location Information Elements

#### –	NR-PosMeas-SignalMeasurementInformation

The IE NR-PosMeas-SignalMeasurementInformation is used by the target device to provide NR positioning measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP in case RSTD measurements are reported. The first TRP in the list may or may not be the same TRP as the first TRP in the TRP list nr-TRS-PRS-Info in the assistance data. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource. The target device also provides a time stamp nr-posMeas-timeStamp corresponding to the measurement instant of the reference resource of the first TRP in the nr_posMeasList.

-- ASN1START

# NR-PosMeas-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-posMeas-timeStamp-r16		NR-PosMeas-timeStamp-r16,

nr-posMeasList-r16				NR-PosMeasList-r16,

...

}

# NR-PosMeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRP-r16)) OF

# NR-PosMeasListElement-r16

# NR-PosMeasListElement-NR-r16 ::= SEQUENCE {

nr-TRP-index-r16					INTEGER (1.. nrMaxTRP-r16),

nr-refResourceSetID-r16				INTEGER (FFS),

nr-prsResourceID-r16				INTEGER (FFS),

nr-rstd-r16							INTEGER (FFS)					OPTIONAL,	-- Cond NotFirstElement

nr-additionalPathsRef-r16			NR-AdditionalPathList-r16		OPTIONAL,	-- Cond AddPaths

nr-toaQuality-r16					NR-TOA-MeasQuality-r16			OPTIONAL,	-- Cond TOAmeas

nr-ueRxTxTD-r16						INTEGER (FFS)					OPTIONAL,	-- Cond RXTXmeas

nr-RxTxSync-Quality-r16				NR-RxTx-SyncQuality-r16 		OPTIONAL,	-- Cond RXTXmeas

nr-rsrp-r16							INTEGER (0..127)				OPTIONAL,	-- Cond RSRPmeas

nr-rsrp-Quality-r16					NR-RSRP-MeasQuality-r16			OPTIONAL,	-- Cond RSRPmeas

nr-sameResSetPosMeasList-r16		NR-SameResSetPosMeasList-r16,	OPTIONAL,	-- Cond ReportRelRes

nr-otherResSetPosMeasList-r16		NR-OtherResSetPosMeasList-r16,	OPTIONAL,	-- Cond ReportRelRes

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..maxPaths-r16)) OF

NR-AdditionalPath-r16

NR-SameResSetPosMeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxPrsResources-1-r16)) OF

NR-PosMeas-MeasPerResource-r16

NR-OtherResSetPosMeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxPrsResourceSets-1-r16)) OF

NR-OtherResSetPosMeasListElement-r16

NR-RelResSetPosMeasListElement-r16 ::= SEQUENCE {

nr-prsResourceSetID-r16			INTEGER (FFS),

nr-relResPosMeasList-r16		NR-RelResPosMeasList-r16,

...

}

NR-RelResPosMeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxPrsResources -r16)) OF

NR-PosMeas-MeasPerResource-r16

NR-PosMeas-MeasPerResource-r16 ::= SEQUENCE {

nr-prsResourceID-r16 				INTEGER (FFS),

nr-rstd-r16							INTEGER (FFS),					OPTIONAL,	-- Cond RSRPmeas

nr-additionalPaths-r16				NR-AdditionalPathList-r16		OPTIONAL,

nr-toa-Quality-r16					NR-TOA-MeasQuality-r16			OPTIONAL,	-- Cond RSRPmeas

nr-prs-rsrp-r16						INTEGER (FFS)					OPTIONAL, 	-- Cond RSRPmeas

nr-prs-rsrp-Quality-r16				NR-RSRP-MeasQuality-r16			OPTIONAL, 	-- Cond RSRPmeas

nr-ueRxTxTD-r16						INTEGER (FFS)					OPTIONAL,	-- Cond RXTXmeas

nr-RxTxSync-Quality-r16				NR-RxTx-SyncQuality-r16 		OPTIONAL,	-- Cond RXTXmeas

...

}

nrMaxPrsResourceSets-1-r16	INTEGER ::= FFS   -- Max nummber of PRS resource Sets per TRP - 1

nrMaxPrsResources-1-r16		INTEGER ::= FFS   -- Max nummber of PRS resource Sets per TRP - 1

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotFirstElement | This field, except for the first element in the list, is mandatory present when RSTD measurements are configured, otherwise it is not present |
| TOAmeas | The field is mandatory present when measurements based on TOA measurements are configured; otherwise it is not present (Need ON). |
| RXTXmeas | The field is mandatory present when UE Rx-Tx measurements are configured; otherwise it is not present (Need ON). |
| ReportRelRes | The field is mandatory present when no timing measurement aggregation has been configured; otherwise it is only present id RSRP measurement have been configured |
| AddPaths | The field is mandatory if the target device supports additional path reporting and has detected additional paths, otherwise it is not present (Need ON) |

NR-OTDOA-SignalMeasurementInformation field descriptions

#### –	NR-PosMeas-TimeStamp

The IE NR-PosMeas-TimeStamp provides the NR positioning measurement time stamp associated to the measurements of the reference resource of a TRP. Devices supporting fine timestamps will also provide the fine UTC time resolution.

-- ASN1START

# NR-PosMeas-TimeStamp-r16 ::= SEQUENCE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utcTime-r16					UTCTime						OPTIONAL,	-- Cond UTCmeas

utcTime-ms-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas

utcTime-us-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas

utcTime-ns-r16				INTEGER (0..999)			OPTIONAL,	-- Cond UTCmeas-fine

utcTime-subns-r16			INTEGER (0..9)				OPTIONAL,	-- Cond UTCmeas-fine

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UTCmeas | The field is mandatory present if the positioning measurement is requested to be associated with an UTC time at ms level or a UTC fine level; otherwise it is not present. |
| UTCmeas-fine | The field is mandatory present if the positioning measurement is requested to be associated with a fine UTC time at sub ns level; otherwise it is not present. |

NR-PosMeas-TimeStamp field descriptions

systemFrameNumber This field specifies the NR System Frame Number (SFN).

utcTime This field provides the UTC time in the form of YYMMDDhhmmssZ.

utcTime-ms This field provides the fractional part of the UTC time in ms resolution

utcTime-us This field provides the fractional part of the UTC time in us resolution

utcTime-ns This field provides the fractional part of the UTC time in ns resolution

utcTime-subns This field provides the fractional part of the UTC time in sub-ns resolution (0.1 ns resolution

#### –	NR-TOA-MeasQuality

-- ASN1START

# TOA-MeasQuality ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	NR-RxTx-SyncQuality

-- ASN1START

# RxTx-SyncQuality ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	NR-RSRP-MeasQuality

-- ASN1START

# NR-RSRP-MeasQuality ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	NR-AdditionalPath

The IE NR-AdditionalPath is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

# NR-AdditionalPath-r16 ::= SEQUENCE {

nr-relativeTimeDifference-r16	FFS,

nr-path-Quality-r16				NR-TOAMeasQuality-r16				OPTIONAL,

...

}

-- ASN1STOP

NR-AdditionalPath field descriptions

nr-relativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing of the reference resource. A positive value indicates that the particular path is later in time than the detected path of the reference; a negative value indicates that the particular path is earlier in time than the detected path of the reference

nr-path-Quality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

#### 6.5.8.6 	NR Positioning Measurement Location Information Request

#### –	NR-PosMeas-RequestLocationInformation

The IE NR-PosMeas-RequestLocationInformation is used by the location server to request NR positioning measurements from a target device. Details of the required measurements (details of TRP configurations) are conveyed in the NR-PosMeas-ProvideAssistanceData IE in a separate Provide Assistance Data message.

-- ASN1START

# NR-PosMeas-RequestLocationInformation ::= SEQUENCE {

assistanceAvailability-r16		BOOLEAN,

rsrpMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

rstdMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

ueRxTxMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

fineTimeStamp-r16				ENUMERATED { requested }		OPTIONAL,		-- Need ON

toaAggreg-r16					ENUMERATED { first, strongest, ... }	OPTIONAL,	-- Need ON

multipathTOA-r16				ENUMERATED { requested }		OPTIONAL,		-- Need ON

motionMeasurements-r16			ENUMERATED { requested }		OPTIONAL,		-- Need ON

...

}

-- ASN1STOP

NR-PosMeas-RequestLocationInformation field descriptions

assistanceAvailability This field indicates whether the target device may request additional NR positioning measurements assistance data from the server. TRUE means allowed and FALSE means not allowed.

rsrpMeasurements This field, if present, indicates that the target device is requested to report DL PRS RSRP per resource.

rstdMeasurements This field, if present, indicates that the target device is requested to report DL PRS RSTD per resource in relation to a reference resource, in consideration of per TRP measurement aggregation if configured.

ueRxTxMeasurements This field, if present, indicates that the target device is requested to report UE Rx-Tx per DL PRS resource in relatiojn to a configured uplink time reference and , in consideration of per TRP measurement aggregation if configured

fineTimeStamp This field, if present, indicates that the target device is requested to report a fine time stamp of the reference TRP, in consideration of per TRP measurement aggregation if configured.

toaAggreg This field, if present, indicates that the target device is requested to report timing aggregated per TRP, and the aggregation strategy. The value ‘first’ means that the DL-PRS TOA per TRP is determined a the DL-PRS TOA of the resource corresponding to the shortest time of flight. The value ‘strongest’ means that the DL-PRS TOA per TRP is determined a the DL-PRS TOA of the resource corresponding to the shortest DL PRS RSRP.

multipathTOA This field, if present, indicates that the target device is requested to report additional detected path timing information.

motionMeasurements This field, if present, indicates that the target device is requested to report the motion measurements (deltaSFN and motionTimeSource) in NR-PosMeas-SignalMeasurementInformation as well as the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

#### 6.5.8.7	NR Positioning Measurements Capability Information

#### –	NR-PosMeas-ProvideCapabilities

The IE NR-PosMeas-ProvideCapabilities is used by the target device to indicate its capability to support NR positioning measurements and to provide its NR positioning measurements capabilities to the location server.

-- ASN1START

# NR-PosMeas-ProvideCapabilities-r16 ::= SEQUENCE {

# rsrpReporting-r16 				ENUMERATED { supported }		OPTIONAL,

# rstdReporting-r16 				ENUMERATED { supported }		OPTIONAL,

# ueRxTxReporting-r16 			ENUMERATED { supported }		OPTIONAL,

# fineTimeStamp-r16				ENUMERATED { supported }		OPTIONAL,

# additionalPathsReport-r16 		ENUMERATED { supported }		OPTIONAL,

periodicalReporting-r16			PositioningModes				OPTIONAL,

idleStateForMeasurements-r16	ENUMERATED { required }			OPTIONAL,

motionMeasurements-r16			ENUMERATED { supported }		OPTIONAL,

# uebLocRtd-r16	 				ENUMERATED { supported }		OPTIONAL,

# uebLocBeam-r16	 				ENUMERATED { supported }		OPTIONAL,

...

}

-- ASN1STOP

NR-PosMeas-ProvideCapabilities field descriptions

rsrpReporting This field, if present, indicates that the target device supports NR DL PRS RSRP measurement reporting

rstdReporting This field, if present, indicates that the target device supports NR DL PRS RSTD measurement reporting

ueRxTxReporting This field, if present, indicates that the target device supports NR UE RxTx measurement reporting

fineTimeStamp This field, if present, indicates that the target device supports fine time stamp reporting

additionalPathsReport This field, if present, indicates that the target device supports reporting of timing information for additional detected paths for reported TOA-based measurements

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform positioning measurements.

motionMeasurements This field, if present, indicates that the target device supports reporting of motion measurements (deltaSFN and motionTimeSource) in NR-SignalMeasurementInformation. The presence of this field implies presence of sensor-MotionInformationSup in IE Sensor-ProvideCapabilities.

uebLocRtd This field, if present, indicates that the target device supports NR UE-based positioning exploiting TRP location and relative time difference information

uebLocBeam This field, if present, indicates that the target device supports NR UE-based positioning exploiting TRP location and beam information

#### 6.5.8.8	NR Positioning Measurements Capability Information Request

#### –	NR-PosMeas-RequestCapabilities

The IE NR-PosMeas-RequestCapabilities is used by the location server to request the capability of the target device to support NR positioning measurements and to request NR positioning measurements capabilities from a target device.

-- ASN1START

# NR-PosMeas-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.8.9	NR Positioning Measurements Error Elements

#### –	NR-PosMeas-Error

The IE NR-PosMeas-Error is used by the location server or target device to provide NR positioning measurements error reasons to the target device or location server, respectively.

-- ASN1START

# NR-PosMeas-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-PosMeas-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-PosMeas-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-PosMeas-LocationServerErrorCauses

The IE NR-PosMeas-LocationServerErrorCauses is used by the location server to provide NR positioning measurements error reasons to the target device.

-- ASN1START

# NR-PosMeas-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

notProvidedAssistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

...

},

...

}

-- ASN1STOP

#### –	NR-PosMeas-TargetDeviceErrorCauses

The IE NR-PosMeas-TargetDeviceErrorCauses is used by the target device to provide NR positioning measurements error reasons to the location server.

-- ASN1START

# NR-PosMeas-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistanceDataMissing,

unableToMeasureAnyTRP,

noUplinkPrsConfigured,

thereWereNotEnoughSignalsReceivedForUeBasedPos,

UebAssistanceDataMissing,

...

},

...

}

-- ASN1STOP