3GPP TSG-RAN WG2 #108	Tdoc R2-1915786

Reno, Nevada, US, 18th – 22nd November 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 2-step RA 38.331 Running CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of 2-step random access |  |  |  |  |  |  |  |  |
| Summary of change: |  | This introduces 2-step RA according to RAN2 agreements up to RAN2#107bis, RAN1-parameters as discussed in the e-mail discussion as an example. RAN2#107 agreements that impact RRC: - If the random access procedure is not successfully completed even after transmitting the msgA ‘N’ times, the UE fallbacks to 4 step RACH procedure i.e. UE only transmits the PRACH preamble - The network can configure the number of times ‘N’, a UE can attempt to re-transmit msgA during the random access procedure - If the UE is configured with 2-step RA, the RSRP is above a configurable threshold then the UE shall use the 2-step RA procedure RAN2#107bis: - The RSRP threshold for 2-step vs 4step CBRA can be configured separately for NUL and SUL - Introduce preamble group A and B for 2-step RACH - Apply the same selection formulas between 2-step preambles group A and B as specified for 4-step in REl-15. For the purpose of data threshold, ra-msgASizeGroupA parameter can be introduced - For 2 step CBRA, RA prioritisation is supported at least for handover and beam failure recovery. - RA prioritization for 2 step CBRA is controlled by networkRA prioritisation parameters are separately configured for 2 step CBRA and 4 step CBRA. – ra-Prioritisation2Step is optionally added to BeamFailureRecoveryConfig IE and RACH-ConfigDedicated IE. - The 2-step RACH resources can be configured on a BWP where 4-step CBRA resrouces are not configured. In that case we will not have 4-step switch. - The 2-step CBRA for SpCell BFR is supported in Rel-16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR 38.321 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.3.2	Radio resource control information elements

#### –	BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}                                      OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                           OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                                                             OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                                                         OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing-v1530        SubcarrierSpacing                                                         OPTIONAL  -- Need M

]],

[[

ra-PrioritizationTwoStep-r16		RA-Prioritization																  OPTIONAL	-- Need XX

]]

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

-- TAG-BEAMFAILURERECOVERYCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

candidateBeamRSList A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

msg1-SubcarrierSpacing Subcarrier spacing for contention free beam failure recovery. Only the values 15 kHz or 30 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable. See TS 38.211 [16], clause 5.3.2.

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free random access to recover from beam failure (see TS 38.213 [13], clause 6).

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources.

rach-ConfigBFR Configuration of contention free random access occasions for BFR.

recoverySearchSpaceId Search space to use for BFR RAR. The network configures this search space to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided. The CORESET associated with the recovery search space cannot be associated with another search space. Network always configures the UE with a value for this field when contention free random access resources for BFR are configured.

ssb-perRACH-Occasion Number of SSBs per RACH occasion for CF-BFR, see TS 38.213 [13], clause 8.1.

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR).

>>Skipped unchanged parts

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonTwoStepRA-r16		SetupRelease {RACH-ConfigCommonTwoStepRA-r16}									OPTIONAL 	-- Need M

]]

}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery).

rach-ConfigCommonTwoStepRA Configuration of cell specific random access parameters for 2-step random access procedure. Full field description FFS.

>>Skipped unchanged parts

#### –	MsgA-PUSCH-Config

The IE MsgA-PUSCH-Config is used to specify the PUSCH allocation for MsgA in 2-step random access procedure.

MsgA-PUSCH-Config information element

-- ASN1START

-- TAG-MSGAPUSCHCONFIG-START

MsgA-PUSCH-Config-r16 ::=              SEQUENCE {

Editor’s note: the PUSCH parameters will be implemented based on RAN1 LS RRC parameters.

}

-- TAG-MSGAPUSCHCONFIG-STOP

-- ASN1STOP

MsgA-PUSCH-Config field descriptions

FFS by RAN1

>>Skipped unchanged parts

#### –	RACH-ConfigCommonTwoStepRA

The IE RACH-ConfigCommonTwoStepRA is used to specify cell specific 2-step random-access parameters.

RACH-ConfigCommonTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-R16-START

RACH-ConfigCommonTwoStepRA-r16 ::=               SEQUENCE {

rach-ConfigGenericTwoStepRA-r16                	RACH-ConfigGeneric-r16,

msgA-TotalNumberOfRA-Preambles-r16          	INTEGER (1..63)                                    OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-TwoStepRA-r16      SEQUENCE {

oneEighth													ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth													ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,64},

oneHalf														ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,64},

one															ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,64},

two															ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four														INTEGER {1..16},

eight														INTEGER {1..8},

sixteen														INTEGER {1..4}

}																										OPIOTNAL, -- Need XX

msgA-CB-PreamblesPerSSB							TBD														OPTIONAL, -- Need XX

OPTIONAL,   -- Need S

groupBconfigured-r16                 		SEQUENCE {

Editor’s note: Details on preamble grouping FFS

},                                                                                                 	OPTIONAL,  -- Need XX

msgA-PRACH-RootSequenceIndex-r16            CHOICE {

l839                                    	INTEGER (0..837),

l139                                    	INTEGER (0..137)

},																										OPTIONAL, 	-- Need S

msgA-PRACH-SubcarrierSpacing-r16            SubcarrierSpacing	                                   	OPTIONAL,   -- Need S

rsrp-ThresholdTwoStepRA-r16           		RSRP-Range                                             	OPTIONAL,   -- Need S

rsrp-ThresholdTwoStepRA-SUL-r16           	RSRP-Range                                             	OPTIONAL,   -- Need S

msgA-RestrictedSetConfig					ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB}								OPTIONAL,	-- Need S

...

}

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-R16-STOP

-- ASN1STOP

RACH-ConfigCommonTwoStepRA field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.2)

msgA-CB-PreamblesPerSSB Field description and whether this field is included is FFS.

msgA-PRACH-RootSequenceIndex Field description and whether this field is included is FFS.

msgA-PRACH-SubcarrierSpacing Field description and whether this field is included is FFS.

msgA-TotalNumberOfRA-Preambles Field description and whether this field is included is FFS.

numberOfRA-PreamblesGroupA Field description and whether this field is included is FFS.

ra-MsgA-SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2).

rsrp-ThresholdTwoStepRA The UE selects 2-step to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). FFS.

rsrp-ThresholdTwoStepRA-SUL The UE selects 2-step to perform random access if SUL carrier is selected based on this threshold (see TS 38.321 [3], clause 5.1.1). FFS.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-TwoStepRA Description and field FFS

RACH-ConfigGenericTwoStepRA 2-step RACH parameters for both regular random access and beam failure recovery.

Editor’s note: Which fields that should be optional and which fields may use 4-step values if not present due to shared/separate RO will be addressed.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if msgA-PRACH-RootSequenceIndex L=139, otherwise the field is absent. |

>>Skipped unchanged parts

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16	RA-Prioritization															  OPTIONAL -- Need XX

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond SSB-CFRA

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles-v1530 INTEGER (1..63)                                                         OPTIONAL -- Cond Occasions

]]

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If the field is absent, the UE performs contention based random access.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| SSB-CFRA | The field is mandatory present if the field resources in CFRA is set to ssb; otherwise it is absent. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |

>>Skipped unchanged parts

#### –	RACH-ConfigGenericTwoStepRA

The IE RACH-ConfigGenericTwoStepRA is used to specify the 2-step random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGenericTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-R16-START

RACH-ConfigGenericTwoStepRA-r16 ::=			SEQUENCE {

msgA-PRACH-ConfigurationIndex-r16           TBD				                                              	OPTIONAL,   -- Need XX

msgA-RO-FDM-r16			   		            ENUMERATED {One, Two, Four, Eight}(TBC)	                        	OPTIONAL,   -- Need XX

msgA-RO-FrequencyStart-r16		            INTEGER (0..maxNrofPhysicalResourceBlocks-1)							OPTIONAL,   -- Need XX

msgA-ZeroCorrelationZoneConfig-r16      	INTEGER (0..15)													   	OPTIONAL,   -- Need XX

msgA-TransMax-r16                    		ENUMERATED {Value FFS},

msgA-PreamblePowerRampingStep-r16          	TBD																		OPTIONAL,   -- Need XX

msgB-ResponseWindow-r16                 	ENUMERATED {Value FFS},

msgA-PUSCH-Config-r16						MsgA-PUSCH-Config-r16,

...

}

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-R16-STOP

-- ASN1STOP

RACH-ConfigGenericTwoStepRA field descriptions

msgA-PUSCH-Config The MsgA PUSCH configuration (see TS 38.xxx).

msgA-PreamblePowerRampingStep Field description and whether this field is included is FFS.

msgA-PRACH-ConfigurationIndex Field description and whether this field is included is FFS.

msgA-RO-FDM Field description and whether this field is included is FFS.

msgA-RO-FrequencyStart Field description and whether this field is included is FFS.

msgA-TransMax Max number of MsgA preamble transmissions performed before declaring a failure (see TS 38.321 [3], clauses FFS).

msgA-ZeroCorrelationZoneConfig Field description and whether this field is included is FFS.

msgB-ResponseWindow MsgB monitoring window length in number of slots (see TS 38.321 [3], clause FFS).

Editor’s note: Which fields that should be optional and which fields may use 4-step values if not present due to shared/separate RO will be addressed.

End of changes