3GPP TSG-RAN WG2 Meeting #108	R2-1915798

Reno, Nevada, USA, 18-22 November 2019

Agenda Item:	6.4.5

Source: 	MediaTek Inc.

Title:  	Asynchronous synchronisation reference sources

Document for:	Discussion and decision

# Introduction

In RAN4#92bis, the group discussed the situation where two UEs need to communicate via sidelink while their synchronisation reference sources are not synchronised with one another, e.g. different gNBs/eNBs ([1]).  RAN4 sent a corresponding LS to RAN2 ([2]) requesting signalling to support timing adjustment in asynchronous scenarios.  This document discusses the situation from a RAN2 perspective and proposes a signalling approach to resolving it.

# Discussion

LTE V2X introduced a Direct Frame Number (DFN) offset to prevent UEs that are synchronised to GNSS from making transmissions that interfere with the uplink.  The offset has a range of [0,1] ms with microsecond resolution and applies when the UE is synchronised to GNSS.  As noted in [1] and shown in Figure 1, this offset could be reused to align timing when one UE is synchronised to GNSS and another to a gNB—instead of shifting UE 1’s timing from GNSS to the gNB, the DFN offset is used to shift UE 2 from the gNB to GNSS, resulting in timing alignment with UE 1.

Figure 1: Use of DFN offset to align timing between UEs synchronised to GNSS and gNB

If the network could always configure GNSS as the synchronisation source for one of the UEs, this would be sufficient to solve the problem, enabling the two UEs to communicate with each other (with UE2 needing to prioritise between UL towards the gNB and SL towards UE1 when the transmissions collide); however, this clearly is not always possible (e.g. tunnel scenarios).  A solution is also needed when both UEs are synchronised to gNBs/eNBs that may not be synchronised with one another.

As suggested in [1], by using the DFN offset, the gNB(s)/eNB(s) can cause both UEs to align their slot boundaries with the GNSS timeline as a kind of indirect synchronisation to GNSS.  However, the DFN offset with its current range can only align to the nearest subframe boundary, meaning that the resulting alignment may be off by an integer number of milliseconds between the two UEs (Figure 2).

Figure 2: offsetDFN produces misalignment by an integer number of subframes

To correct for this misalignment, it should be possible for the eNB/gNB to configure the UE with an additional slot-level offset as well as the DFN offset.  The result is that the network can align the UE to an arbitrary slot timing on the GNSS timeline, and in particular, two (or more) UEs that need to communicate can have their timing aligned.

Proposal 1: The UE can be configured with a slot-level offset in addition to the DFN offset, to adjust the slot timing in relation to a sync source such as GNSS.

The operation of the two offsets together is as shown in Figure 3.

Figure 3: Operation of DFN offset and slot offset in combination

These offsets can be signalled in two different ways, depending on whether they are maintained as separate fields (microsecond offset to GNSS-aligned subframe timing + millisecond offset to frame timing) or a single field (microsecond offset directly to the desired frame timing).

In the first approach, the gNB needs to deliver two fields: the DFN offset with microsecond granularity and a range of [0,1) ms, and the slot offset with subframe granularity and a 10-ms range (e.g. [-5,5) or [0,10) ms).  Since the offsets are used to adjust from the gNB timing to GNSS timing, both can be common parameters at the per-carrier/per-cell level.

In the second approach, the two offsets are combined into a single field, which would need to have microsecond granularity and a total range of 10 ms, e.g. a range of (0..9999) or (-5000..4999).  The single offset could serve the combined functions of the DFN offset and slot level offset as described above, resulting in alignment to GNSS with the desired frame timing.

Either signalling approach is valid; we have a slight preference for the combined offset as being clearer to interpret.  The offsets should be included in the per-carrier/per-cell configurations; they are not needed in preconfigurations (there may be situations where a UE is synced to a gNB but using a preconfiguration, e.g. because the serving gNB does not offer any V2X configurations, but in such a case it is unclear how the preconfiguration could account for the alignment between the gNB and GNSS), and a UE-specific value is not needed since all UEs under the same gNB will have the same offset.

Proposal 2: In the NR per-carrier/per-cell V2X configurations, offsetDFN is introduced with units of microseconds and a range of (0..9999).

In the LTE signalling (for UEs controlled by an eNB), an extended offsetDFN could be introduced as an extension in the same way as in NR, or a separate slot offset could be added in addition to the existing offsetDFN.  Again the single offset may be slightly clearer to interpret (especially considering alignment with the NR signalling), but either approach is valid.

Proposal 3: In the LTE per-carrier/per-cell V2X configurations, an extended version of offsetDFN is introduced with units of microseconds and a range of (0..9999).  If the extended offset is signalled, the legacy offsetDFN field is disregarded.

# Conclusion

This document promulgated the following proposals:

Proposal 1: The UE can be configured with a slot-level offset in addition to the DFN offset, to adjust the slot timing in relation to a sync source such as GNSS.

Proposal 2: In the NR per-carrier/per-cell V2X configurations, offsetDFN is introduced with units of microseconds and a range of (0..9999).

Proposal 3: In the LTE per-carrier/per-cell V2X configurations, an extended version of offsetDFN is introduced with units of microseconds and a range of (0..9999).  If the extended offset is signalled, the legacy offsetDFN field is disregarded.

# References

[1]	R4-1910900, “Discussion on NR V2X communication in asynchronous network”, MediaTek Inc., RAN4#92bis

[2]	R4-1912826/R2-19xxxxx, “LS on sidelink synchronization under multiple synchronization sources with different timing”, LS from RAN4 to RAN2, RAN4#92bis/RAN2#108