3GPP TSG-RAN WG2 #108	Tdoc R2- 1915801

Reno, Nevada, US, 18 – 22 November 2019	Revision of R2- 1913776

Agenda Item:	7.2.3

Source:	Ericsson

Title:	Group WUS

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 Work Item Descriptions for LTE-M [1] and NB-IoT [2] contain a common objective on enhancing the wake-up signal (WUS) that was introduced in Rel-15:

Improved DL transmission efficiency and/or UE power consumption: - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

So far the following GWUS RAN1 agreements have been made that are relevant for RAN2:

Agreement UE grouping is based on at least UE ID or some function of UE ID Agreement The number of UE groups is configurable and broadcasted in SIB. Agreement If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. Agreement - Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation. - Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap. Agreement The specification supports configurability to enable UE group to alternate between WUS resources Agreement A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs. Agreement For group WUS resources sharing WUS gap, per default, the number of UE groups per resource is configured for all group WUS resources. Optionally, the number of UE groups for each WUS resource can be configured individually. Agreement The number of UE groups per WUS resource is 1, 2, 4, or 8 Agreement Design pre-defined method to allow alternating UE group to monitor different WUS resources at different POs implicitly. - Consider both cell-specific DRX cycle and UE-specific DRX cycle. - At least the following parameters are used in the pre-defined method - H_SFN of current PO - DRX cycle

So far the following GWUS RAN2 agreements have been made:

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. RAN2#104 agreements: RAN2 will decide on the UE to WUS group mapping. RAN2#105 agreements: Feasibility of the solution based on the following attributes for deriving the service-type for GWUS can be studied further: - Paging Probability - Mobility. RAN2#105bis agreements: Additional grouping based on DRX/eDRX is not supported Coverage based grouping is not supported Additional grouping based on gap is not supported - FFS whether number of groups can depend on gap duration. RAN2#106 agreements: WUS grouping based on paging probability is beneficial for improving false wake-up probability for UEs which are not frequently paged, but may increase false wake-up probability for UEs which are frequently paged. RAN2#107 agreements: - Paging probability information is negotiated between the UE and MME via NAS signalling - This paging probability information needs to be provided by S1 paging - eNB configures, via broadcast, the relation between this paging probability information and WUS group on Uu interface. RAN2#107bis agreements: FFS: A WUS group cannot be mapped to both UEs using UE-ID based only and UEs using UE-ID + paging probability based grouping. UE WUS group hopping between WUS resources is configured per cell. RAN2 assumes one or more UEs may not be configured at NAS with paging probability. If R16 WUS is configured, at least one WUS group will be assigned to UEs that are not configured with paging probability. FFS Mapping of paging probabilities to WUS group. R16 WUS can be supported on all or subset of the paging carriers/paging narrowbands.

In this contribution we discuss RAN2 related aspects of paging probability based WUS groups.

# 2	Discussion

## 2.1	Paging probability based WUS grouping

The following was agreed in RAN2#107 and this introduced many open issues both for RAN and RAN-CN interaction:

Agreements

- Paging probability information is negotiated between the UE and MME via NAS signalling

- This paging probability information needs to be provided by S1 paging

- eNB configures, via broadcast, the relation between this paging probability information and WUS group on Uu interface.

As described in our previous contribution R2-1910434, knowing the exact paging probability of a UE is very difficult and would require an exchange of information between eNB and MME (UE-specific paging capabilities also further complicate this). This may not be feasible, and in practice MME could instead assign UEs to different ‘Paging Probability Classes’ (PP-classes) based on the information it has. According to the agreement above, GWUS UEs would then be assigned to a PP-class over NAS signalling by the MME. Further, the PP-class information of a UE would be included in the paging message sent from MME to eNB. With the use of this information, both eNB and UE could determine in which resource and paging probability group the UE is to be paged. This is illustrated in Figure 1.

Figure 1: Illustration of CN-RAN interaction for group WUS.

As in legacy the paging frame (PF), paging occasion (PO), and paging narrowband (LTE-M) or paging carrier (NB-IoT) of a UE is determined from UE_ID. As of Rel-15 WUS operation the UE’s WUS position if different depending on if it is configured with DRX gap, short eDRX gap, or long eDRX gap. In Rel-16, the WUS UE groups are added on top of this as illustrated in Figure 2.

Figure 2: Illustration of 4 WUS groups in 3 WUS resources in 3 paging narrowbands.

In RAN1 it has been agreed that ‘The number of UE groups per WUS resource is 1, 2, 4, or 8’ and that therefore 8 WUS sequences are introduced for this purpose. In total up to 10 WUS sequences could therefore be configured in the cell due to the agreed common WUS and the legacy Rel-15 WUS sequence. What remains to be determined is how a UE’s WUS UE group is determined among all WUS resources. To reduce RAN2 complexity, it is beneficial to be as agnostic as possible of the RAN1 WUS resources for the WUS group selection. We therefore propose, as illustrated at the top of Figure 3, that WUS UE groups are considered over all WUS resources and that group indexing implicitly continues in the WUS resources with higher index.

For WUS UE group selection, only the total number of WUS UE groups over all WUS resources are considered (i.e., no need to indicate the WUS resource).

Figure 3: Illustration of WUS UE groups in multiple WUS resources and PP-class partitioning.

In the figure ‘C’ indicates the Common WUS sequence, ‘L’ indicates the Rel-15 legacy WUS sequence, and ‘?’ indicates unused WUS sequences.

In addition to receiving the UE’s PP-class in the paging message from MME, eNB may initially also from the MME obtain the information about how many PP-classes have been configured in total. This unless the number of PP-classes is preconfigured. Note that using a percentage for the paging probability is just the special case of having 100 preconfigured PP-classes. In our view, using a percentage is however somewhat misleading since the MME cannot determine the exact paging probability, and further such high granularity is not needed.

The least complex way to do the mapping of PP-classes to WUS UE groups is to introduce partitioning of the WUS UE groups for different PP-classes, as illustrated at the bottom of Figure 3. That is, in determining the WUS UE group of a UE, the WUS UE groups belonging to the PP-class of the UE are considered in the selection (in the WUS resources with the UEs WUS gap type; DRX, short eDRX, or long eDRX).

The WUS UE groups belonging to the UE’s Paging Probability Class are considered in the selection of the UE’s WUS UE group.

In the example of Figure 3, a UE configured to belong to PP-class=’medium paging probability’ would consider WUS UE groups 8-13 for the selection. RAN1 has agreed that there will be a common WUS per WUS-resource, and not per gap type which would be the alternative. Therefore, since ‘common WUS’ will be used if many UEs in different WUS UE groups need to be paged at the same time, there could be a negative impact on PP-Class=Low UEs if they share a WUS resource with PP-Class=High UEs. But if the partitioning is introduced to go from Low to High such negative impact would be minimized.

In the next step, a uniform distribution of the UEs in all the WUS UE groups belonging to a PP-class must be ensured. This is most easily achieved by UE_ID based selection, i.e. in a similar way to how the PF and paging narrowband/carrier is selected in legacy operation.

UE_ID based WUS UE group selection is used over all WUS UE groups that belong to a Paging Probability Class.

The overall procedure could be outlined as follows:

- (MME informs eNB about the number of PP-classes configured. Optional, i.e. if number is not fixed in specification).

- eNB broadcasts information about the mapping between PP-class and WUS UE groups in SI.

- The UE is configured with a PP-class by the MME over NAS.

- MME sends a paging request to the eNB and includes the UE’s PP-class in paging message.

- UE and eNB determine the UE’s WUS group from UE_ID, the UE’s WUS Rel-15 gap type, and the UE’s PP-class.

For the last step, it is, as shown in R2-1910433, important to use other UE_ID bits since UE_ID is already used to determine the PF and the paging narrowband/carrier. If not, interdependencies are created and all the UEs which have the UE_ID to select a certain PF on a certain paging narrowband/carrier will select the same WUS group leaving others unused. A solution to this is to use the following equation to determine a UEs WUS UE group:

WUSgroup=floorfloorUEIDN×NsNn mod Nw

where Nw is the number of WUS UE groups in the UE’s PP-class. It has been evaluated in MATLAB that this equation does indeed produce a uniform distribution of UEs over the WUS groups of a PP-class. In Figure 4, the results are shown for 100000 UEs. UE_ID is first used to uniformly distribute the UEs over 16 PFs (DRX=512 radio frames and nB=T/32) and over 3 paging narrowbands. Remaining UE_ID bits are then used with the equation above to distribute the UEs in one paging narrowband over the 4 WUS UEs groups which belong to a certain PP-class. As seen the result is a uniform distribution with close to 525 UEs in each WUS UE group.

Figure 4: Result of UE_ID based distribution of UEs over WUS UE groups for 3 paging narrowbands and 4 WUS UE groups of a certain PP-class.

Therefore, we propose the following:

The WUS UE group index for a UE is determined by the following equation:

- WUSgroup=floorfloorUEIDN×NsNn mod Nw

- , where Nw is the number of WUS UE groups in the UE’s PP-class.

Note that in Figure 3, a ‘PP-Class = UE-ID based only’ is included since the MME may only have the possibility to assign a UE to estimate the paging probability of a UE after some time of data collection, or may not assign it for other reasons. This is in line with the RAN2 agreements from the last meeting:

RAN2 assumes one or more UEs may not be configured at NAS with paging probability.

If R16 WUS is configured, at least one WUS group will be assigned to UEs that are not configured with paging probability.

This also resolves the following RAN2 FFS, this since PP-Class = ‘UE-ID based only’ would have separate WUS groups configured:

FFS: A WUS group cannot be mapped to both UEs using UE-ID based only and UEs using UE-ID + paging probability based grouping.

A Paging Probability Class for ‘UE-ID based only’ is supported.

## 2.2	Mobility WUS UE group

All gains for Rel-16 GWUS stem from reduced false paging rate. As shown in our earlier contribution R2-1910432, a single mobile WUS UE can increase the false paging rate of UEs in the entire Tracking Area (TA) proportional to the number of cells contained in the TA. E.g. 2 orders of magnitude larger false paging rate for very many UEs in a TA containing 100 eNBs. This problem can easily be avoided if mobile GWUS UEs monitor another WUS sequence (or is allocated to a special WUS UE group) after cell change. One WUS UE group could be set aside for this purpose (i.e. among all WUS resources for a given WUS gap).

A mobile UE can increase the false paging rate of other UEs in the entire TA (by an order of magnitude corresponding to the number cells in the TA).

Figure 5: Illustration of mobility WUS group monitoring in A) the last-known cell, and B) any other cell.

Now from the latest RAN1 agreements it is clear even though up to 8 WUS UE groups are supported, there will be, including the Rel-15 WUS and Rel-16 common WUS, 10 different WUS sequences available. As seen from the upper part of Figure 3, the Rel-15 legacy WUS is at most included in one WUS resource and therefore it could be reused as ‘mobility WUS UE group’ in one of the other WUS resources without any additional cost or negative impact.

Rel-15 legacy WUS sequence index can be reused in another Rel-16 WUS resource as a ‘mobility WUS UE group’ to provide gains without negative impact.

A configurable mobility WUS group is supported which can be used for paging in all other cells than the last-known cell (i.e. the most recent cell in which the UE was in RRC_CONNECTED).

## 2.3	Alternation of WUS resources

Additional open issues were introduced by the earlier RAN1 agreement that Rel-15 legacy WUS could optionally be used as the ‘common WUS’ for Rel-16 UEs. Since there is only one Rel-15 WUS for a WUS gap, this would, as seen from Figure 3, mean that GWUS UEs in the WUS resource containing Rel-15 WUS would have a higher false paging rate than GWUS UEs in the other WUS resources since they would be woken up every time a Rel-15 UE is being paged. Our proposal to limit the complexity introduced by this was that Rel-15 legacy WUS would only be used as the ‘common WUS’ when there are very few Rel-15 WUS UEs in the cell, and hence the negative impact is limited. This was not agreed, but instead RAN1 has agreed the following to compensate for the unfairness among GWUS UEs introduced:

Agreement

The specification supports configurability to enable UE group to alternate between WUS resources

Agreement

Design pre-defined method to allow alternating UE group to monitor different WUS resources at different POs implicitly.

- Consider both cell-specific DRX cycle and UE-specific DRX cycle.

- At least the following parameters are used in the pre-defined method

  - H_SFN of current PO

  - DRX cycle

This introduced additional complexity and must be addressed in RAN2. Again, referring to the upper part of Figure 3, the simplest solution to this would be to alternate between entire WUS resources, i.e. as agreed by RAN1. That is, the indexing of WUS resources used for WUS UE group selection is made time dependent by use of SFN. I.e. differentiating the WUS resource configuration index from the WUS resource index for WUS UE group selection this could be solved by something similar to the following:

WUSindexUE group=WUSindex+floorHSFN×1024+SFNTmod(NWUS)

where WUSindexUE group is the index of the WUS resource used for WUS UE group selection, WUSindex the WUS resource index used in configuration, T the default DRX cycle length, and NWUS the number of WUS resources configured for Rel-16 GWUS. That is, the WUS resource index used for WUS UE group selection will alternate with the number of DRX wake up in the H-SFN period. (Not that just alternation with SFN will not work when there are 2 or 4 WUS resources configured since the UEs DRX cycle is a power of 2, see Appendix A1. Also using SFN in the expression above instead of H-SFN will result in that there will not be any alternation for a DRX cycle of 10.24s). We therefore propose the following, which is according to the RAN1 agreements above:

WUS resource index used for WUS UE group selection is alternated with the use of the following equation:

- WUSindexUE group=WUSindex+floorHSFN×1024+SFNTmod(NWUS)

How such alternation would be configured has already been agreed by RAN1:

UE WUS group hopping between WUS resources is configured per cell.

As a consequence, a 1-bit indication would have to be included in the Rel-16 GWUS configuration in system information.

A 1-bit indication in SI indicates if WUS resource alternation is enabled or not.

## 2.4	GWUS configuration

In Rel-15 WUS is always associated to a UE’s PO and does not complicate the UEs selection of paging carrier/narrowband. I.e., for NB-IoT the Rel-14 paging carrier weights wi are configured based on how much paging load and capability there is per carrier. Rel-15 WUS works with the same weights since it is associated to POs and will therefore use the same paging carrier. However, In the last meeting the following RAN2 agreement was made:

R16 WUS can be supported on all or subset of the paging carriers/paging narrowbands.

This may require other paging weights for GWUS on top of the already existing wi, which would mean a lot of unnecessary complexity. Therefore, we propose to revert the agreement above:

Rel-16 WUS is associated to the paging carriers/narrowbands which contain the UE’s PO.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A mobile UE can increase the false paging rate of other UEs in the entire TA (by an order of magnitude corresponding to the number cells in the TA).

Observation 2	Rel-15 legacy WUS sequence index can be reused in another Rel-16 WUS resource as a ‘mobility WUS UE group’ to provide gains without negative impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For WUS UE group selection, only the total number of WUS UE groups over all WUS resources are considered (i.e., no need to indicate the WUS resource).

Proposal 2	The WUS UE groups belonging to the UE’s Paging Probability Class are considered in the selection of the UE’s WUS UE group.

Proposal 3	UE_ID based WUS UE group selection is used over all WUS UE groups that belong to a Paging Probability Class.

Proposal 4	The WUS UE group index for a UE is determined by the following equation:

- WUSgroup=floorfloorUEIDN×NsNn mod Nw

, where Nw is the number of WUS UE groups in the UE’s PP-class.

Proposal 5	A Paging Probability Class for ‘UE-ID based only’ is supported.

Proposal 6	A configurable mobility WUS group is supported which can be used for paging in all other cells than the last-known cell (i.e. the most recent cell in which the UE was in RRC_CONNECTED).

Proposal 7	WUS resource index used for WUS UE group selection is alternated with the use of the following equation:

WUSindexUE group=WUSindex+floorHSFN×1024+SFNTmod(NWUS)

Proposal 8	A 1-bit indication in SI indicates if WUS resource alternation is enabled or not.

Proposal 9	Rel-16 WUS is associated to the paging carriers/narrowbands which contain the UE’s PO.

# 4	References

RP-191356, “Revised WID for Additional MTC enhancements for LTE”, Ericsson, RAN#84, Newport Beach, USA, June 2019.

RP-192313, “WID revision: Additional enhancements for NB-IoT”, Futurewei, RAN#85, Newport Beach, USA, Sept 2019.

# Appendix

## A.1	Alternation of WUS resources with SFN

- As discussed above, the most simple solution would be to alternate between WUS resources with SFN:

WUSindexUE group=WUSindex+SFNmod(NWUS)

This simple solution unfortunately does not work with an even number of WUS resources since the UEs DRX cycle is a power of 2. That is, in this case there will be no alternation and the UE will always obtain the same WUS resource index for UE group selection.  E.g. if there are 2 paging narrowbands/carriers configured and if the UE has a DRX cycle T=256 and wakes up at SFN={0, 256, 512, 756} to monitor paging, the modulus operation above will return the same index every time and hence there will be no alternation.