3GPP TSG-RAN WG2 Meeting #108	R2-1916106

Reno, USA, 18 – 22 November 2019	Update of R2-1913594

Agenda item: 	6.8.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On-demand and dynamic PRS configuration for DL-TDOA

WID/SID: 	NR_Pos-Core – Release 16

Document for: 	Discussion and Decision

# 1	Introduction

As listed in the WID description, “The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning techniques while considering E911 (e.g. floor level) and commercial requirements for horizontal and vertical positioning accuracies according to TR38.855.”

For DL-TDOA implemented in FR2, the PRS might need to be transmitted in a beamformed fashion to compensate the higher path loss at higher carrier frequencies and therefore the beam sweeping procedures need to be followed. A schematic view of the PRS transmission in beams, along with non-beamformed PRS transmission (as is, for example, in LTE), is provided in Figure 1.

The main implication of transmitting PRS in all beam sweeping directions with static allocation of resources is two-fold (see also [1]-[3]):

a) Unnecessary PRS transmissions: It results in an unnecessary transmission of PRSs, in the sense that not necessarily all PRS transmissions are exploited for determining the position of a target UE, leading thus to spectrum usage inefficiency.

b) Static allocation of PRS resources: It does not permit a dynamic optimization of PRS resources. That is, the static PRS resource allocation does not allow for a temporal increase of the PRS resources for meeting higher positioning accuracy requirements in certain area and instance. Equivalently, the static PRS resource allocation does not account for any decrease of PRS resources in case the positioning requirements can be met even with fewer PRS resources. Such PRS resource control parameters may correspond to, for instance, PRS bandwidth and duration and periodicity of PRS positioning occasions.

Figure 1: Non-beamformed (left) and beamformed (right) PRS transmission

Observation 1: PRS might need to be transmitted in a beamformed fashion to compensate the higher path loss at higher carrier frequencies. The PRS transmission in all beam sweeping directions results in an unnecessary transmission of PRSs depending on positioning needs, while the static allocation of PRS resources does not allow for a PRS resource optimization based on positioning requirements.

The scope of this paper is to address the issue of PRS resource efficiency and highlight potential solution approaches towards this end.

# 2 	On-demand PRS vs dynamic PRS

The terms “on-demand PRS” and “dynamic PRS” have been used in discussions related to an opportunistic use of PRS resources [1]-[4]. In some cases, these two terms have been used to refer to similar or even identical concepts. Here, we aim at distinguishing between the concepts of on-demand and dynamic PRS and highlight their differences.

On-demand PRS in FR2 refers to PRS transmission upon request from UE and is associated with the concept of transmitting PRS only on a subset of beams originating from a given transmission point. This subset of beams corresponds to those beams which serve at least one target UE associated with a positioning session. In other words, on-demand PRS implies that a given beam does not convey a PRS unless there is an indication (or equivalently, a “demand”) that such PRS would be measured by at least one target terminal. The concept of on-demand PRS in fact addresses problem a) stated in Section 1.

Dynamic PRS is dynamic PRS resource allocation and refers to the concept of temporarily increasing/decreasing the PRS resources in a given area and instance, in order to accommodate the respective accuracy requirements. Dynamic PRS allocation addresses problem b) stated in Section 1.

There are conceptual differences between on-demand and dynamic PRS:

i) rather than a simple on-off PRS transmission per beam as in the case of on-demand PRS, a dynamic PRS allocation determines the exact PRS resources at given area and time;

ii) dynamic PRS allocation is based on positioning accuracy requirements, which are typically known to the location server, whereas on-demand PRS does not. In fact, on-demand PRS considers only whether a target UE exists at a given direction, irrespective of the associated requirements.

iii) dynamic PRS allocation comprises a relevant (re-)configuration of PRS resources, as stated in Section 1. On the contrary, on-demand PRS does not involve PRS resource (re-)configuration.

Observation 2: Both notions of on-demand PRS and dynamic PRS are used for PRS resource optimization purposes, however there is a conceptual distinction. Dynamic PRS relates to the positioning requirements and results in optimal PRS resource allocation per area and instance; on-demand PRS dictates whether or not a beam conveys PRS, depending on whether there is a need for PRS transmission at a given direction.

# 3        Positioning-specific measurements

Here we analyse the measurements associated with the concepts of on-demand and dynamic PRS. We keep the analysis general enough such that it covers both cases of PRS management carried out at the LMF and PRS management carried out at the gNB. For this reason, we refer to the entity where the PRS management is performed as “PRS management entity”, which can be either the LMF or the gNB. This Section contains the analysis on the measurement configuration; the ensuing Section 4 provides a comparison between the two approaches.

For the case of on-demand PRS, the PRS management entity needs to obtain information about the beams that the UE can detect in order to select where the PRS needs to be transmitted. Equivalently for dynamic PRS, the PRS management entity needs to obtain feedback from the UE related to the quality of the detected PRS; based on such feedback the PRS management entity determines the allocation of PRS resources in a dynamic fashion.

It follows from the above that on-demand and dynamic PRS allocation are in principle associated with different UE measurement configuration, as well as different signalling procedure. Specifically,

- On-demand PRS requires DL PRS RSRP feedback from the UE to the PRS management entity. In case the PRS management entity is the LMF, such feedback can be requested and obtained via LPP, as part of the DL-TDOA LPPRequestLocationInformation and LPPProvideLocationInformation message.

- In addition to the DL PRS RSRP feedback, dynamic PRS requires positioning-specific information to be provided to the PRS management entity, along with positioning-specific configuration of measurement characteristics. In particular, the PRS management entity needs to configure measurement parameters such as the PRS quality and the periodicity of reporting PRS-RSRP to LMF, which are associated with the update of PRS resources. For instance, in case higher accuracy is temporarily needed due to an emergency, the PRS resource allocation update should be followed by a report from the UEs as of whether the new PRS are measured with sufficient quality. In case the PRS management entity is the LMF, this entails a new positioning-specific measurement configuration message introduced in LPP.

Observation  3: The concepts of on-demand PRS and dynamic PRS are associated with different UE measurement configurations: On-demand PRS utilizes DL PRS RSRP measurements, already available as part of the LPP request and provide location information process. Dynamic PRS needs additional positioning-specific measurement configuration and reporting as part of LPP.

In light of the above observation, it is proposed that, for supporting dynamic PRS and in case the PRS management entity is the LMF, a positioning-specific measurement configuration is enabled by the LMF. Specifically, the LMF configures the measured PRS quality and periodicity of PRS measurements, which facilitate the update of PRS resources.

## 3.1 Case of LMF being the PRS management entity: Signalling steps

In case the PRS management entity is the LMF, the dynamic determination of beams for PRS transmission would require these additional steps:

- the LMF to configure the UE to perform positioning-specific measurements, and the UE to report back to the LMF the measurement report according to that configuration.

- the LMF to reconfigure the PRS resources (if needed), as these were described in Section 1.

These new steps would require certain extensions on the LPP and NRPPa protocols, as illustrated in Figure 2 (blue font indicates changes).

Figure 2: Positioning assistance measurement configuration

Proposal 1:  For dynamic PRS, enable a positioning-specific measurement configuration by the LMF, which facilitates the adaptation of the subset of beams for PRS transmission and therefore, an efficient use of the resources.

Considering that the need for beamformed PRS will be mostly addressed for the FR2, the described solution must be considered as an optional feature. Considering heterogenous deployment, the LMF may consider the request for positioning-specific measurements in FR2.

Observation 4: Positioning-specific measurement configuration, as well as on-demand and dynamic PRS configuration, should be considered as an optional feature.

# 4	LMF-based vs gNB-based dynamic PRS

As mentioned above, the dynamic PRS configuration can be carried out either at the LMF or at the gNB, corresponding to a LMF-based and gNB-based dynamic PRS approach, respectively. Next, the advantages and disadvantages associated with the LMF-based and gNB-based dynamic PRS approach are discussed:

- Advantages of LMF-based dynamic PRS approach:

  - The LMF is typically aware of the positioning requirements. A PRS resource configuration by the LMF is thus facilitated in this respect.

  - A dynamic PRS configuration in principle involves multiple gNBs. As a result, the LMF-based option facilitates a centralized control of PRS transmitted by neighbouring transmission points, reducing thus Xn signalling.

- Advantages of gNB-based dynamic PRS approach:

  - The gNB is aware of the beam configuration (e.g., number of beams per transmission point, azimuth and vertical angles, etc), as well as other resource allocation issues. Such resource allocation is typically done at the RAN. Hence, the gNB-based dynamic PRS configuration reduces the signaling overhead between RAN and core network (LMF).

  - The gNB-based PRS configuration adapts easily to changes to beam configurations at the gNB.

Observation 5: The dynamic allocation of PRS resources can be carried out either at LMF or at gNB. LMF-based dynamic PRS configuration is a centralized approach which facilitates the resource allocation of neighbouring transmission points, whereas gNB-based dynamic PRS configuration is more adaptive to changes to beam configurations and reduces the signalling to the core network.

Proposal 2: RAN2 to discuss the pros and cons of LMF-based and gNB-based dynamic PRS configuration and decide whether the LMF or the gNB shall be responsible for managing such dynamic PRS configuration.

# 5         Conclusions

Observation 1: PRS might need to be transmitted in a beamformed fashion to compensate the higher path loss at higher carrier frequencies. The PRS transmission in all beam sweeping directions results in an unnecessary transmission of PRSs depending on positioning needs, while the static allocation of PRS resources does not allow for a PRS resource optimization based on positioning requirements.

Observation 2: Both notions of on-demand PRS and dynamic PRS are used for PRS resource optimization purposes, however there is a conceptual distinction. Dynamic PRS relates to the positioning requirements and results in optimal PRS resource allocation per area and instance; on-demand PRS dictates whether or not a beam conveys PRS, depending on whether there is a need for PRS transmission at a given direction.

Observation 3: The concepts of on-demand PRS and dynamic PRS are associated with different UE measurement configurations: On-demand PRS utilizes DL PRS RSRP measurements, already available as part of the LPP request and provide location information process. Dynamic PRS needs additional positioning-specific measurement configuration and reporting as part of LPP.

Proposal 1:  For dynamic PRS, enable a positioning-specific measurement configuration, which facilitates the adaptation of the subset of beams for PRS transmission and therefore, an efficient use of the resources.

Observation 4: Positioning-specific measurement configuration, as well as on-demand and dynamic PRS configuration, should be considered as an optional feature.

Observation 5: The dynamic allocation of PRS resources can be carried out either at LMF or at gNB. LMF-based dynamic PRS configuration is a centralized approach which facilitates the resource allocation of neighbouring transmission points, whereas gNB-based dynamic PRS configuration is more adaptive to changes to beam configurations and reduces the signalling to the core network.

Proposal 2: RAN2 to discuss the pros and cons of LMF-based and gNB-based dynamic PRS configuration and decide whether the LMF or the gNB shall be responsible for managing such dynamic PRS configuration.

# References

[1] R2-1901373, On Demand Transmission of PRS for NR, Qualcomm Incorporated, RAN2 #105, Feb. 2019

[2] R2-1901533, Co-ordination among UE, gNB and Location Server for PRS Configuration and Feedbacks, Ericsson, RAN2 #105, Feb. 2019

[3] R2-1907657, Dynamic PRS configuration  for DL-TDOA positioning, Nokia, RAN2 #106, May 2019

[4] R1-1905265, Measurements for PRS on-demand, Nokia, RAN1 #96bis, April 2019